| Ref | Field Name | Action Type | POTS Resale | PAL – Co-Providers | BRI ISDN Resale | PBX | Listings Only | Designed Trunks Resale | DSL | PRI ISDN Resale Trunks | UNE-P ISDN BRI | UNE-P PRI ISDN Trunk | UNE-P PBX Designed Trunk Trunk | UNE-P DSS Trunk | Negotiated Business Rules Rules apply to individual products. Product number precedes the applicable business rule (e.g: 1-4 means rule applies to products 1, 2, 3 & 4. 1,4 means rule applies only to 1 & 4) | Field Lengths | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 3 | 8 | 10 | 13 | 17 | 21 | 28 | 29 | 33 | 35 | 37 | N=Not Req’d., R=Required, O=Optional, C=Conditional, P=Prohibited, Blank=Action Type is N/A , * = Repeating Field (# of stars indicates level of repetition) |  |  |  |
|  | 3.2 Administrative Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PON | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 | a/n |  |
| 2 | VER | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | a/n |  |
| 3 | AN | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 16 | a/n |  |
| 4 | ATN | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 | a/n |  |
| 5 | RSQTY | Populated by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | n |  |
| 6 | ORD | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | C C N C C C C C N C | N N N N N N N N | N N N N N N N N N N N N N N | N | C C N C C C C C C C | C N C C C C N C C N N | C C N C C C C | N N N N N N N N | C C N C C C C | C C N C C C C C C C | C C N C C C C | Products 3, 17, 21, 28, 33, 35, 37: This entry represents the CenturyLink assigned order number. It would not be required on initial order activity unless it was provided during pre-order. The CenturyLink order number as provided on the FOC should be entered on any supplements. | 10 | a/n |  |
| 7 | PG_of_ | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | n |  |
|  | 3.3 Service Details |  |  |  |  |  |  |  |  |  |  |  |  |  | This is a repeating section. LNUM through FEATURE DETAIL may repeat as a group. |  |  |  |
| 8 | LOCNUM | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a/n |  |
| 9 | LNUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | R C N R R R R R R R R R N C | R C N R R R R R N C | R C N R R C R R | R C N R R R R C R R R R N C | R | R C N R R R R R C O | C N R R R R C R R N N | R C N R R R R | R C N R R C R R | R C N R R R R | R C N R R R R R C O | R C N R R R R | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: This field is required if LNA is populated. LNUM 0001 should always be the primary Account Number. LNUM 0001 will be processed as the primary AN, and will be the number published if it is associated with a directory listing. Products 28, 33, 37: LNUM must be unique within a single request/PON and sequential on the initial order starting with 0001. | 4 | n |  |
| 10 | LNEX* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 | n |  |
| 11 | NPI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C C C C C N N N | C N C C C C C N N N | C N C C C C C N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | N N N N N N N | C N C C C C C N | N N N N N N N | N N N N N N N N N N | N N N N N N N | Products 1, 3, 8, 29: For Information Only - When ordering Port In or Port Within, NPI should be populated. | 1 | a | A = Port Out Reserved TN B = Port Out Working TN C = Port in Working TN D = Port in Reserved TN Z = Port Within |
| 12 | LNA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | R C N R R R R R R R R R N C | R C N R R R R R N C | R C N R R C R R | R C N R R R R O R R R R N C | R | R C N R R R R R C O | C N R R R R O R R N N | R C N R R R R | R C N R R C R R | R C N R R R R | R C N R R R R R C O | R C N R R R R | Products 1: If ACT = V, LNA = W, V, N, D, X. If ACT = Z, LNA = W, V, N, D, X. If ACT = Z and LNA = N or D, then the TN cannot be the BTN or - For Information Only - involve Directory Listing changes. For Information Only - If ACT = V or Z and there is a TN Change, then LNA=V. LNA = X is used for TN changes only. If other changes are desired along with a TN change, use LNA=C. Products 3, 8, 10, 17, 21, 28, 29, 35: If ACT = V, LNA = W, V, N, D. If ACT = Z, LNA must be W, V, N, D. If ACT = Z and LNA = N or D, then the TN cannot be the BTN or - For Information Only - involve Directory Listing changes. Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: If ACT = N, LNA = N. If ACT = T, LNA = N - for each line to be moved to the new location in a full move. Note: If TN is changing and transfer of calls is requested, all information should be included with LNA=N. Lines not addressed with LNA of N will be disconnected automatically. If ACT = T, LNA = D, for any line to be disconnected during a full move, a transfer of calls is to be established for the disconnected line(s). Products 1, 3, 8, 10, 17, 21, 28, 29, 35: If ACT = C, LNA = N, C, D, X, P. If LNA is populated, TN must be populated. For Information Only - If LNA = W, then the only applicable field is TN. Products 1, 3, 10, 17, 35: For Information Only - If ACT = B, LNA = L for seasonal restore, but LNA is not populated for deny restore. If ACT = L, LNA = L. For Information Only - If LNA = L, all lines to be suspended must be specified. Products 8, 29, 35: If ACT = M, LNA = C. Product 13: If ACT = R, LNA = R. Products 33, 37: If ACT = V, LNA = V, N, D. If ACT = Z, LNA must be V, N, D. If ACT = Z and LNA = N or D, then the TN cannot be the BTN or - For Information Only - involve Directory Listing changes. If ACT = C, LNA = N, C, D, X, P. If LNA = X, OTN must be populated. If LNA is populated, TN must be populated. | 1 | a | Key: X = TN Change C = Change N = New D = Disconnect V = Conversion of service as specified P = PIC change L = Seasonal Suspend W = Conversion as is R = Record Activity Products 1, 3, 10, 17, 35: X, C, N, D, V, P, L, W Products 8, 21, 28, 29: X, C, N, D, V, P, W Product 13: R Products 33, 37 X, C, N, D, V, P |
| 13 | LTOS* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | a/n |  |
| 14 | SOE* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 | n |  |
| 15 | NOTYP | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 16 | TNS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C C N C C C C N C C C C N C | R C N R R R R R N C | C C N C C C C C | C C N C C C C N C C C C N C | R | R C N R R R R R C C | C N C C C C N C C N N | R C N R R R R | C C N C C C C C | R C N R R R R | R C N R R R R R C C | R C N R R R R | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: Dashes are required in positions 4 & 8. Products 3,: If the line activity is new line (LNA='N') or change TN (LNA='X') and a Co-Provider attempts the Pre-Order TN Reservation functionality, then placeholders are acceptable. For other line activities, the TN must already exist. If TN reserved in pre-order, then an LSR without fatal errors must be received within a pre-determined time frame (e.g., 30 calendar days) or the TN is returned and the LSR is rejected. For Suspend/Restore – Required If LNA = L, otherwise N. Products 1, 8, 10, 29: If TLI is not populated, TN is required. If TLI is populated, TN is optional. If the line activity is new line (LNA='N') or change TN (LNA='X') and a Co-Provider attempts the Pre-Order TN functionality, the placeholders are acceptable unless TLI is populated for Multi-Line Hunt Group. For Information Only: For certain conversion or change activities when OTN is populated, the TNS field represents the new number and it must not exist on the current customer account. For other line activities, the TN must already exist unless TLI is populated for Multi-Line Hunt Group. If TN reserved in pre-order, then an LSR without fatal errors must be received within a pre-determined time frame (e.g.,30 calendar days) or the TN is returned and the LSR is rejected. Products 1, 10: For Suspend/Restore - Required If LNA = L unless TLI is populated for Multi-Line Hunt Group, otherwise O. Products 17, 35: If the line activity is new line (LNA='N') or change TN (LNA=X), then the TN must be a place holder or must have been reserved for this CCNA/PON in pre-order. For other line activities, the TN must already exist. If ACT = L or B then required if LNA = L. Required if ACT = M and LNA = C. Product 21: If TLI field on the Resale form is not populated, TN is required. If TLI field on the Resale form is populated, TN is optional. If the line activity is new line (LNA='N') or change TN (LNA='X'), then the TN must be a place holder or must have been reserved for this CCNA/PON in pre-order either manually or using IMA unless TLI is populated for Multi-Line Hunt Group. For all line activities, the TN must already exist unless TLI is populated for Multi-Line Hunt Group. If TN reserved in pre-order, then an LSR without fatal errors must be received within a pre-determined time frame (e.g., 30 calendar days) or the TN is returned on the LSR is rejected. If a placeholder is used, no checking against pre-order is done. Products 28, 33, 37: If the line activity is new line (LNA='N') or change TN (LNA=X), then the TN must be a place holder or must have been reserved for this CCNA/PON in pre-order. For other line activities, the TN must already exist. | 12 | a/n | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: 303-555-1212 Dashes are required in positions 4 & 8. Dashes are the only alpha characters allowed in this field. Or Actual TN or placeholder (000-000-0000, 000-000-0001, 000-000-0002, etc.) if TN was not assigned during Pre-Order. IMA will reject the request with a TN place holder if the user has not attempted to reserve a TN, with the exception of conversion to stand alone broadband. See CenturyLink Broadband for Resale PCAT for clarification. IMA will accept the place holder if the TN Reservation System did not return any TNs or the TN Reservation system is down. Placeholder TN reference must be unique per PON. |
| 17 | S* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 18 | SPLD* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  | N N N N N N N |  | 1 | a |  |
| 19 | TERS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C C N C C C C N C C C C N C | N N N N N N N N N N | C C N C C C C C | C C N C C C C N C C C C N C | N | N N N N N N N N N N | C N C C C C N C C N N | N N N N N N N | C C N C C C C C | N N N N N N N | N N N N N N N N N N | N N N N N N N | Terminal: Indicates the terminal number on a multi-line hunting line. Products 1, 8, 10, 21, 29: This field is used to indicate the terminal number on a multi-line hunting line. It is required when TLI is populated. | 4 | n | 0-9999 |
| 19a | TLI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C C N C C C C N C C C C N C | N N N N N N N N N N | C C N C C C C C | C C N C C C C N C C C C N C | N | N N N N N N N N N N | C N C C C C N C C N N | N N N N N N N | C C N C C C C C | N N N N N N N | N N N N N N N N N N | N N N N N N N | Telephone Line Identifier: Identifies the pilot number of a multi-line hunt group. Products 1, 8, 10, 21, 29: This is the lead number of the multi-line hunt group. This field is required for any line included in a multi-line hunt group. For multi-line hunting, the TLI is repeated for each TER. For example, if the TLI is 303-123-4567 and there are three related terminals - TER1, TER2, and TER3, and the lines are being set up as new, then the first LNUM would have a TLI= 303-123-4567 and a TER = 0001. The second LNUM would have a TLI=303-123-4567 and a TER = 0002. The third LNUM would have a TLI=303-123-4567 and a TER = 0003. TLI is required if TER is populated. When LNA = N, W, or V, TLI must match a TLI on the HGI form. For other line activities, the TLI must already exist. | 12 | a/n | Products 1, 8, 10, 21, 29: Dashes are required in positions 4 & 8. Actual TLI or place holder (000-000-0000, 000-000-0001, 000-000-0002, etc.) Change Field Length to 12 to match CenturyLink telephone number format standards. |
| 20 | OTN* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N C N C N N N | N N N C C C C N N N | N N N C C C C N | N N N C C C N N C N C N N N | N | N N N N N C C N N N | N N C C C N N C N N N | N N N N N C C | N N N C C C C N | N N N N N C C | N N N N N C C N N N | N N N N N C C | Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: This field is required if LNA = X. OTN cannot be equal to TNS. For Information Only - Required if ACT = T and LNA = N and the TN is changing. Required if LNA=C and TN is changing. Product 21: This field is required if LNA = X, OTN cannot be equal to TNS. For Information Only - It is also required if LNA = C and TN is changing. | 12 | a/n | Products 1, 3, 8, 10, 17, 21. 28, 29, 33, 35, 37: 303-555-1212 Dashes are required in positions 4 & 8. |
| 21 | PRIBD* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | R N N R R R R | N N N N N N N N | R N N R R R R | N N N N N N N N N N | N N N N N N N | Products 28, 33: This field is used to identify ISDN PRI “B” and “D” channel configurations by DS1. Positions 1 & 2 identify the sequencing of the first and all subsequent DS1’s. Positions 3 & 4 represent the quantity of “B” channels ordered per DS1. Positions 5 & 6 represent the number of “D” channels per DS1. Position 7 is needed to designate Non Facility Associated Signaling (NFAS) | 7 | a/n | Products 28, 33: Positions 1 & 2: Sequential starting with 01 Positions 3 & 4: 01 through 24 Positions 5 & 6: 00 through 02 Position 7: NFAS = Y No NFAS = N |
| 22 | ISPID* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N R R R R R | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | N N N N N N N | N N N R R R R R | N N N N N N N | N N N N N N N N N N | N N N N N N N |  | 15 | a/n | Products 8, 29: The format of the ISPID is NPA555121211-11 where 5551212 represents the telephone number. The NPA and 11-11 are always the same, only the telephone number changes which is alpha/numeric due to the dashes. |
| 22a | PTLI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | R C N R R R R R C C | N N N N N N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | R C N R R R R R C C | N N N N N N N | PBX Lead Telephone Line: Identifies the lead telephone line identifier assigned to the Trunk Group. Products 17, 35: This field is required when the TN field is populated. This entry identifies the lead telephone line identifier assigned to the Trunk Group. | 12 | a/n | Example: xxx-xxx-xxxx |
| 22b | PTKCON* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | C N N C C C C C N N C N N N | N | C N N C C C C N N N | N N N N N N N N N N N | R N N R N O R | N N N N N N N N | R N N R N C R | C N N C C C C N N N | R N N R N C R | PBX Trunk Configuration: Identifies the directional configuration of the PBX trunk being ordered. Product 10: (TOS 2nd char =J). If LNA = N or V, PTKCON is required. For Information Only - If LNA = C, PTKCON is required when trunk type is changed. Products 17, 28, 33, 35, 37: This field required when the LNA = N or V and optional for LNA = C. This entry identifies the directional configuration of the PBX trunk being ordered. | 1 | a | Products 10, 17, 28, 33, 35, 37: T = Two-way I = Incoming only O = Out going only |
| 23 | TSP* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O N N O O O O N N N O N N N | N N N N N N N N N N | O N N O O O O N | O N N O O O O N N N O N N N | N | O N N O O O O N N N | N N O O O O N N N N N | O N N O O O O | O N N O O O O N | O N N O O O O | O N N O O O O N N N | O N N O O O O | Products 17, 28, 33, 35, 37: This entry indicates the provisioning and restoration priority defined under the TSP Service Vendor Handbook. A TSP ending in 00 indicates the TSP is being removed. | 12 | a/n | Products 1, 8, 10, 17, 21, 28, 29, 33, 35, 37: Example: TSP12345C-E1 There is 1 preprinted hyphen. |
| 24 | SAN* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O O N O O O O O O O O O N O | O N N O O O O N N N | O O O O O O O O | O O N O O O O O O O O O N O | N | O O N O O O O O O O | N N N N N N N N N N N | O O N O O O O | O O O O O O O O | O O N O O O O | O O N O O O O O O O | O O N O O O O | Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: For Information Only: An entry is required if the 1st character of TOS = 3 | 30 | a/n |  |
| 25 | CKR* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 41 | a/n |  |
| 26 | ECCKT* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N O N R R R O N | O C N C C C R N R N C R N R | N | O C N C C C C R R N | N N R R R N N N N N N | C C N C C C C | N O N R R R O N | C C N C C C C | O C N C C C C R R N | C C N C C C C | Products 10, 17, 35: Required for ACT = V, Z, L, B. For PBX (TOS 2nd char = J), optional if ACT = N, T, V or Z and LNA = N. If ACT = T and LNA = D, required. If ACT = D and TC OPT of EU = S or T, required. If ACT = C and LNA = N, Not required. For all other LNA when ACT=C, Required. Products 8, 29: The value in this field can be associated with multiple telephone numbers. Optional for Disconnect and Outside Move because the account can be accessed by the telephone number. Products 28, 33, 37: For Information Only - This field is Required for all activities when the CLCI is changing. | 42 | a/n |  |
| 26a | MEGACENTNM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C N N N C N N N | N N N N N N N N N N | N N N N N N N N | C N N N N C N N N N C N N N | N | N N N N N N N N N N | N N C C C C N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | C N N N N C C N N N | N N N N N N N | Mega Central Name: This field is used to populate ISP information for CenturyLink Broadband. Field will be populated with the Mega Central Name that matches the valid Mega Central Circuit ID. Products 1, 10, 21, 35: Required if adding broadband to an existing line (FA = N) or if the ISP information is changing. CenturyLink QC Broadband USOCs are required if MEGACENTNM is populated. When ordering CenturyLink.Net, USWNET shall be accepted as a placeholder for the ISP name. CenturyLink will identify and substitute the valid ISP for the service area. | 48 | a/n |  |
| 27 | FPI* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 28 | PIC* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | R N N C C C N N N N C N N N | R N N C C C C N N N | R N N R R C C N | C N N C C C N N N N C N N N | N | C N N C C C C N N N | N N C C C N N N N N N | R N N C C C C | R N N R R C C N | R N N C C C C | C N N C C C C N N N | R N N C C C C | Products 1, 3, 8, 28, 29, 33, 37: This field is required when LNA = N or V. Product 10: This field is required when LNA = N or V and PTKCON does not equal I. If PTKCON = I then an entry is not required. Products 17, 35: This field is required when LNA = N or V and PTKCON = T or O. An entry is not allowed for One-Way -In (PTKCON=I) PBX trunks if TOS 2nd char = J. Product 21: This field is required when LNA = V. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: If LNA = P, PIC and/or LPIC must be populated. For Information Only - If other changes are desired along with a PIC change, use LNA=C. If LNA = C, the PIC only needs to be specified if the PIC is changing. LNA = X is used for TN changes only. If other changes are desired along with a TN change, use LNA=C. Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: If ACT = T and LNA = D the PIC is optional. | 4 | a/n | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: Valid PIC Code Values NONE = No Carrier Selected DFLT = No Carrier Selected |
| 29 | LPIC* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | R N N R R C N N N N C N N N | R N N R R C C N N N | R N N R R C C N | R N N R R C N N N N C N N N | N | R N N R R C C N N N | N N R R C N N N N N N | R N N R R C R | R N N R R C C N | R N N R R C R | R N N R R C C N N N | R N N R R C R | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: If LNA = P, PIC and/or LPIC must be populated. For Information Only - If LNA = C, the LPIC only needs to be specified if the LPIC is changing. Products 28, 33, 37: If ACT = V, Z or T, LPIC is required. Products 1, 3, 8, 10, 17, 29, 35: If ACT = T and LNA = D the LPIC is optional. | 4 | a/n | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: Valid LPIC Code Values NONE = No Carrier Selected DFLT = CenturyLink Carrier Selected |
| 30 | IPIC* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 | a/n |  |
| 31 | SDI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | C N N C C C C N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | N N N N N N N | C N N C C C C N | N N N N N N N | N N N N N N N N N N | N N N N N N N | Products 8, 29: This field is conditional on the use of multiple IXC selection. | 1 | a | Products 8, 29: E, F, G, H, I, J, K, L, M Where: E = Circuit Switched Data and Voice F = Packet Switched Data G = Packet Switched Data and Voice H = Circuit Switched Data & Packet Switched Data and Voice I = Circuit Switched Data & Packet Switched Data J = Packet Switched Data on Channel D K = Switched Data on Channel B @ 56 Kbps L = Switched Data on Channel B @ 64 Kbps M = Voice Only |
| 32 | MATN* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 33 | TC OPT* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N O O O N N N N O O N N | N N N O O O O O N N | N N N O O O O N | N N N O O O N N N N O O N N | N | O N N O O O O O N N | N N O O O N N N N N N | N N N O O O O | N N N O O O O N | N N N O O O O | O N N O O O O O N N | N N N O O O O | Products 8, 29: This is available for ISDN voice calls only (identified by USOCs). Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: If the field is not populated the basic intercept message - no new NBR referral will be activated for the disconnected numbers. | 1 | a | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: N = None (basic intercept message - no new number referral); S = Standard (i.e. forwarded to the new number) T = Split Transfer of Calls (forwarded to multiple new numbers) Not populated (basic intercept message - no new NBR referral.) |
| 34 | LEAN* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | a/n |  |
| 35 | LEATN* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 | n |  |
| 36 | TC TO PRI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT = S (for standard) or T (for split). Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 12 | a/n | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: 303-555-1212 Dashes are required in position 4 & 8. |
| 36a | TCID* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Transfer of Calls to Identifier: Identifies the primary telephone number and name associated with transfer of calls. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT = S (for standard) or T (for split). Name associated with the TC TO PRI field. Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 2 | n | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: 04 - Number Change, Non-List 05 - Number Change, Non-Published 06 - Number Change, Not yet connected 25 - Disconnect, Referral to number 30 - Temporary Disconnect, No referral customer request 39 - Temporary Disconnect, No referral - non-pay 45 - Temporary Disconnect, Referral 55 - Number Change, Referral 65 - Temporary Number Change, Referral 70 - Split Number Referral 74 - TTY Basic intercept - No referral 75 - TTY Number Change, Referral 76 - TTY Disconnect, Referral to number . |
| 36b | TC NAME* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Mov | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Transfer of Calls to Name: Identifies the name associated with TC TO PRI to which calls are to be referred when transfer of calls is requested. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT=S (for standard) or T (for split). Name associated with the TC TO PRI field. Virgule (/) is not a valid character. Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 35 | a/n |  |
|  | Transfer of Calls Secondary Section |  |  |  |  |  |  |  |  |  |  |  |  |  | Assumption: This section (fields TC TO SEC, TC ID and TC NAME represents a secondary set of information within the Transfer of Calls Section. It repeats for as many ‘additional’ parties requested |  |  |  |
| 37 | TC TO SEC** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Products 1, 3, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT is populated with T (for split). Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 12 | a/n | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: 303-555-1212 Dashes are required in position 4 & 8. |
| 38 | TC PER* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT is populated with S (for standard) or T (for split). Products 8, 29: This is available for ISDN voice calls only. | 8 | a/n | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: ccyymmdd |
| 39 | TCID** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT is populated with T (for split). Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 2 | n | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: 04 - Number Change, Non-List 05 - Number Change, Non-Published 06 - Number Change, Not yet connected 25 - Disconnect, Referral to number 30 - Temporary Disconnect, No referral customer request 39 - Temporary Disconnect, No referral - non-pay 45 - Temporary Disconnect, Referral 55 - Number Change, Referral 65 - Temporary Number Change, Referral 70 - Split Number Referral 74 - TTY Basic intercept - No referral 75 - TTY Number Change, Referral 76 - TTY Disconnect, Referral to number . |
| 40 | TC NAME** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N C C C N N N N C C N N | N N N C C C C C N N | N N N C C C C N | N N N C C C N N N N C C N N | N | N N N C C C C C N N | N N C C C N N N N N N | N N N C C C C | N N N C C C C N | N N N C C C C | N N N C C C C C N N | N N N C C C C | Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if TC OPT is populated with T (for split). The Name associated with the TC TO SEC field. Virgule (/) is not a valid character. Products 8, 29: This is available for ISDN voice lines only (identified by USOCs). Data lines do not allow for transfer of calls. | 35 | a/n |  |
| 41 | JK CODE* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C N N N C N N N | C N N C C C C N N N | C N N C C C C C | C N N C C C C N N N C N N N | N | C N N C C C C N N C | N N C C C C N N N N N | C N N C C C C | C N N C C C C C | C N N C C C C | C N N C C C C N N C | C N N C C C C | Indicates the standard code for the particular registered or non-registered jack used to terminate the service. Registered jacks used to terminate category 1 and 3 services begin with the designation “RJ”. This field is required when JR is populated, otherwise prohibited. | 5 | a/n |  |
| 42 | JK NUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | N N N N N N N N N N | N N N N N N N |  | 2 | a/n |  |
| 43 | JK POS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | N N N N N N N N N N | N N N N N N N |  | 2 | n |  |
| 44 | JR* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O N N O O O O N N N O N N N | O N N O O O O N N N | O N N O O O O O | O N N O O O O N N N O N N N | N | O N N O O O O N N O | N N O O O O N N N N N | O N N O O O O | O N N O O O O O | O N N O O O O | O N N O O O O N N O | O N N O O O O | Identifies a request for a new jack. Co-Provider must provide USOC to support Jack request Jack requests are only supported in Minnesota and Oregon for Resale services. | 1 | a | Y = Yes |
| 45 | NIDR* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O N N O O O N N N O O N N N | N N N N N N N N N N | N N N N N N N N | O N N O O O N N N O O N N N | N | O N N O O O O N N N | N N O O O N N N O N N | O N N O O O O | N N N N N N N N | O N N O O O O | O N N O O O O N N N | O N N O O O O | Products 1, 10, 17, 21, 28, 33, 35, 37: If a NID is needed, populate with a Y. | 1 | a | Y = Yes N = No |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The following 2 fields repeat as a group up to 4 times within the repeated Service Detail Section. |  |  |  |
| 46 | IWJK** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O N N O O O N N N N O N N N | O N N O O O O N N N | N N N N N N N N | O N N O O O N N N N O N N N | N | O N N O O O O N N O | N N O O O N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | O N N O O O O N N O | N N N N N N N | Products 1, 3, 10, 17, 21, 35: This Field is valid only in states where Co-Providers have negotiated inside wiring. | 5 | a/n | Products 1, 3, 10, 17, 21, 35: Valid Jack USOCs Example: RJ21X |
| 47 | IWJQ** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C N N N N C N N N | C N N C C C C N N N | N N N N N N N N | C N N C C C N N N N C N N N | N | C N N C C C C N N C | N N C C C N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | C N N C C C C N N C | N N N N N N N | Products 1, 3, 10, 17, 21, 35: This field is required if IWJK is populated, otherwise N/A. | 2 | n |  |
| 48 | SGNL* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | O N N O O O O N N N | N N N N N N N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | O N N O O O O N N N | N N N N N N N |  | 2 | a/n | Products 17, 35: E1 - E& M type 1, E2 - E& M type 2, or E3 - E& M type 3 |
| 49 | SSIG* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C N N N C N N N | N N N N N N N N N N | N N N N N N N N | C N N C C C C N N N C N N N | N | C N N C C C C N N N | N N C C C C N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | C N N C C C C N N N | N N N N N N N | Products 1: This field is required only if GS (Ground Start) is requested. Products 10, 17, 35: This field is required when LNA = N or V. Products 1, 10, 17, 21, 35: If LNA = C, SSIG needs to be specified if signaling is changing. | 2 | a/n | Products 1, 10, 17, 21, 35: LS – Loop Start (default for POTS) GS – Ground Start. |
| 50 | PULSE* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C O N N N C N N N | N N N N N N N N N N | N N N N N N N N | C N N C C C O N N N C N N N | N | C N N C C C C N N N | N N C C C O N N N N N | N N N N N N N | N N N N N N N N | N N N N N N N | C N N C C C C N N N | N N N N N N N | Products 1, 10, 17, 21, 35: For Information Only - The PULSE field is only required in South Dakota for the rules specified as follows: An entry is required if ACT=C and LNA=N. An entry is required if ACT=V or Z and LNA=N or V An entry is required if ACT=N and LNA=N. An entry is required if ACT=T and LNA=N. | 4 | a | Products 1, 10, 17, 21, 35: DP = dial pulse (rotary), DTMF = dual tone multi freq (touchtone). 2 or 4 alpha characters. |
| 51 | LSCP* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O N N O O O O O O O O O O O | O N O O O O O O O O | O N O O O O O O | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N O O O O O O O O O O | N N N N N N N N | O N O O O O O O | N N N N N N N | N N N N N N N N N N | N N N N N N N | Products 1, 3, 8, 21, 29: LSCP = ‘A’ is required if Co-Provider is requesting a Local Exchange Freeze. LSCP = ‘B’ is required if Co-Provider is requesting the removal of a Local Exchange Freeze. If the LSCP field is populated, the ACT must be N, V, Z, C, T, L, or M, otherwise the request will be rejected. | 1 | a/n | Products 1, 3, 8, 21, 29: A = Prohibit a change of current local service provider B = Remove the prohibition |
| 52 | BA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C N N N C N N N | R N N R R C R N N N | C N N C C C C N | C N N C C C C N N N C N N N | N | C N N C C C C N N N | N N C C C C N N N N N | C N N C C C C | C N N C C C C N | C N N C C C C | C N N C C C C N N N | C N N C C C C | Products 1, 8, 10, 17, 21, 28, 29: If ACT = N the only valid value is BA=E, to establish the end state of blocking on the account. Otherwise not populated. If ACT = V, Z or T and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. If ACT = V, Z, or T and BA is not populated, then blocking will be removed from the line. If ACT = C, the BA field is not required unless the Co-Provider specifically wishes to address adding or removing blocking on the account. If ACT = C, the valid BA values allowed are N, D, E, Z or not populated. If ACT = C, and BA is not populated, blocking that is associated with the current line will not be changed. Blocking USOCs & FIDs are not allowed in the FEATURE (Field 64) & FEATURE DETAIL (Field 65). These entries will be derived by CenturyLink from the BA (Field 52) and BLOCK (Field 53) entries. BA = E indicates the end state of the account. Existing block values not specified will be removed. Product 3: BA = Z not valid for PAL. If ACT = N the only valid value is BA=E, to establish the end state of blocking on the account. Otherwise not populated. If ACT = V, Z or T and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. If ACT = V, Z, or T and BA is not populated, then blocking will be removed from the line. If ACT = C, the BA field is not required unless the Co-Provider specifically wishes to address adding or removing blocking on the account. If ACT = C, the valid BA values allowed are N, D, E, Z or not populated. If ACT = C, and BA is not populated, blocking that is associated with the current line will not be changed. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: Required when the BLOCK field is populated. | 1 | a | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: E = End state of the account D = Delete N = No change Z = Remove all blocking |
| 53 | BLOCK* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C N N C C C C N N N C N N N | C N N C C C C N N N | C N N C C C C N | C N N C C C N N N C N N N | N | C N N C C C C N N N | N N C C C C N N N N N | C N N C C C C | C N N C C C C N | C N N C C C C | C N N C C C C N N N | C N N C C C C | Products 1, 8, 10, 17, 21, 28, 29, 33, 35, 37: This field is required if BA = E or D. If BA = N, Z, this field is prohibited. Blocking USOCs & FIDs are not allowed in the FEATURE (Field 64) & FEATURE DETAIL (Field 65). These entries will be derived by CenturyLink from the BA (Field 52) and BLOCK (Field 53) entries. Duplicate BLOCK values not allowed in the BLOCK field. The following single BLOCK values or BLOCK value combinations must be specified on the request if BA=E or D: Group 1: The following single BLOCK values block billing to the end-user’s account: A = No collect and third party (Billed Number Screening) B = No third party (Billed Number Screening) C = No collect call (Billed Number Screening) If BLOCK contains A, BLOCK cannot contain B or C. If BLOCK contains B, BLOCK cannot contain A or C. If BLOCK contains C, BLOCK cannot contain A or B. The valid values specified as part of Group 1 may be added in addition to the values specified in Group 2, Group 3 or Group 4 on the request. Group 2: The following combinations of BLOCK values are valid combinations, which block outgoing calls and equate to the CenturyLink Block products indicated. The blocking combination associated with the Block Product must be specified in it's entirety as a group of values on the request: F, G, H, K, M, N = Long Distance Restriction D, G, H, K, M, N, Q, 0 = Customnet Option 1 with PROX FID D, G, H, K, M, Q, 0 = Customnet Option 1 without PROX FID H, K, M, N, 0 = Customnet Option 2 with PROX FID H, K, M, 0, 9 = Customnet Option 2 without PROX FID H, K, M, 0, = Customnet Option 2 without PROX FID (OR, WA) D, G, H, K, M, N,Q, 0 = Selective Class of Call Screening K, M = Pay Per Call Restriction For information only: For Customnet Option 1 and Selective Class of Call Screening, the CAS codes added by CenturyLink will be: 90 = Special Needs DA Exempt accounts 93 = All other accounts. Other CAS codes must be ordered using MANUAL IND = Y showing the CAS code in REMARKS. For Customnet Option 2, the CAS codes used by CenturyLink will be: 91 = Special Needs DA Exempt Accounts 92 = All other accounts. Other CAS codes must be ordered using MANUAL IND = Y showing the CAS code in REMARKS. The valid values specified as part of Group 2 may be added in addition to the values specified in Group 1 or Group 3 on the request. Group 3: The following BLOCK values block Pay Per Use features: 2 = No call trace (Call Trace Blocking) 3 = No continuous redial (Continuous Redial Blocking) 4 = No continuous redial deluxe (Continuous Redial Deluxe Blocking) 5 = No I-called – originating (I –Called Originating Blocking) 6 = No I-called – terminating (I-Called Terminating Blocking) 7 = No last call return (Last Call Return Blocking) 8 = No 3-way calling (3-Way Calling Blocking) The values of 2, 3, 4, 5, 6, 7, or 8 can be added in combination with one another. The valid values specified as part of Group 3 may be added in addition to the values specified in Group 1, Group 2 or Group 4 on the request. Group 4: G = No 011 (International) (International blocking) N = No casual calling (Carrier Access Code Blocking) Pay per use Block value: H = No Directory Assistance, Call Completion (DACC) (Complete A Call Blocking) If BLOCK = G, H or N, then G, H, or N must not be present in a Block combination in Group 2. The valid values specified as part of Group 4 can be added in addition to the values specified in Group 1 or Group 3. Group 5: The following BLOCK values are invalid stand-alone values. CenturyLink has no product, which equates to these BLOCK values as individual items. D = No 1+ E = No 0+ Local F = No 1+ or +0+ Local K = No 976 M = No 900. Q = No 411 0 = Screen 0+ 9 = Screen Casual Calling. The valid values specified as part of Group 5 may not be present on the request as a single value or in combination with Group 1, Group 2, Group 3 or Group 4. Product 3: This field is required if BA = E or D. BLOCK 2, 3, 7, 8, A, K,and Mare required on all PAL lines, except in Minnesota, where it is optional. Duplicate BLOCK values not allowed in the BLOCK field. Example: 2,2,3,7,8,A,K,M would not be allowed because 2 is present more than once. 4. CenturyLink will provision the Fraud Protection USOCs on PAL classes of service in lieu of the equivalent CustomNet USOCs unless the CustomNet USOCs already exist on the account and are not being changed. 5. CenturyLink will provision Fraud Protection using the state USOCs unless the FCC USOCs exist on the account and are not being changed, or the FCC USOCs are requested in REMARKS using MANUAL IND Y. Product 1: 1) If BLOCK contains F, G, H, K, M, N & Q (Long Distance Restriction) as a combination, FEATURE cannot contain OC4 or NLUAC with FA = N or T. 2) If BLOCK contains G (International blocking) only, FEATURE cannot contain OC4 or NLUAC with FA = N or T. 3) If BLOCK contains K & M (Pay Per Call Restriction) as a combination, FEATURE cannot contain OC4 or NLUAC with FA = N or T. 4) If BLOCK contains D, G, H, K, M, N, Q & 0 (Customnet Option 1 with PROX FID) as a combination, FEATURE cannot contain OC4 or NLUAC with FA = N or T. 5) If BLOCK contains D, G, H, K, M, Q & 0, (Customnet Option 1 without PROX FID) as a combination, FEATURE cannot contain OC4 or NLUAC with FA = N or T 6) If BLOCK contains H, K, M, N & 0, FEATURE cannot contain OC4 or NLUAC with FA = N or T. 7) If BLOCK contains H, K, M, 0, & 9 , FEATURE cannot contain OC4 or NLUAC with FA = N or T. 8) If BLOCK contains, D, G, H, K, M, Q & 0 (Selective Class of Call Screening) as a combination, FEATURE cannot contain OC4 or NLUAC with FA = N or T. | 32 | a/n | Products 1, 3, 8, 17, 21, 28, 29, 33, 35, 37: A = No collect and third party B = No third party C = No collect call D = No 1+ E = No 0+ Local F = No 1+ and 0+ Local G = No 011 (International) H = No Directory Assistance, Call Completion (DACC) K = No 976 M = No 900 N = No casual calling Q = No 411 0 = Screen 0+ 2 = No call trace 3 = No continuos redial 4 = No continuos redial deluxe 5 = No I-Called – originating 6 = No I-Called – terminating 7 = No last call return 8 = No 3-Way calling 9 = Screen all casual calling This field can contain multiple blocking values based on the Negotiated Business Rules. Product 3: A = No collect and third party D = No 1+ G = No 011 (International) H = No Directory Assistance, Call Completion (DACC) K = No 976 M = No 900 N = No casual calling Q = No 411 0 = Screen 0+ 2 = No call trace 3 = No continuos redial 7 = No last call return 8 = No 3-Way calling 9 = Screen all casual calling This field can contain multiple blocking values based on the Negotiated Business Rules. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: When requesting blocking commas, spaces, and special characters are not allowed. |
| 54 | CC-ACT* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 55 | CC-NO* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 14 | n |  |
| 56 | CCT* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 57 | FLI* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | a/n |  |
| 58 | DI* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | a |  |
| 59 | CCDD* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 10 | a/n |  |
| 60 | CNAM* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 15 | a/n |  |
| 61 | BSPRAO* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | a/n |  |
| 62 | CFA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | N N N N N N N N N N N N N N | N N N N N N N N N N | N N N N N N N N | N N N N N N N N N N N N N N | N | N N N N N N N N N N | N N N N N N N N N N N | O N N N N O O | N N N N N N N N | O N N N N O O | N N N N N N N N N N | O N N N N O O | Products 28, 33, 37: For Information Only - Field is Required for PRI ISDN Trunk groups only. Not applicable for DID numbers. | 54 | a/n | Products 28, 33, 37: The delimiter between each element of the CFA field is either a single virgule (/) or a single period. |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The following 3 fields repeat as a group within the Service Detail Section. |  |  |  |
| 63 | FA** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | R C N C C C R N N N C N N N | R C N R R C C N N N | R C N C C C C C | R C N C C C R N N N C N N N | R | R C N R R C C N N C | C N C C C R N N N N N | R C N R R C R | R C N C C C C C | R C N R R C R | R C N R R C C N N C | R C N R R C R | Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: Required if ACT = V or Z, and LNA = V, N, or D If LNA = C, FA = N, C, D, T. If LNA = N, FA = N. If LNA = V, FA = N, C, D, V, or T and must be populated. If LNA=X or P, FA is n/a. Products 3, 8, 10, 17, 28, 33, 35, 37: If non-line-level USOC changes use FA = N&D. If the (line-level or non-line-level/feature) USOC remains the same or changes and the FID or FID detail is changing you must use FA = C&T. Product 29: If the (line-level or non-line-level/feature) USOC remains the same or changes and the FID or FID detail is changing you must use FA = C&T. Products 8, 28, 29, 33, 37: For Information Only - Required for ACT = C if Add/Delete Features or Add/Delete Hunting or Add/Delete listing requires a chargeable USOC. Product 13: If non-line-level USOC changes use FA=N&D. If the (line-level or non-line-level/feature) USOC remains the same or changes and the FID or FID detail is changing you must use FA = C&T. FA = V is only valid if this Listings only request is associated with a physical work conversion request. Product 21: Required if ACT = V or Z, and LNA = V, N, or D. If LNA = X, W, P or Y FA is n/a. If LNA = C, FA = N, C, D, T. If LNA = D, FA = D. If LNA = V, FA = N, C, D, V, or T and must be populated. Products 28, 33, 37: For Information Only - If ACT = D, required if add/delete features and add/delete listing requires a chargeable USOC. Products 1: If LNA = C, if a line-level USOC is changing, use FA =C&T. If the USOC (line-level or non-line-level) remains and FID/FID Data is changing use FA=T. For Information Only - If a non-line-level USOC is changing, use FA = N&D. If ACT=V or Z, if a line-level USOC is changing, use FA =C&T. If the USOC (line-level or non-line-level) remains same and FID/FID Data is changing you must use FA=T. If ACT = V or Z and LNA = V, the CLEC may choose to designate the desired account end state by specifying each desired USOC using FA=N. Alternatively, the CLEC may use FA = C and T to designate changes to existing line-levelUSOCs, FA = V to convert a USOC without changes, and FA = N to add new USOCs. In this case, FA = D is optional. Important: If a non-line-level/feature USOC is changing, you must use FA = N&D. Products 1, 3, 8, 10, 17, 21, 28: FA = N when FEATURE contains PROMO USOC. Products 1, 3, 8, 10, 17, 29, 35: If ACT = T, LNA = D and TCOPT = T, the FA is required and must be N, otherwise optional. Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37 Not required if Act = C and LNA = D. * the next two fields repeat for each USOC | 1 | a | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: N = Add C = Change (old values) D = Disconnect V = Conversion As Specified T = Change (new values) |
| 64 | FEATURE** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C C N C C C C N N N C N N N | C C N C C C C N N N | C C N C C C C C | C C N C C C C N N N C N N N | C | C C N C C C C N N C | C N C C C C N N N N N | C C N C C C C | C C N C C C C C | C C N C C C C | C C N C C C C N N C | C C N C C C C | Products 1, 8, 10, 17, 21, 29, 35: If ACT = V or Z and the LSR HTQTY = 0, then Hunting USOCs are prohibited. Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37 Blocking USOCs & FIDs are not allowed in the FEATURE (Field 64) & FEATURE DETAIL (Field 65). These entries will be derived by CenturyLink from the BA (Field 52) and BLOCK (Field 53) entries. If BA=E, Block=8 and Feature Activity does not equal D, then Feature ESC, E2H, MVP11 is prohibited. Products 1, 3, 8, 10, 21, 29: This field is required if FA is populated. Always need line level USOCs. The feature for the first line will have the account level and line level USOCs listed. Products 1, 3, 8, 10, 17, 21, 28: When FEATURE = PROMO, FA must = N, MANUAL IND must = Y, and REMARKS must contain service details for the requested promotion. Products 1: If FA = N, then Intraoffice Call Forwarding USOCs (EVD, EVB, EVO, EVF, EVK or EV2) are prohibited in the Feature Field. If FA = T, then Intraoffice Call Forwarding USOCs (EVD, EVB, EVO, EVF, EVK or EV2) are allowed in the Feature Field. Product 10: If TOS 2nd char=J, and if ACT=N, T, V, or Z, the first inward USOC (FA=N, V) must be Class of Service (COS). Product 13: Listing USOC required here when FA is present. Listing USOCs are generally 3 characters. Products 1, 10, 21, 35: MEGACENTNM is required if CenturyLink QC Broadband USOCs are populated. Products 28, 33, 37: This field is for CenturyLink Trunk and DID USOCs; it is Required if FA is populated Products 1, 3, 8, 10, 17, 21, 28, 29, 33, 35, 37: If ACT = N, V, Z, C, or T, and LNA = N, V, or C, and FEATURE DETAIL is not blank, then can not request multiple actions for the same(USOC + (FID & FID value)) combination unless the action (FA) is the same for each of those combinations: can not have the same FEATURE + FEATURE DETAIL combination repeat in the same service details section unless the actions (FA) are the same. If the actions (FA) are different, the LSR will be rejected. For All Regions: For Business: If ACT = N, V, Z, or T and LNA = N or V and FA = N, V, or T, and if requesting NWT then must also request one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR, NDY and ESX or MVPCW. If ACT = C and LNA=C and FA=N or T, and if requesting NWT then CLEC must also have or request one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR, NDY and ESX or MVPCW. If ACT=C, LNA=C, FA=D and NWT (business) USOC then one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR or NDY must be removed and one of the following USOCs: ESX or MVPCW must also be removed. If ACT=C, LNA=C, FA=D and N2W (residential) USOC then one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR or NDY must be removed. For Residence: If ACT = N, V, Z, or T and LNA = N or V and FA = N, V, or T, and if requesting N2W then must also request one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR, NDY. If ACT = C and LNA=C and FA= N or T and if requesting N2W then CLEC must also have or request one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR, NDY. If ACT=C, LNA=C, FA=D and N2W USOC then one of the following USOCs: NNK, N6S, N6SD1, NSD, NDR or NDY must be removed. Products 1, 3, 8, 10, 17, 28, 29, 33, 35, 37: If FEATURE contains NLUXG, then FA cannot be N or T. If FEATURE contains NLUXG, R7N, or R8W, then FA cannot be N or T (AZ only). Informational only: Product 1: EDIT: Cannot act on USOC not already on line where ACT = V, Z or C LNA = V or C FA = V or C - validates that every USOC modified on the LSR, already exists for the same line on the CSR. Description of Reject Scenario: If the modified USOC on the LSR does not exist for the same line on the CSR EDIT: Cannot add USOC already on line where ACT = C LNA = C FA = N - rejects an LSR if same USOC already exists for the same line on the CSR Description of Reject Scenario: If the added USOC on the LSR already exists for the same line on the CSR. | 5 | a/n | Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: Valid values are 3 or 5 characters in length. |
| 65 | FEATURE DETAIL** | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change C=Add/Delete Feature C=Add/Delete Listings C=Add/Delete Hunting C=Drop/Replace NID T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | O O N C C C C N N N C N N N | C C N C C C C N N N | O O N O O O O O | O O N O O O O N N N O N N N | O | C C N C C C C N N C | O N C C C C N N N N N | C C N C C C C | O O N O O O O O | C C N C C C C | C C N C C C C N N C | C C N C C C C | Products 1, 8, 10, 17, 21, 29, 35: If ACT = V or Z and the LSR HTQTY = 0, then Hunting FIDs are prohibited. Products 1, 3, 8, 10, 13, 17, 21, 25, 28, 29, 33, 35, 37: This field is for FID and data entry only. Must be a FID followed by a qualifier. For Information Only - Form is: "/<FID> <data>". All CenturyLink QC FID entries not supported by OBF fields are required here. For example, the LCC FID is required when the Co-Provider wants Customized Routing (chooses not to have CenturyLink DA/OS). If the feature activity is new (FA = N) or change to (FA = T) and the feature is for Custom Ringing, (e.g., FEATURE = 'RGGnA/B/C' - custom ringing), a TN place holder or TN reserved in pre-order for this CCNA/PON, either manually or using IMA, must follow the TN FID in the FEATURE DETAIL field. If TN in this field was reserved in pre-order, then an LSR without fatal errors must be received within a pre-determined time frame (e.g., 30 calendar days) or the TN is returned and the LSR is rejected. If a placeholder is used, no checking against pre-order is done. Product 13: All CenturyLink QC FID entries for listing USOCs generally require FID data referencing the ALI code. Products 1: If ACT = N, V, Z, T, C and FA=N, V, T then the Co-Provider should enter FID Detail for the following FIDs in the formats specified: (“n” represents a blank field to be populated by a numeric character from 0 to 9 only. “x” represents a blank field to be populated by a numeric value from 0-9 or a alphabetical character from A to Z only. /CFN nnn nnn-nnnn /CFNB nnn nnn-nnnn /CFND nnn nnn-nnnn /RCYC n /WW nnn nnn-nnnn /SH1 nnn nnn-nnnn /MSS *xxxxxx MSSGRP.xxxx /MSS *xxxxx MSSGRP.xxxx /MSS *xxxx MSSGRP.xxxx If ACT=N,V,Z,T,C and, FA=N,T,V, D then the following shall be required if requesting any of the Call forwarding/Blocking/Custom Ringing USOCs ALL STATES: 1) If FEATURE** field contains any one of the following USOCs: ERD, EVD, EVDHG, EVF, EVFHG, or FVJHG then FEATURE DETAIL** must be populated with RCYC FID 2) If FEATURE** field contains any one of the following USOCs: EVB, or EVBHG then FEATURE DETAIL** field must be populated with CFNB FID 3) If FEATURE** field contains any one of the following USOCs: RGG1+, RGG2+, or RGG3+ then FEATURE DETAIL** field must be populated with TN FID 4) If FEATURE** field contains any one of the following USOCs: EVD, or EVDHG then FEATURE DETAIL** field must be populated with CFND FID 5) If FEATURE** field contains any one of the following USOCs: EVF,EVFHG, or FVJHG then FEATURE DETAIL** field must be populated with CFN FID 6) If FEATURE DETAIL** field contains MCFI FID, then it must be populated with TN FID. 7) IF FEATURE** field contains any one of the following USOCs: EVO, EVOHG then FEATURE DETAIL** field must be populated with CFNB FID ALL STATES except OR, WA: 8) If FEATURE** field contains any one of the following USOCs: EVK, or EVKHG then FEATURE DETAIL** field must be populated with CFN and RCYC FIDs ALL STATES except Northern ID: 9) If FEATURE** field contains any one of the following USOCs: FBJ, FBJHG then FEATURE DETAIL** field must be populated with CFNB FID 10) If FEATURE** field contains FDJ USOC then FEATURE DETAIL** field must be populated with CFND FID 11) If FEATURE** field contains any one of the following USOCs: FDJ, FVJ then FEATURE DETAIL** field must be populated with RCYC FID 12) If FEATURE** field contains FVJ USOC then FEATURE DETAIL** field must be populated with CFN FID. WA and OR Only: 13) If FEATURE** field contains EV2 USOC then FEATURE DETAIL** field must be populated with CFN and RCYC FIDs For Information Only - If ACT = N, V, Z, or C and FA=N, V, T and if requesting PPU Blocking do not submit as a FID and/or FID Data the following values: RCU, COT, AC, SDS, VFA, AR, and TWC. | 512 | a/n |  |
|  | Remarks Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65a | REMARKS | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C O N C C C C O N O | C O C C C C C O O O | C O C C C C C O | O O N O O O O O N O | O | O O O O O O O O O O | O O O O O O N | O O N O O O O | C O C C C C C O | O O N O O O O | O O O O O O O O O O | O O N O O O O | Remarks: Identifies a free flowing field which can be used to expand upon and clarify other data on the form. Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: Remarks are recommended if SUP=3 to explain the changes made on the LSR. Remarks are required if basic installation with testing is requested. In case of a held order, use this field to indicate that this LSR is for a held order. Enter CDLR as a Remark if appropriate. Products 1, 3, 8, 29: Required when NPI field is populated. Products 1, 3, 8, 10, 17, 21, 28: If a Resale promotion is ordered (FEATURE = PROMO), REMARKS are required to explain the service details for the requested promotion, e.g. PROMO SPEED DIALING 8-01 THRU 10-01. The MANUAL IND must = Y. | 255 | a/n |  |
| 65b | MANUAL IND | N=New Installation D=Disconnect W=Conversion As Is V=Conv. As Specified Z=Conv. Spec/No DL C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore R=Record M=Inside Move | C C N C C C C C N C | C C C C C C C C C C | C C C C C C C C | C C N C C C C C N C | C | C C C C C C C C C C | C C C C C C N | C C N C C C C | C C C C C C C C | C C N C C C C | C C C C C C C C C C | C C N C C C C | Manual Indicator: Indicates that the Co-Provider acknowledges that the request contains information that must be processed manually by the CenturyLink Service Center. Products 1, 3, 8, 10, 13, 17, 21, 28, 29, 33, 35, 37: MANUAL IND must be set to Y by the Co-Provider if the REMARKS field contains information that must be processed manually by the CenturyLink Service Center or if any other rule indicates the MANUAL IND should be set to 'Y'. Products 1, 3, 8, 29: When NPI field is populated, MANUAL IND must be set to “Y”. Products 1, 3, 8, 10, 17, 21, 28: If a Resale promotion is ordered (FEATURE = PROMO), the MANUAL IND must = Y, and REMARKS are required to explain the service details for the requested promotion. | 1 | a/n | Products 1, 3, 8, 10, 13, 17, 21, 25, 28, 29, 33, 35, 37: Y = REMARKS must be processed manually. N = REMARKS do not need manual attention. |
| 66 | TCMI | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 | n |  |
| 67 | TCMESS | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  | 250 | n |  |