| Ref | Field Name | Action Type | Centrex Plus/Centron | Centrex 21 | UNE Centrex (P or STAR) | UNE Centrex 21 (P or STAR) | Negotiated Business Rules Rules apply to individual products. Product number precedes the applicable business rule. (e.g., 1 - 4 means rule applies to products 1, 2, 3 & 4. 1, 4 means rule applies only to 1 & 4.) | Field Length | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 9 | 9a | 30 | 31 | N=Not Req’d., R=Required, O=Optional, C=Conditional, P=Prohibited, Blank=Action Type is N/A, * = Repeating Field (# of stars indicates level of repetition) |  |  |  |
|  | 3.2 Administrative Section |  |  |  |  |  |  |  |  |  |
| 1 | PON | Not used by CenturyLink |  |  |  |  |  | 16 | a/n |  |
| 2 | VER | Not used by CenturyLink |  |  |  |  |  | 2 | a/n |  |
| 3 | RSQTY | Populated by CenturyLink |  |  |  |  | Identifies the quantity of resale services (e.g., station lines). | 3 | n |  |
| 4 | ORD | Not used by CenturyLink |  |  |  |  |  | 9 | a/n |  |
| 5 | ATN | Not used by CenturyLink |  |  |  |  |  | 16 | a/n |  |
| 6 | AN | Not used by CenturyLink |  |  |  |  |  | 16 | a/n |  |
| 7 | CB | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R R | R N N R R R R N N | N R R | R N N R R R R N N | This is the Common Block ID. Products 9, 9a, 30, 31: There can only be one Common block per Centrex Resale Services form. | 18 | a/n | This value can be 1-18 characters in length. |
| 7a | CMS ID | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N N N N N N N | N C C | N N N N N N N N N | Centrex Management System Identifier: Identifies the presence of an optional system feature of the Centrex Plus/Centron and is unique for each Central Office that has a Centrex built in. This unique ID gives the Co-Provider the ability to make feature changes on Centrex Plus/Centron lines via remote access. Products 9, 30: Required if Centrex Management System (CMS) is associated with the common block. | 8 | a/n |  |
| 8 | PG_of_ | Not used by CenturyLink |  |  |  |  |  | 4 | n |  |
|  | 3.3 Common Block Details This section defines features associated with the common block and this will not be applicable for Centrex Analog Non-Design. |  |  |  |  |  |  |  |  |  |
| 9 | SN | Not used by CenturyLink |  |  |  |  |  | 17 | a/n |  |
| 10 | APS | Not used by CenturyLink |  |  |  |  |  | 1 | n |  |
| 11 | SDD | Not used by CenturyLink |  |  |  |  |  | 3 | a/n |  |
| 12 | SMDR | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 13 | SMDRAC | Not used by CenturyLink |  |  |  |  |  | 12 | a/n |  |
| 14 | CPIC | Not used by CenturyLink |  |  |  |  |  | 4 | a/n |  |
| 15 | CLPIC | Not used by CenturyLink |  |  |  |  |  | 4 | a/n |  |
| 16 | CIPIC | Not used by CenturyLink |  |  |  |  |  | 4 | a/n |  |
| 17 | OAR | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 18 | TA | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 19 | TMTC | Not used by CenturyLink |  |  |  |  |  | 2 | n |  |
| 20 | CBA | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 21 | CBLOCK | Not used by CenturyLink |  |  |  |  |  | 16 | a |  |
| 22 | DIALING | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 23 | CPATH | Not used by CenturyLink |  |  |  |  |  | 5 | n |  |
| 24 | CPATHA | Not used by CenturyLink |  |  |  |  |  | 5 | n |  |
| 25 | CFPI | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 26 | LSCP | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 27 | CBFA | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 28 | CB FEATURE | Not used by CenturyLink |  |  |  |  |  | 6 | a/n |  |
| 29 | CB FEATURE DETAIL | Not used by CenturyLink |  |  |  |  |  | 24 | a/n |  |
| 29a | COS | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | R R R | R R R R R R R R R | R R R | R R R R R R R R R | Class of Service: Account Level USOC that defines the billing structure, screening, and type of line. | 5 | a/n | ###### UNE-P Centrex: RHCXX, RHBXX UNE-P Centrex21: RH2XX UNE-STAR Centrex: RHGXX, RHTXX UNE-STAR Centrex21:RH3XX UNE-STAR Centron: RHTXX UNE-STAR Centrex Plus: RHGXX Resale Centron: EZB++ (++ = 2X, UX, JX, YX) Resale Centrex Plus: FFN++ (++ = 2X, UX, JX, DX, YX, NX) Resale CTX 21: C21XX Resale Centrex Prime: 1FM++( ++ = 2X, UX, JX, DX, YX) |
| 29b | XLI | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | N N N N N N N N N | C C C | N N N N N N N N N | Centrex Location Information: The code assigned to an address for Centrex. Products 9, 30: This field is only required for an Eastern region request. This field is required in Eastern Region when DPA is populated. | 3 | n | 1-999 Values are available on the CSR. |
| 29c | DEPT | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | N N N N N N N N N | C C C | N N N N N N N N N | Department Number: The Department Number for Departmental billing. Products 9, 30: This field is only required for a Western region request. This field is required in Western Region when LOC is populated. | 6 | a/n | This field can be 4-6 alphanumeric characters in length. Values are available on the CSR. |
| 29d | DPA | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | N N N N N N N N N | C C C | N N N N N N N N N | Different Premise Address: The code associated with the end user location within the XLI. Products 9, 30: This field is only required for an Eastern region request. This field is required in the Eastern Region when XLI is populated. | 3 | n | 1-999 Values are available on the CSR. |
| 29e | LOC | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | N N N N N N N N N | C C C | N N N N N N N N N | Location Code: The location code for an end user address. Products 9, 30: This field is only required for a Western region request. This field is required in Western Region when DEPT is populated. | 4 | n | 0001-9999 Values are available on the CSR. |
| 29f | MIL | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N O O | N N N N N N N N N | N O O | N N N N N N N N N | Mileage Indicator: The Mileage indicator field displays the distance in quarter miles between the serving Central Office and the customer location. It is used in determining the customer’s line rate. Products 9, 30: Eastern Region - The field is not required. Central & Western Region - The field is optional for ACT = V, C. | 3 | a/n | Central Region – This field can be 1-3 characters in length. Western Region – This field must be 3 numeric characters (001-999). |
|  | 3.4 Station Details* This section repeats as a group from LOCNUM through REMARKS |  |  |  |  |  |  |  |  |  |
| 30 | LOCNUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N N | N N N N N N R N N | N N N | N N N N N N R N N | Products 9a, 31: LOCNUM = 2 for ACT = T. | 3 | n |  |
| 31 | LNUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R R | R R N R R R R R N | N R R | R R N R R R R R N | Products 9, 9a, 30, 31: LNUM 0001 should always be the primary Account Number. LNUM 0001 will be processed as the primary AN, and will be the number published if it is associated with a directory listing. | 4 | n |  |
| 32 | LNEX* | Not used by CenturyLink |  |  |  |  |  | 5 | n |  |
| 33 | NPI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | O O O | N N O O N O N N N | O O O | N N O O N O N N N | Products 9, 9a, 30, 31: For Information Only - When ordering Port In or Port Within, NPI should be populated. | 1 | a | C = Port in Working TN D = Port in Reserved TN Z = Port within |
| 34 | LNA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R R | R R N R R R R R N | N R R | R R N R R R R R N | Products 9, 30: If ACT = V, LNA = V, N, D. If ACT = C, LNA = N, C, D, T, X, P. If LNA = X, OTN must be populated. Products 9a, 31: If ACT = N, LNA = N If ACT = D, LNA = D If ACT = Z, LNA = V, N, D If ACT = V, LNA = V, N, D. If ACT = C, LNA = N, C, D, X, P. If ACT = T, LNA = N, D If ACT = B, LNA = L If LNA = X, OTN must be populated. | 1 | a/n | Products 9, 9a, 30, 31: N = New C = Change D = Disconnect V = Conversion as specified X = Telephone number change P = PIC change Products 9a, 31: L = Seasonal Suspend Products 9, 30: T = Outside move within the Central Office |
| 35 | SOE* | Not used by CenturyLink |  |  |  |  |  | 2 | n |  |
| 36 | NOTYP* | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 37 | TNS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C C N C C C C C N | N C C | C C N C C C C C N | Products 9, 30: If TLI (field38a) is not populated, TNS is required. If TLI (field38a) is populated, TNS is optional. If LNA is populated, either TLI or TNS must be populated. Products 9a, 31: If the line activity is new line (LNA = N) or TN (LNA = X), a Co-Provider attempts the Pre-Order TN Reservation, then placeholders are acceptable if they are in the TNS field, unless TLI is populated for Multi-line Hunt Group. For other line activities, the TN must already exist if populated. If TN is reserved in pre-order, then an LSR without fatal errors must be received within a pre-determined time frame (i.e. 24 business hours) or the TN is returned and the LSR is rejected | 12 | a/n | Dashes are required in positions 4 & 8. Dashes are the only alpha characters allowed in this field. Products 9a, 31: Actual TN or placeholder (000-000-0000, 000-000-0001, 000-000-0002, etc.) if TN was not assigned during Pre-Order. IMA will reject the request with a TN place holder if the user has not attempted to reserve a TN. IMA will accept the place holder if the TN Reservation System did not return any TNs or the TN Reservation system is down. Placeholder TN reference must be unique per PON. Placeholder TN reference must be unique per PON. |
| 38 | TERS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Terminal Number: Identifies a non-lead line in a multi-line hunt group. Products 9, 9a, 30, 31: This field is required when TLI is populated. | 4 | n | 0-9999 |
| 38a | TLI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Telephone Line Identifier: The lead number of the multi-line hunt group. Products 9, 9a, 30, 31: This field is required for any line included in a multi-line hunt group. For multi-line hunting, the TLI is repeated for each TER. For example, if the TLI is 303-123-4567 and there are three related terminals - TER1, TER2, and TER3, and the lines are being set up as new, then the first LNUM would have a TLI= 303-123-4567 and a TER = 0001. The second LNUM would have a TLI=303-123-4567 and a TER = 0002. The third LNUM would have a TLI=303-123-4567 and a TER = 0003. TLI is required if TERS is populated. When LNA= N or V, TLI must match a TLI on the HGI form. | 12 | a/n | Dashes are required in positions 4 & 8. |
| 39 | API* | Not used by CenturyLink |  |  |  |  |  | 1 | n |  |
| 40 | OTN* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Products 9, 9a, 30, 31: Required if LNA = X or LNA = V or T and the TN is changing. | 12 | a/n | Dashes are required in positions 4 & 8. |
| 41 | DSN* | Not used by CenturyLink |  |  |  |  |  | 7 | n |  |
| 42 | CLN* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N N N N N N N | N C C | N N N N N N N N N | Products 9, 30: This field is required when LNA = V, N, or T and Centrex Line Name is associated with the common block. | 25 | a/n | Examples: John Smith Liz Smith, Marketing Dept |
| 43 | PIC* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R C | R N N R R C C N N | N R C | R N N R R C C N N | Products 9, 9a, 30, 31: This field is required when LNA = V, N, or T. If LNA = P, PIC and/or LPIC must be populated. If LNA = C, PIC only needs to be specified if PIC is changing. | 4 | a/n | Valid PIC Code Values NONE = No Carrier Selected DFLT = No Carrier Selected |
| 44 | LPIC* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R C | R N N R R C C N N | N R C | R N N R R C C N N | Products 9, 9a, 30, 31: This field is required when LNA = V, N, or T for all states. If LNA = C or P, LPIC only needs to be specified if LPIC is changing. If LNA = P, PIC and/or LPIC must be populated. | 4 | a/n | Valid LPIC Code Values NONE = No Carrier Selected DFLT = CenturyLink Carrier Selected |
| 45 | IPIC* | Not used by CenturyLink |  |  |  |  |  | 4 | a/n |  |
| 46 | LTC* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R C | R N N R R C C N N | N R C | R N N R R C C N N | This field indicates the NCOS (Network Class of Service) or CAT (category) code. Products 9, 30: LTC only needs to be specified if LTC is changing. Products 9a, 31: If LNA = T, LTC is required. If LNA = C, LTC only needs to be specified if LTC is changing. | 3 | n |  |
| 47 | LSCP* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Products 9, 9a, 30, 31: LSCP = ‘A’ is required if Co-Provider is requesting a Local Exchange Freeze. LSCP = ‘B’ is required if Co-Provider is requesting the removal of a Local Exchange Freeze. If the LSCP is populated, the ACT must be N, V, Z, C, or T, otherwise the request will be rejected. | 1 | a | A = Prohibit a change of current local service provider B = Remove the prohibition |
| 48 | BA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Products 9, 9a, 30, 31: Blocking USOCs & FIDs are not allowed in the FEATURE & FEATURE DETAIL. These entries will be derived by CenturyLink from the BA and BLOCK entries. For Information Only: If LCC FID is used in conjunction with Blocking, then it is required in the Feature Detail field. BA = E indicates the end state of the account. Existing block values not specified will be removed. If ACT = C, the BA field is not required unless the Co-Provider specifically wishes to address adding or removing blocking on the account. If ACT = C, the valid BA values allowed are N, D, E, Z or not populated. If ACT = C, and BA is not populated, blocking that is associated with the current line will not be changed. Required when the BLOCK field is populated. Products 9, 30: If ACT = V and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. If ACT = V and BA is not populated, then blocking will be removed from the line. Products 9a, 31: If ACT = N the only valid value is BA=E, to establish the end state of blocking on the account. Otherwise not populated. If ACT = V, Z or T and blocking is part of an existing line, BA = E is the only valid value allowed to indicate the end state of the account. If ACT = V, Z, or T and BA is not populated, then blocking will be removed from the line. | 1 | a | E = End state of the account D = Delete N = No change Z = Remove all blocking |
| 49 | BLOCK* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Products 9, 9a, 30, 31: This field is required if BA = E or D. If BA = Z, this field is prohibited. Blocking USOCs & FIDs are not allowed in the FEATURE (Field 75) & FEATURE DETAIL (Field 76). These entries will be derived by CenturyLink from the BA (Field 48) and BLOCK (Field 49) entries. Duplicate BLOCK values not allowed in the BLOCK field. The following single BLOCK values must be specified on the request if BA=E or D: Products 9 and 30: Group 1: The following single BLOCK values block billing to the end-user’s account: ###### A = No collect and third Party calls (Billed Number Screening) B = No third party calls (Billed Number Screening) ###### C = No Collect calls (Billed Number Screening) If BLOCK contains A, BLOCK cannot contain B or C. If BLOCK contains B, BLOCK cannot contain A or C. If BLOCK contains C, BLOCK cannot contain A or B. The valid values specified as part of Group 1 may be added in addition to the values specified in Group 2 on the request. Group 2: The following BLOCK values block Pay Per Use features. 2 = No call trace (Call Trace Blocking) 3 = No continuous redial (Continuous Redial Blocking) 4 = No continuous redial deluxe (Continuous Redial Deluxe Blocking) 5 = No I-called – originating (I –Called Originating Blocking) 6 = No I-called – terminating (I-Called Terminating Blocking) 7 = No last call return (Last Call Return Blocking) H = No Directory Assistance, Call Completion (DACC) (Complete A Call Blocking) The valid values specified as part of Group 2 may be added in addition to the values specified in Group 1 on the request. Products 9a and 31: Group 1: The following single BLOCK values block billing to the end-user's account: A = No collect and third Party calls (Billed Number Screening) B = No third party calls (Billed Number Screening) C = No Collect calls (Billed Number Screening) If BLOCK contains A, BLOCK cannot contain B or C. If BLOCK contains B, BLOCK cannot contain A or C. If BLOCK contains C, BLOCK cannot contain A or B. The valid values specified as part of Group 1 may be added in addition to the values specified in Group 2 or 3 on the request. Group 2: The following combinations of BLOCK values are valid combinations, which block outgoing calls and equate to the CenturyLink Block products indicated. The blocking combination associated with the Block Product must be specified in it's entirety as a group of values on the request: D, G, H, K, M, N, Q, 0 = Customnet Option 1 with PROX FID D, G, H, K, M, Q, 0 = Customnet Option 1 without PROX FID H, K, M, N, 0 = Customnet Option 2 with PROX FID H, K, M, 0, 9 = Customnet Option 2 without PROX FID H, K, M, 0, = Customnet Option 2 without PROX FID (OR, WA) If Customnet Values are selected as part of the BLOCK request the FEATURE DETAIL field must contain the Line Class Code FIDS of either N68 or N69. For information only: For Customnet Option 1 and Selective Class of Call Screening, the CAS codes added by CenturyLink will be: 90 = Special Needs DA Exempt accounts 93 = All other accounts. Other CAS codes must be ordered using MANUAL IND = Y showing the CAS code in REMARKS. For Customnet Option 2, the CAS codes used by CenturyLink will be: 91 = Special Needs DA Exempt Accounts 92 = All other accounts. Other CAS codes must be ordered using MANUAL IND = Y showing the CAS code in REMARKS. The valid values specified as part of Group 2 may be added in addition to the values specified in Group 1 or Group 3 on the request. Group 3: The following BLOCK values block Pay Per Use features. 2 = No call trace (Call Trace Blocking) 3 = No continuous redial (Continuous Redial Blocking) 4 = No continuous redial deluxe (Continuous Redial Deluxe Blocking) 5 = No I-called - originating (I -Called Originating Blocking) 6 = No I-called - terminating (I-Called Terminating Blocking) 7 = No last call return (Last Call Return Blocking) H = No Directory Assistance, Call Completion (DACC) (Complete A Call Blocking) The valid values specified as part of Group 2 may be added in addition to the values specified in Group 1 or Group 3 on the request. A Call Blocking) The valid values specified as part of Group 2 may be added in addition to the values specified in Group 1 or Group 3 on the request. | 32 | a/n | Products 9, 9a, 30, 31: A = No collect and third party B = No third party C = No collect call D = No 1+ G = No 011 (International) H = No Directory Assistance, Call Completion (DACC) K = No 976 M = No 900 N = No casual calling Q = No 411 0 = Screen 0+ 2 = No call trace 3 = No continuous redial 4 = No continuous redial deluxe 5 = No I-called – originating 6 = No I-called – terminating 7 = No last call return 9 = Screen Casual Calling. This field can contain multiple blocking values based on the Negotiated Business Rules. When requesting blocking commas, spaces, and special characters are not allowed. |
| 50 | CC-ACT* | Not used by CenturyLink |  |  |  |  |  | 1 | a/n |  |
| 51 | CC-NO* | Not used by CenturyLink |  |  |  |  |  | 14 | n |  |
| 52 | CCT* | Not used by CenturyLink |  |  |  |  |  | 1 | a/n |  |
| 53 | FLI* | Not used by CenturyLink |  |  |  |  |  | 3 | a/n |  |
| 54 | DI* | Not used by CenturyLink |  |  |  |  |  | 1 | a/n |  |
| 55 | CKR* | Not used by CenturyLink |  |  |  |  |  | 36 | a/n |  |
| 56 | PRIBD* | Not used by CenturyLink |  |  |  |  |  | 7 | a/n |  |
| 57 | SDI* | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 58 | MATN* | Not used by CenturyLink |  |  |  |  |  | 1 | a |  |
| 59 | TSP* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N O O | N N N N N N N N N | N O O | N N N N N N N N N | This entry indicates the provisioning and restoration priority defined under the TSP Service Vendor Handbook. | 12 | a/n | example: TSP12345C-E1 There is one pre-printed hyphen. |
| 60 | SAN* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | O O O | O O O O O O O N N | O O O | O O O O O O O N N | Products 9, 9a, 30, 31: For Information Only: The Co-Provider is responsible to track. Required if 1st character of TOS = 3. | 30 | a/n |  |
| 60a | MEGACENTNM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move L=Seasonal Suspend B=Restore Y=Deny | N N C | C N N C C C C N N N | N N C | C N N C C C C N N N | Mega Central Name: This field is used to populate ISP information for CenturyLink Broadband. Field will be populated with the Mega Central Name that matches the valid Mega Central Circuit ID. Product 9, 9a, 30, 31: Required if adding broadband to an existing line (FA = N) or if the ISP information is changing. CenturyLink QC Broadband USOCs are required if MEGACENTNM is populated. When ordering CenturyLink.Net, USWNET shall be accepted as a placeholder for the ISP name. CenturyLink will identify and substitute the valid ISP for the service area. | 48 | a/n |  |
| 61 | LEAN* | Not used by CenturyLink |  |  |  |  |  | 20 | a/n |  |
| 62 | LEATN* | Not used by CenturyLink |  |  |  |  |  | 12 | n |  |
| 62a | TC OPT* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N O O | N O N O O O O N N | N O O | N O N O O O O N N | Transfer of Calls Option: Identifies the type of transfer of call option the end user has requested. Products 9, 9a, 30, 31: Optional if LNA = D or X. | 1 | a | N = None (basic intercept message – no new number referral.) S = Standard (i.e., forwarded to the new number) T = Split Transfer of Calls (forwarded to multiple new numbers.) |
| 62b | TC TO PRI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Primary Number: Identifies the primary telephone number to which calls are to be referred. Products 9, 9a, 30, 31: Required if TC OPT is populated with S (Standard) or T (Split). | 12 | a/n | Dashes are required in positions 4 and 8. |
| 62c | TCID* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Identifier: Identifies the primary telephone number and name associated with transfer of calls. Products 9, 9a, 30, 31: Required if TC OPT is populated with S (Standard) or T (Split). | 2 | n | Products 9, 9a, 30, 31: 04 - Number Change, Non-List 05 - Number Change, Non-Published 06 - Number Change, Not yet connected 25 - Disconnect, Referral to number 30 - Temporary Disconnect, No referral customer request 39 - Temporary Disconnect, No referral - non-pay 45 - Temporary Disconnect, Referral 55 - Number Change, Referral 65 - Temporary Number Change, Referral 70 - Split Number Referral 74 - TTY Basic intercept - No referral 75 - TTY Number Change, Referral 76 - TTY Disconnect, Referral to number |
| 62d | TC NAME* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Name: Identifies the name associated with TC TO PRI to which calls are to be referred when transfer of calls is requested. Products 9, 9a, 30, 31: Required if TC OPT is populated with S (Standard) or T (Split). Name associated with TC TO PRI field. Virgule (/) is not a valid character. | 35 | a/n |  |
|  | Transfer of Calls to Secondary Section This section represents a secondary set of information within the Transfer of Calls section, it repeats for as many additional parties requested. |  |  |  |  |  |  |  |  |  |
| 62e | TC TO SEC** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Secondary Number: Identifies the secondary telephone number to which calls are to be referred. Products 9, 9a, 30, 31: Required if TC OPT is populated with T (Split). | 12 | a/n | Dashes are required in positions 4 & 8. |
| 62f | TCID** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Identifier: Identifies the sequence of telephone numbers and names associated with split transfer of calls. Products 9, 9a, 30, 31: Required if TC OPT is populated with T (Split). | 2 | n | Products 9, 9a, 30, 31: 04 - Number Change, Non-List 05 - Number Change, Non-Published 06 - Number Change, Not yet connected 25 - Disconnect, Referral to number 30 - Temporary Disconnect, No referral customer request 39 - Temporary Disconnect, No referral - non-pay 45 - Temporary Disconnect, Referral 55 - Number Change, Referral 65 - Temporary Number Change, Referral 70 - Split Number Referral 74 - TTY Basic intercept - No referral 75 - TTY Number Change, Referral 76 - TTY Disconnect, Referral to number |
| 62g | TC NAME** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls to Name: Identifies the name(s) associated with TC TO SEC fields to which calls are referred when split transfer of calls is requested. Products 9, 9a, 30, 31: Required if TC OPT is populated with T (Split). Virgule (/) is not a valid character. | 35 | a/n |  |
| 62h | TC PER* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | N N N C C C C N N | N C C | N N N C C C C N N | Transfer of Calls Period: Indicates the requested date that the transfer of calls, specified in the TC OPT field, is to be removed and the standard recorded announcement is to be provided. Products 9, 9a, 30, 31: Required if TC OPT is populated with S (Standard) or T (Split). | 8 | a/n | ccyymmdd |
| 63 | FPI* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N O O | O N N O O O O N N | N O O | O N N O O O O N N | For Information Only: CenturyLink does not have a current use for this information. If field data is populated by the customer, integrity edits are applied against field length, characteristics, and valid values. | 1 | a | A=Freeze IntraLATA PIC B=Freeze Inter & IntraLATA PIC E=Freeze InterLATA PIC R=Remove InterLATA freeze S=Remove IntraLATA freeze T=Remove Inter & IntraLATA freezes |
| 64 | BSPRAO* | Not used by CenturyLink |  |  |  |  |  | 3 | a/n |  |
| 65 | SGNL* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Products 9, 9a, 30, 31: This entry is required if GS (ground start) is requested. | 2 | a/n | Default to LS = Loop Start GS = Ground Start |
| 66 | SSIG* | Not used by CenturyLink |  |  |  |  |  | 2 | a/n |  |
| 67 | PULSE* | Not used by CenturyLink |  |  |  |  |  | 4 | a |  |
| 68 | JR* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N O | O N N N N O O N N | N N O | O N N N N O O N N | Identifies a request for a new jack. Co-Provider must provide USOC to support Jack request Jack requests are only supported in Minnesota and Oregon for Resale services. | 1 | a | Y = Yes |
| 69 | JK CODE* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N C | C N N N N C C N N | N N C | C N N N N C C N N | Indicates the standard code for the particular registered or non-registered jack used to terminate the service. Registered jacks used to terminate category 1 and 3 services begin with the designation “RJ”. This field is required when JR is populated, otherwise prohibited. | 5 | a/n |  |
| 70 | JK NUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N N | N N N N N N N N N | N N N | N N N N N N N N N |  | 2 | a/n |  |
| 71 | JK POS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N N | N N N N N N N N N | N N N | N N N N N N N N N |  | 2 | n |  |
| 71a | NIDR* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N O | O N N N N O O N N | N N O | O N N N N O O N N | NID Request: Indicates a request for a new network interface device (NID) Products 9, 9a, 30, 31: If a NID ID is needed, populate with Y. Valid only in states where Co-Provider has negotiated inside wiring. | 1 | a | Y = Yes |
|  | The following 2 fields repeat as a group up to 4 times within the repeating Station Details Section. |  |  |  |  |  |  |  |  |  |
| 72 | IWJK** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N O | O N N N N O O N N | N N O | O N N N N O O N N | Products 9, 9a, 30, 31: Valid only in states where Co-Provider has negotiated inside wiring. | 5 | a/n | Valid Jack USOCs |
| 73 | IWJQ** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N N C | C N N N N C C N N | N N C | C N N N N C C N N | Products 9, 9a, 30, 31: Required if IWJK field is populated, otherwise N/A. | 2 | n | 01 – 99 |
|  | Feature Information The following 3 fields repeat as a group with the repeating Station Details Section. |  |  |  |  |  |  |  |  |  |
| 74 | FA** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R O | R N N R R O R N N | N R O | R N N R R O R N N | Products 9, 9a, 30, 31: If new line, all entries must be N for add features. If LNA = C, FA = D, N, C, or T. If LNA = N, FA = N. If LNA = D, FA = D. If LNA = T, FA = N, D. If LNA = X, FA = N/A. If LNA = P, FA = N/A. If USOC changes, use FA = N & D. If the USOC is staying the same and FID or FID Detail is changing, use FA = C & T. Products 9, 30: If LNA = V, FA = V, N or D and must be populated. Products 9, 9a: FA = N when FEATURE contains PROMO USOC. Product 9a, 31: If LNA = V, FA = V, N, C, T or D and must be populated. If LNA = C, if a line-level USOC is changing, use FA =C&T. If a non-line-level/feature USOC is changing, you must use FA = N&D. If the USOC (line-level or non-line-level) remains and FID/FID Data is changing use FA=C&T. If ACT = V or Z and LNA = V, FA = D is optional. Product 9a: If ACT=V or Z, if a line-level USOC is changing, use FA =C&T. If a non-line-level USOC is changing, you must use FA = N&D. If the USOC (line-level or non-line-level/feature) remains same and FID/FID Data is changing you must use FA=C&T. If ACT = V or Z and LNA = V, FA = D is optional. Product 31: If ACT = V or Z and LNA = V, the CLEC may choose to designate the desired account end state by specifying each desired USOC using FA=N. Alternatively, the CLEC may use FA = C and T to designate changes to existing line-level USOCs, FA = V to convert a USOC without changes, and FA = N to add new USOCs. In this case, FA = D is optional. Important: If a non-line-level/feature USOC is changing, you must use FA = N&D. | 1 | a | C = Change (old values) V = Conversion As Specified N = Add D = Disconnect T = Change (new values) |
| 75 | FEATURE** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N R C | R N N R R C R N N | N R C | R N N R R C R N N | Products 9, 9a, 30, 31: Blocking USOCs & FIDs are not allowed in the FEATURE (Field 75) & FEATURE DETAIL (Field 76). These entries will be derived by CenturyLink from the BA (Field 48) and BLOCK (Field 49) entries. Products 9, 9a, 30, 31: This field is for FID and FID data entry only. Format is: “/<FID><data>”. If ACT = N, V, Z or C and FA=N, V, T and if requesting PPU Blocking do not submit as a FID and/or FID Data the following values: RCU, COT, AC, SDS, VFA, AR, and TWC. MEGACENTNM is required if CenturyLink QC Broadband USOCs are populated. Products 9, 30: If ACT = V, or C and FA = N, V, or T of a Line Level USOC, then FEATURE DETAIL** field must be populated with LCC FID. Products 9a, 31: If ACT=N, V, Z, T, C and, FA=N, T, V then the following shall be required if requesting any of the Call forwarding/Blocking/Custom Ringing USOCs ALL STATES: 1) If FEATURE** field contains any one of the following USOCs: RGG1+, RGG2+, or RGG3+ then FEATURE DETAIL** field must be populated with TN FID 2) If FEATURE DETAIL** field contains MCFI FID, then it must be populated with TN FID. The other USOCs only apply to UNE-P POTS and Resale POTs, not Centrex 21 (resale or UNE-P). 3) If ACT = N, V, Z, C, T and FA = N, V, or T of a Line Level USOC, then FEATURE DETAIL** field must be populated with LCC FID. | 5 | a/n |  |
| 76 | FEATURE DETAIL** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | N C C | C N N C C C C N N | N C C | C N N C C C C N N | Products 9, 9a, 30, 31: Blocking USOCs & FIDs are not allowed in the FEATURE (Field 75) & FEATURE DETAIL (Field 76). These entries will be derived by CenturyLink from the BA (Field 48) and BLOCK (Field 49) entries. Products 9, 9a, 30, 31: This field is for FID and FID data entry only. Format is: “/<FID><data>”. All CenturyLink QC FID entries not supported by OBF fields are required here. For example, the LCC FID is required. If ACT = N, V, Z or C and FA=N, V, T and if requesting PPU Blocking do not submit as a FID and/or FID Data the following values: RCU, COT, AC, SDS, VFA, AR, and TWC. Products 9a, 31: If ACT=N, V, Z, T, C and, FA=N, T, V then the following shall be required if requesting any of the Call forwarding/Blocking/Custom Ringing USOCs ALL STATES: 1) If FEATURE** field contains any one of the following USOCs: RGG1+, RGG2+, or RGG3+ then FEATURE DETAIL** field must be populated with TN FID 2) If FEATURE DETAIL** field contains MCFI FID, then it must be populated with TN FID. The other USOCs only apply to UNE-P POTS and Resale POTs, not Centrex 21 (resale or UNE-P). If BLOCK field contains the following: D, G, H, K, M, N, Q, 0 = Customnet Option 1 with PROX FID D, G, H, K, M, Q, 0 = Customnet Option 1 without PROX FID H, K, M, N, 0 = Customnet Option 2 with PROX FID H, K, M, 0, 9 = Customnet Option 2 without PROX FID H, K, M, 0, = Customnet Option 2 without PROX FID (OR, WA) Then FEATURE DETAIL must contain either N68 or N69 Line Class Codes FIDS. If ACT = V or Z and the LSR HTQTY = 0, then Hunting FIDs are prohibited. Products 9, 30: If ACT = V and the LSR HTQTY = 0, then Hunting FIDs are prohibited. | 512 | a/n |  |
|  | Remarks Section |  |  |  |  |  |  |  |  |  |
| 76a | REMARKS* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | O O C C O C O N N | C C C | O O C C O C O N N | Remarks: This field gives additional information on the request. Products 9, 9a, 30, 31: Required if TN is being Chipped In and intercom services need to be indicated. Required if PSTN is utilized (if no ARS FIDs are present). If the REMARKS field has information the associated MANUAL IND is required to have either Y or N. This field is required if the NPI field is populated. Products 9, 9a: If a Resale promotion is ordered (FEATURE=PROMO), REMARKS are required to explain the service details for the requested promotion. The MANUAL IND must = Y. | 255 | a/n |  |
| 76b | MANUAL IND* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move B=Restore Y=Deny | C C C | C C C C C C C N N | C C C | C C C C C C C N N | #### Manual Indicator: Indicates that the Co-Provider acknowledges that the request contains information that must be processed manually by the CenturyLink Service Center. Products 9, 9a, 30, 31: MANUAL IND must be set to Y by the Co-Provider if the REMARKS field contains information that must be processed manually by the CenturyLink Service Center, or if any other rule indicates the MANUAL IND should be set to ‘Y’. Required to be set to ’Y’ if NPI field is populated. Products 9, 9a: If a Resale promotion is ordered (FEATURE=PROMO), the MANUAL IND must = Y, and REMARKS are required to explain the service details for the requested promotion. | 1 | a/n | ‘Y’ – REMARKS must be processed manually. ‘N’ – REMARKS do not need manual attention. |