| Ref | LSOG6 POP 120 REF | Field Name | N/R/C/O | Negotiated Business Rules | Field Lengths | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | N = Not Req’d R = Required C = Conditional O = Optional |  |  |  |  |
|  |  | Raw Loop Data Query (RLDQ) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| RLDQ1 | 1 | CCNA | R |  | 3 | a/n |  |
| RLDQ2 | 2 | TXNUM | R | Co-Provider generated and may be reused one month after initial inquiry. | 22 | a/n |  |
| RLDQ3 | 3 | D/TSENT XML: DT_TM_SENT | R |  | 12 | a/n | CCYYMMDDHHMinMin Military Time |
| RLDQ4 | 4 | TXTYP | R |  | 1 | a/n | R = Raw Loop Data Query |
| RLDQ5 | 5 | TXACT | R |  | 1 | a/n | A = New Inquiry |
|  |  | Raw Loop Data Query Section |  |  |  |  |  |
| RLDQ6 |  | TNADDRCKTIND | R | Telephone Number, Address, Circuit Indicator: This field indicated how the query is being done. | 1 | a/n | T = TN A = Address C = Circuit ID |
| RLDQ7 |  | ASGNIND | C | Assignment Indicator Required if TNADDRCKTIND = A. | 1 | a/n | A = Assigned U = Unassigned |
|  |  | The following two fields repeat as a group up to 24 times.. |  |  |  |  |  |
| RLDQ8 |  | WTNNUM* | C | Number of Working Telephone Numbers: This is the total of Telephone Numbers being queried on. Required if TNADDRCKTIND = T. Index of TN/rows in query. | 2 | n |  |
| RLDQ9 | 54 | WTN* | C | Required if TNADDRCKTIND = T. Dashes are required in positions 4 and 8. | 12 | a/n |  |
|  |  | Address Query Section |  |  |  |  |  |
| RLDQ10 | 14 | AFT | O |  | 1 | a/n | A = Rural Route or Route and Box number B = Unnumbered C = Provider assigned house number D = Descriptive |
| RLDQ11 | 15 | SAPR | O |  | 5 | a/n |  |
| RLDQ12 | 16 | SANO | C | Required if TNADDRCKTIND = A and AHN is blank. This field is required when SAPR or SASF are populated. | 8 | a/n |  |
| RLDQ13 | 18 | SASF | O | Optional for TNADDRCKTIND = A and address is numbered. | 4 | a/n |  |
| RLDQ14 | 19 | SASD | O |  | 2 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDQ15 | 20 | SASN | C | Required if TNADDRCKTIND = A. This field is required when SASD, SATH, or SASS are populated. | 50 | a/n |  |
| RLDQ16 | 21 | SATH | O |  | 10 | a/n |  |
| RLDQ17 | 22 | SASS | O |  | 4 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDQ18 | 23 | LD1 | C | Required when LV1 is populated, otherwise prohibited. | 4 | a/n | APT LOT RM SLIP UNIT SUIT |
| RLDQ19 | 24 | LV1 | C | Required when LD1 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ20 | 25 | LD2 | C | Required when LV2 is populated, otherwise prohibited. | 4 | a/n | FLR |
| RLDQ21 | 26 | LV2 | C | Required when LD2 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ22 | 27 | LD3 | C | Required when LV3 is populated, otherwise prohibited. | 4 | a/n | BLDG WNG PIER |
| RLDQ23 | 28 | LV3 | C | Required when LD3 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDQ24 |  | AHN | C | Assigned House Number: This field is used for unnumbered addresses. Required if TNADDRCKTIND = A and SANO is blank. Can only be populated if SASN begins with an '@' sign. The SANO Field can not be populated. | 8 | a/n |  |
| RLDQ25 |  | ROUTE | O | Route: Rural Route | 3 | a/n |  |
| RLDQ26 |  | BOX | O | Box: Box Number | 12 | a/n |  |
| RLDQ27 | 30 | CITY | C | Required if TNADDRCKTIND = A. | 32 | a/n |  |
| RLDQ28 | 31 | STATE | C | Required if TNADDRCKTIND = A. | 2 | a/n |  |
| RLDQ29 | 32 | ZIP | C | Required when CALA is not populated. Required if TNADDRCKTIND = A. | 5 | n |  |
| RLDQ30 |  | CALA | C | Customer Address Location Area: Required when ZIP is not populated. Required if TNADDRCKTIND = A | 4 | a/n | BLVU CHY CO CPR DNV EAST EUGN IA ID IDO MAL MN MT MTA NCO ND NE NMX OLYM OREA PNX PORT SCO SD SEAT SPOK TSN UT UTA VANC WI WO WY WYO |
|  |  | Circuit ID Query Section |  |  |  |  |  |
| RLDQ31 |  | PRIMARY_NPANXX |  | Required if TNADDRCKTIND = C | 6 | n |  |
| RLDQ32 |  | ECCKT |  | Required if TNADDRCKTIND = C | 42 | a/n | Valid ECCKT format: N<n>.AAAA.NNN<nnn>..AA |
|  |  | Raw Loop Data Response (RLDR) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| RLDR1 | 1 | CCNA | R |  | 3 | a/n | Echo from query |
| RLDR2 | 2 | TXNUM | R |  | 22 | a/n | Echo from query |
| RLDR3 | 3 | D/TSENT XML: DT_TM_SENT | R |  | 12 | a/n | CCYYMMDDHHMinMin Military Time |
| RLDR4 | 4 | TXTYP | R |  | 1 | a/n | R = Raw Loop Data |
| RLDR5 | 5 | TXACT | R |  | 1 | a/n | A = New Inquiry |
| RLDR6 |  | RESPONSE | R | Response: This field indicates if ‘Good’, ‘Bad’, or ‘Mixed’ Response is being returned. | 1 | a/n | G = Good B = Bad M = Mixed |
| RLDR7 |  | Mixed Response Type (MIXTYPE) | C | Mixed Response Type: This field categorizes the type of mixed response returned. If RESPONSE = M, then MIXTYPE is required. | 1 | a/n | P = Partial response |
|  |  | Raw Loop Data Section This section is filled in only if RESPONSE = G. |  |  |  |  |  |
| RLDR8 |  | ECCKTNUM* | R | Number of ECCKTS: This field gives the total number of ECCKTs being returned. Index of ECCKT/rows in response. | 2 | n |  |
| RLDR9 | 63 | ECCKT* | R |  | 60 | a/n |  |
| RLDR10 | 54 | WTN (Telephone Number)* | R |  | 12 | a/n | Ex: 303-545-9731 |
| RLDR11 |  | TPRDI* | C | Third Party, Resale, or broadband Indicator: Indicates the third party voice, resale, or broadband Populated if returned | 20 | a/n | Third Party Voice Resale broadband |
| RLDR12 | 14 | AFT* | N | Not used by CenturyLink. | 1 | a/n |  |
| RLDR13 | 15 | SAPR* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 5 | a/n |  |
| RLDR14 | 16 | SANO* | O | ###### Service Address House Number When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 8 | a/n | Example: 3636 |
| RLDR15 | 18 | SASF* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 4 | a/n |  |
| RLDR16 | 19 | SASD* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 2 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDR17 | 20 | SASN* | O | ###### Service Address Street Name When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 50 | a/n | Example: Roundtree Ct |
| RLDR18 | 21 | SATH* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 10 | a/n |  |
| RLDR19 | 22 | SASS* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 4 | a/n | N = North S = South E = East W = West NE = Northeast NW = Northwest SE = Southeast SW = Southwest |
| RLDR20 | 23 | LD1* | C | Required when LV1 is populated, otherwise prohibited. | 4 | a/n | APT LOT RM SLIP UNIT SUIT |
| RLDR21 | 24 | LV1* | C | Required when LD1 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR22 | 25 | LD2* | C | Required when LV2 is populated, otherwise prohibited. | 4 | a/n | FLR |
| RLDR23 | 26 | LV2* | C | Required when LD2 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR24 | 27 | LD3* | C | Required when LV3 is populated, otherwise prohibited. | 4 | a/n | BLDG WNG PIER |
| RLDR25 | 28 | LV3* | C | Required when LD3 is populated, otherwise prohibited. | 10 | a/n |  |
| RLDR26 | 30 | CITY* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 32 | a/n | Example: Greenwood Village |
| RLDR27 | 31 | STATE* | O | When TNADDRCKTIND = A and ASGNIND = U (unassigned for address) echo from address query. | 2 | a/n | Example: CO |
| RLDR28 | 32 | ZIP* | O | If ZIP is present in address query, echo from address query; otherwise populate as null. | 10 | a/n | Example: 80202-1234 |
| RLDR29 |  | WCCLLI * | O | Wire Center CLLI: This is the wire center CLLI Code. | 8 | a/n | Example: BLDRCOMA |
| RLDR30 |  | MLTDIST * | O | MLT Distance: The mechanized loop test distance. | 10 | n | Example: 34766 |
| RLDR31 | 65 | LOOPSTAT* | O | ###### Loop Status | 4 | a/n | CNF = Connected Facility; non-primary end-to-end loops to a unique living unit CT = Connected Through; primary connected through spare PCF = Partially Connected Facilities; loops which are connected only in the latter segments, e.g. crossbox to customer RWKG = Remote Working WKG = Working |
| RLDR32 |  | SEGMENTNUM ** | C | Segment Number: The actual segment number Populated if returned. | 2 | n | 1-99 |
| RLDR33 |  | TERMINAL_ID** | O | Terminal ID: The terminal identification. | 50 | a/n |  |
| RLDR34 |  | BRIDGE_TAP_OFFSET_DESC** | O | Bridged Tap Offset: This provides information about bridge taps. | 150 | a/n |  |
| RLDR35 |  | MAKE_UP_DESC** | O | Make Up Description: This provides data about loop makeup. | 400 | a/n |  |
| RLDR36 |  | CABLE_NAME** | O | Cable Name: The cable identifier being queried. If CABLE_NAME** is FTH you cannot resell data services. | 10 | a/n | Valid Cable Name Prefixes BB = Broadband Network Unit VDSL Fiber to the Curb CH = Carrier Hub CMP = Campus Wire Environment DSL = Digital Subscriber Line, ADSL DSLB = Next Level Digital Subscriber Line, ADSL NLC Broadband Network Unit ADSL Fiber to the Curb EX = Remote Switch Unit / Optical Remote Module FCP = Field Connection Point FH = Fiber Hub FTH = Fiber to the Premises IPG = Integrated Digital Loop Carrier IPGXB = Cross wire center boundary IDLC L = Fiber LH = Fiber Hub LPGB = VDSL-equipped USAM (universal service access multiplexer) VDSL-equipped BSAM (broadband services access multiplexer) MTE = Multi-tenant Environment PG = Pair Gain PGBR#a = BR1/10 ISDN single-system channel bank or side A of a dual- system channel bank PGBR#b = Side B of a dual system channel bank PGL = Next Level DLC Applications PWR = Power Cable RMU = Remote Measurement Unit T = Interoffice Cable UDC = Universal Digital Carrier UDF = Unbundled Dark Fiber |
| RLDR37 |  | PAIR_GAIN_TYPE** | O | Pair Gain Type: This identifies the type of pair gain, if present. | 9 | a/n |  |
| RLDR38 |  | PAIR_NUMBER** | O | Pair Number: The unique number of pair being queried. | 4 | n |  |
| RLDR39 |  | LOADPTAMT ** | O | ###### Load Points Amount | 2 | n | Example: 2 |
| RLDR40 | 81 | LCT** | O | ###### Load Coil Type | 5 | a/n |  |
|  |  | Informational Section This section is filled in only if RESPONSE = M and MIXTYPE = P. |  |  |  |  |  |
| RLDR41 |  | WTN* | C | WTN: Indicates the telephone number associated with the information text Populated if returned. | 12 | a/n |  |
| RLDR42 |  | ITYPE* | C | Information Type: This field identifies the information type related to the loop qualification. Populated if WTN above is populated. | 1 | a/n | I = Incomplete U = Undetermined |
| RLDR43 |  | ITEXT* | C | Information Text: Indicates information text associated with the information type above. Populated if ITYPE above is populated. | 255 | a/n |  |
|  |  | Error Section This section filled in only if RESPONSE = B. |  |  |  |  |  |
| RLDR44 |  | ERRCODE* | C | Error Code: Indicates a predetermined error code. | 14 | a/n | Format (Character positions): 1-3 : Form Name 4-11 : Field Name 12-14 : Error Code |
| RLDR45 |  | ERRMESG* | C | Error Message: Indicates additional information about the error. | 255 | a/n |  |