DMS-100: Ordering Information

| Feature Package | USOC | FID | Definition | Data Information | Data Example |
| --- | --- | --- | --- | --- | --- |
| Qwest Standard Feature Package | BAFHB, in all states except WA BAKHB, WA only ACBHB, UT only, when ordered as an unbundled package |  | Standard Feature Package provided with ISDN BRI. | The TN and CTX FIDs are not used with this USOC |  |

| ISDN BRI Loop Entries | USOC | FID | Definition | Data Information | Data Example (+ = variable data) |
| --- | --- | --- | --- | --- | --- |
| ISDN BRI loop | N2Q |  | Sometimes called the Digital Subscriber Line (DSL) Carries two B Channels and one D Channel | Required whenever installing a new ISDN BRI loop Several FIDs must be follow this USOC – CLS, LSO, FSO, BS, MTER, MDPK, and DCLS The TN and CTX FIDs are not used with this USOC |  |
| Local Serving Office |  | LSO | Indicates the NPA/NXX associated with the wire center, which serves a customer's premises in which a circuit terminates. | Required entry on the N2Q NPA and NXX must be specific to the customer’s usual serving office Obtain from address validation of preorder | /LSO NPA NXX |
| Foreign Serving Office |  | FSO | The switching entity used for a circuit termination. | Used when ISDN BRI dial tone and telephone number are being supplied by a Qwest backhaul or customer designated serving office. NPA and NXX must be specific to the CO supplying the dial tone | /FSO NPA NXX |
| Bearer Service |  | BS | Indicates the basic services provided on the B and D channels | One entry is required for each channel. The B Channel entry is always: CSDV – Circuit Switched Data and Voice D Channel entries may be: BLANK = Signaling only PSD = Packet Switched Data | /BS CSDV CSDV PSD /BS CSDV CSDV BLANK |
| All Call Blocking | NDD |  | The NRC associated with the Custom Local Area Signaling Services (CLASS) feature, All Call Blocking, applies to the ISDN BRI line/loop (N2Q) USOC Only one NRC applies per ISDN BRI loop, regardless of how many TNs are being blocked. | Required if Caller ID Blocking is desired. The NDD USOC is entered after the N2Q USOC The /SUPP FID and its information follows all affected PDN USOCs | Example below |
| Suppress Network name ID Information |  | SUPP | The network name identification is to be suppressed | This Caller ID Blocking FID follows the affected LTH1X, LNMIX, and LTH5X USOCs. Data is always: PUBLIC DN.Y, NAME.Y One entry required per affected PDN | LTHIX /etc./SUPP PUBLIC DN.Y, NAME.Y etc. = data omitted. |

| B Channel Logical Terminal Entries Feature | USOC | FID | Definition | Data Information (+ = variable data) | Data Example (+ = variable data) |
| --- | --- | --- | --- | --- | --- |
| Logical Terminal on the B Channel- also called the PDN USOC | LTHIX |  | Sets the Primary Directory Number or (PDN) associated with the terminal and provides information to support use of the terminal in an ISDN environment | USOC followed by FIDs identified below. |  |
| Telephone Number |  | TN | Indicates the number assigned to the terminal | Required following the logical terminal USOC | /TN NPA NXX-XXXX |
| Primary Number |  | PN | Indicates the primary telephone number of the terminal | Required following the logical terminal USOC | /PN NPA NXX-XXXX |
| Service Profile Identifier |  | SPID | Allows the CPE to identify itself to the CO switch | Each LTH1X/LNM1X will have its own identifier Required data is the 10-digit PDN plus a suffix of 11-11 (no spaces) | /SPID NPANXXXXXX11-11 |
| Digital Services Network Assignment |  | DSNA | Common language of the ISDN BRI loop associated with the service. | Required to follow the logical terminal USOC and all associated USOCs except the N2Q and optional features Same data as the CLS assigned to the ISDN BRI loop | /DSNA ++.IBSA.++++++..++ |
| Terminal End Point Identifier |  | TEI | Indicates whether the TEI is static or dynamic for a terminal on a given ISDN BRI. | Required data - Circuit Switched = D Packet Switched = 21-26 (Each device is numbered in sequence) | /TEI D /TEI 21 |
| Key Number |  | KEY | Used to indicate on which KEY (or button) a line or feature works | Required data is: 1 through 69 = Indicates the specific key assigned to the line or feature | /KEY 5 |
| Key List |  |  | Used with features to indicate the key numbers on which this feature is going to work | Data is the Key to which the feature is assigned and the keys on which the feature will function | /KEY 2, 2-4 |
| Ring On Line |  | ROL | Denotes whether the line rings or not | Required data is: Y = DN rings N = DN does not ring | /ROL Y /ROL N |
| Maximum Keys |  | MXK | Indicates the maximum number of KEYS on an ISDN terminal | Required numeric value of 64 or less, or 9 or less depending on the system The entry is CPE specific. | /MXK 22 /MXK 9 |
| Authorized Bearer Services Exceptions |  | ABS | Entries are services that customers can not access | Required data is: NOVOICE = No voice services NOVBD = No voice band data NOCMD = No circuit mode data NOPMD = No packet mode data | /ABS NOPMD, NOVBD |
| Logical Terminal Class |  | LCTL | Indicates the service capabilities of the terminal | Data is: BRAFS (Functional Signaling) CS (Circuit Switched Capabilities - Y = Yes - N = No PS (Packet Switched Capabilities) - N = No - D = D Channel Packet PVC - VERSION FUNC2 (Western) - FUNC2 (Eastern and Central NOTE: A semicolon separates the PS and PVC data. | /LTCL BRAFS CS Y PS N; PVC FUNC2 |
| Local Area Signaling Service |  | LASS | Used to provision ISDN CLASS Features | Required data is: COT – Customer Originated Trace SCF – Selective Call Forwarding SCRJ – Selective Call Rejection BLGOPT – Billing Option Name AMA = Automatic Message Accounting record created NOAMA = No AMA record created STAT – Status Indicator ACTIVE INACTIVE RGRM – Ring Reminder NA – Customer group ring is used | /LASS COT BLGOPT.AMA (Customer Originated Trace) /LASS SCRJ BLGOPT.NOAMA, STATE.INACTIVE (Selective Call Rejection) |
| Additional Functional Call Option |  | AFCO | Allows up to 5 call appearances of a DN on one terminal. Works on EKTS and non-EKTS sets | Used only with a non-MADN directory number with more than one call appearance Populate with the number of additional call appearances over one. Data = 1 through 4 | /AFCO 2 |
| Additional Call Offering Service |  | ACOS | Indicates how many incoming calls are supported for the PDN Indicates an incoming call when both B Channels are in use (similar to the analog call waiting feature) | Used only with non-EKTS terminals Data is: ACOU (Additional Call Offering Unrestricted) NBL (Notification Busy Limit) – populate with the number of call appearances available for incoming calls. Can not exceed the data following the /AFCO entry 1 through 4 | /ACOS ACOU NBL.2 |
| Sub Address |  | SUBA | Indicates the sub address, reservation number, type, and optional terminating priority call types for call appearances on a terminal. | Subscription Feature Name: - PROVCGS = Calling party sub address - PROVCDS = called party sub address - PROVLLC = lower layer compatibility - PROVHLC = higher layer compatibility Call type Name: - TYPE.VBINFO – voice band - TYPE.CMDATA = circuit mode data This entry is CPE specific | /SUBA PROVLLC TYPE.VBINFO, PROVLLC TYPE.CMDATA, PROVHLC TYPE.VBINFO, PROVHLC TYPE.CMDATA |
| Terminal Type |  | TTYD | Used to indicate that a terminal has Electronic Key Telephone Service (EKTS) and/or Call Appearance Call Handling (CACH) capabilities. | Data is: - EKTS or -EKTS, CACH Absence of this FID indicates a non-EKTS terminal. | /TTYD EKTS, CACH |
| Multiple Appearance Directory Number |  | MADN | Indicates that a DN terminates on more than one terminal. It is used on both the primary appearance and other appearances of that number. | Required if a PDN or SDN appears on another terminal. Do not use this FID on a Shared Directory Number (DO8) Can not have MADN numbers on a non-EKTS terminal. MADN Type - S – Single Call Member Type - P – Primary N - N – Non-primary Ringing - R – Ring - NR – No Ring Denial Treatment - Silence - Tone Bridging Allowed - Y – Yes - N – No If bridging is allowed, the following entries are required: Conference Bridge Size - 3 through 30 Bridge Tone - Y – Yes - N – No Initial Privacy Status Indicator - Private - Non-Private Privacy Release Mode Indicator - Manual - Auto | /MADN S P R SILENCE N /MADN S P R SILENCE Y 5 Y PRIVATE MANUAL |
| Multiple Appearance DN Ring Forward (Automatic) |  | MRF | Used to activate MADN ringing preferences | Used if there is a MADN group and at least one member has Abbreviated or Delayed Ringing If all members of a MADN group will ring or not ring, the MRF FID is not needed Data is: Automatically Activated - Y – Yes - N – No MRF Timer (required when automatically activated) - 0 through 60 (seconds) Ring Type - ALWAYS - NEVER - ABBR - DELAY | /MADN S P R SILENCE N/MRF Y 15 ALWAYS /MADN S P R SILENCE N /MRF Y 20 ABBR |
| Multiple Appearance DN Ring Forward (Manual) |  | MRFM | The MRFM is used to activate MADN ringing forward manually. The feature is assigned to a button or key and activates the ring forward feature against the specified MADN appearances on the set. A MADN group with manual ring forward needs the MRFM assigned on the abbreviated SET. When the abbreviated ringing SET user pushes the MRFM key, the ringing stops on the abbreviated ringing SET and starts ringing on the delayed ringing SET. An additional feature, MRFM provides the ability to manually override the auto MADN Ring Forward. By assigning the /MRFM to a key on a SET with abbreviated ringing, the ring cycle can be made shorter than the automatic time. | Used if there is a MADN group and at least one member has Abbreviated or Delayed Ringing There is no data following this FID. | /MRFM |
| Call Forwarding Simultaneous Screening Options |  | CFSO | Used to indicate how many calls the CLASS feature Selective Call Forwarding will forward at one time. | Used only if the customer is requesting the CLASS feature Selective Call Forwarding Data is always: - SIM N (only one call at a time will forward) | /CFSO SIM N |
| Call Pickup |  | CPG | Indicates the call pickup group number for a PBX station, or a stored program controlled switch line, when the line is contained in a call pickup group and the number of each line in the group. | Used with Call Pickup Data is the telephone number of one of the numbers in the call pickup group. | /CPG 406 238-1414 |
| Feature Name |  | FNM | Provides translations for a feature that is not provisioned via a separate FID or USOC. | Used only when a feature is requested that is not provisioned via a separate FID or USOC. Data is the keyword for the feature name that will typed into the switch. | /FNM CBU |
| Intercom |  | ICM | Indicates: - intercom group name - type of intercom - auto or two-digit - auto dial TN - TN intercom number - call appearance button used for the intercom and number | This feature should be specified on the request or by the ordering code chosen Data is: ICMGRP.+++++, where + is 1-8 SOURCE.++++, where + is AUTO or DIAL TNPOS.++, where + is 0-9 (for AUTO) or 00-99 (for DIAL) CA.+++ where +++ is 1 to 3 digit call appearance button number NBR.++ where ++ is 1 to 2 digit number of call appearances | /ICM ICMGRP.50234, SOURCE.DIAL, TNPOS.02, CA.5, NBR.1; ICMGRP.50244, SOURCE.AUTO, AUTODP.0, TNPOS.1, CA.6, NBR.1 |
| Miscellaneous provisioning USOC, per key (button) | ETQPB |  | Directory/software numbers that can be assigned to buttons/keys on the electronic business sets. | Used when a feature does not have its own USOC and needs a Key assignment, This USOC is followed by FIDs that bring about the actual provisioning of the feature |  |
|  |  | ICM | Intercom Feature Used to provision an automatic intercom between 2 terminals in the same CO. | Data is: The 7-digit PDN of the other end of the intercom link This is followed by SIC and the key number assigned to the intercom, which must be the same number for both ends of the intercom The SIC entry is followed by DOR.N, SMDR.N | On PDN 333-2211: ETQPB /KEY 6/ICM 333-2233 SIC.6, DOR.N, SMDR.N On PDN 333-2233: ETQPB /KEY 6/ICM 333-2211 SIC.6, DOR.N, SMDR.N |
|  |  | DSA | Distribution And Supplemental Assignment Used by SWITCH to read non-ISDN features as ISDN features | Required on all feature USOCs that have been defined as non-ISDN Data is: IS | USOC /DSA IS |
|  |  | FC | ISDN Flexible Calling Used to provision the “big thee” features – Conference, Drop, and Transfer; or six port conferencing Used for voice calls only – is assigned to the PDN and applies to all directory numbers on a terminal (See the NZ6PK USOC for use of this FID to provision six-way conferencing) | Data is: 3 = 3 way conference DROP # = conference member number (CPE specific), number must be higher than the Key # assigned to the flexible calling XFER # = conference (CPE specific), number must be higher than the Key # assigned to the flexible calling CTALL = call transfer all type calls | ETQPB /FC 3, DROP 26, XFER 27, CTALL |
|  |  | DSNA | Digital Services Network Assignment Common language of the ISDN BRI loop associated with the service. | Required to follow the logical terminal USOC and all associated USOCs except the N2Q and optional features Data is: The circuit ID assigned to the ISDN BRI loop | /DSNA ++.IBSA.++++++..++ |
|  |  | GIC | Group Intercom Used to provision a group intercom, which can have up to 100 members. | Data is: Group Intercom Name assigned by NSAC – can be 1 to 8 characters followed by a dash (no spaces) Member Number is a 2-digit entry (00-99) followed by a comma and a space SMDR.N = Station Message Detail Recording, which is followed by a comma and a space MSB.Y = No Make Set Busy | /GIC 45678-03, SMDR.N, MSB.Y |

| D Channel Logical Terminal Entries | USOC | FID | Definition | Data Information | Data Example |
| --- | --- | --- | --- | --- | --- |
|  | LTH4X |  | D Channel Packet Logical Terminal USOCs LTH4X used regionally |  |  |
| Data Terminal Number |  | DTN | Used to indicate the data terminal number for dialing the device | Used with D Channel packets only – no voice. Data consists of the NPA, prefix and line number. May be the same number as the PDN, or can be a different number. The DSNA FID follows the data following the DTN. | /DTN 719 636-1234/DSNA 29.IBSA.123456..MS |
| Public Packet Line Class Code |  | PLCC | Line Class Code for the packet switched services | Data is IPS | /PLCC IPS |
| Logical Channel Layout |  | LCL | Indicates the maximum number of channels and the allocation of the channels associated with an X.25 access line Two logical channels are provided per ISDN BRI loop. If more than two are requested, one NW9AL USOC must be added to the order for each additional X.25 logical channel | Data is: - 2WL – Lowest two-way logical channel number – will always be 1 - 2WH – Highest two-way logical channel number of channels – can be 1-15 | /LCL 2WL.1, 2WH.2 |
| Destination Network Identification Code |  | DNIC | Permits routing over a particular interface providing gateways to packet switched networks or databases. Corresponds to the PIC code. | Data is 4 numeric characters | /DNIC 3147 |
| Throughput Class and Flow Control |  | TWN | Gives the throughput, window size, and packet size negotiation for the packet switched data | Data is: - T (Throughput Negotiation) Y = Yes, N = No - W (Window Size Negotiation) Y = Yes, N = No P (Packet Negotiation) Y = Yes, N = No The SPID, CLS and PN should be floated following this entry | /TWN T.Y, W.Y, P.Y/SPID 303555121211-11/CLS 29.QZSA.24919..MS/PN 303 555-1212 |
| Fast Select Options |  | FS | Indicates the fast select options available on the X.25 packet service | This feature should be specified on the request. To Initiate Fast Select, data entries are: - IB – Both restricted and unrestricted response - IR – Restricted response - IU – Unrestricted response To Accept Fast Select. data entries are: - AY – Accept Fast Select - AN – Fast Select Not Accepted The CLS and DTN are floated following this entry | /FS IB AY/CLS 19.QSZA.123456..MS/DTN 602 235-6789 |