Information Technologies, Inc.

MEDIACC

Improved Options for POTS Repair

Technical Specifications

MEDIACC 5.08.13

Author:  itdocs@qwest.com

Date:     December 15, 2003

.

Version Notice

Revision History

Revisions of this document are listed in chronological order. There is no relationship between the document release number and the software release number.

| Document Release | Date | Description | Software Release |
| --- | --- | --- | --- |
| Version 1 | 11/20/2003 | Initial Draft | MEDIACC 5.08.13 |
| Version 2 | 12/15/2003 | Final Draft | MEDIACC 5.08.13 |

# Document Ownership

This document is prepared by Qwest Information Technologies, Inc.

Contact itdocs@qwest.com for more information and queries.

Trademarks

Qwest Inc. is a registered trademark of QWEST Communications International, Inc.

Contents

Document Ownership	ii

Introduction	1

Proposed Implementation	1

Reference	2

# Introduction

The T1.227 ALocationAccessHours attribute is used to allow CMIP Managers to specify the specific hours for each day of the week during which access to the A location is available. 
 
Currently, the following usage of the ALocationAccessHours attribute is defined in all JIAs (Joint Implementation Agreements) between CMIP Managers and QWEST:

QWEST requires a week mask with current day set to a valid value and hours defaulted to between 0:00 and 23:59. QWEST will disregard any A Location Access Hours provided by the Manager and assume the default values of current day and 0:00-23:59 for hours. The current OSS generated commitment time based on the current service condition (Affecting Service or Out of Service) is returned. Arranging premises access due to access restrictions is a manual process.

This technical specification will describe how the usage of the ALocationAccessHours attribute can be extended to specify a four hour appointment for POTS trouble reports that require dispatch.

# Proposed Implementation

If a CMIP Manager wishes to specify a four hour appointment for POTS trouble reports that require dispatch, the appointment can be specified within the ALocationAccessHours attribute in the following manner:

- Set the Sunday bit to 1 in the daysOfWeek bit field.

Note: All other day settings will be ignored.

- In the IntervalsOfDay field, specify the access times:

  - Set intervalStart to the start time for the appointment.

  - Set intervalEnd  to the end time for the appointment.

Note: The start and end times should be entered in the format that the CMIP user is accustomed (i.e. local or GMT format).

Here’s an example of the how the ALocationAccessHours can be used to specify a 9:00am to 1:00pm (local time format) appointment:

T1227:aLocationAccessHours {A.LocationAccessHours {

{

daysOfWeek '1000000'B,

intervalsOfDay {

{

intervalStart {

hour 09,

minute 00

},

intervalEnd {

hour 13,

minute 00

}

}

}

}

### Reference

T1.227-1995, OAM&P Extensions to Generic Network Information Model for Interfaces between Operations Systems across Jurisdictional Boundaries to Support Fault Management (Trouble Administration)