ISDN BRI Optional Feature Ordering Information

| Additional Numbers and Call Appearances Optional Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature | USOC | FID | Definition | Data Information | Data Example |
| Secondary Directory Number | DO6 |  | Used to assign an additional (non-primary) telephone number to a terminal | Can only be assigned after a Primary Directory Number (PDN) has been assigned, and can only be used on a Terminal Type (TTYP) C terminal FIDs Telephone Number (TN), Primary Number (PN), and Digital Services Network Assignment (DSNA) follow the DO6 USOC | DO6 /TN 612 555-1111/PN 612 555-1111/ /DSNA 15.IBSA.123456..NW |
| Multiple Appearance of a TN |  | MADN | Used to indicate whether a shared telephone number is the primary appearance or a subsequent appearance of the telephone number | Required if a PDN or Secondary Directory Number (SDN) appears on another terminal. This would be used on the LTH1X or the DO6 USOC Do not use this FID on the Shared Directory Number (DO8) USOC | /MADN S P R SILENCE Y 6 Y PRIVATE MANUAL |
| Multiple Appearance DN Ring Forward (Automatic) |  | MRF | Used to activate MADN ringing preferences If all members of a MADN group will ring or not ring, the MRF FID is not needed | Used if there is a MADN group and at least one member has Abbreviated or Delayed Ringing Data is: Automatically Activated Y – Yes N – No MRF Timer (required when automatically activated) 0 through 60 (seconds) Ring Type ALWAYS NEVER ABBR DELAY | /MADN S P R SILENCE N/MRF Y 15 ALWAYS /MADN S P R SILENCE N /MRF Y 20 ABBR |
| Multiple Appearance DN Ring Forward (Manual) |  | MRFM | The MRFM is used to activate MADN ringing forward manually. The feature is assigned to a button or a key and activates the ring. | Used if there is a MADN group and at least one member has Abbreviated or Delayed Ringing. There is no data following this FID. | /MRFM |
| Shared Directory Number | DO8 |  | Allows users to share one or more call appearances of a specific directory number | If a DN assigned to a terminal as a PDN or SDN appears on another terminal, it is a DO8 on the other terminal FIDs TN, PN, and DSNA follow this USOC TN and PN must be different telephone numbers – TN is the actual telephone number of the shared telephone number and PN is the telephone number assigned to the terminal. | DO8 /TN 612 555-1111/PN 612 543-1212/DSNA 15.IBSA.144958..NW |
| Appearance Telephone Number |  | ZATN | This FID is used when the telephone number has already been established on another USOC. | Used in Western Region only: Use if there is a Shared Call Appearance (DO8) or an Analog Call Appearance (MAZ) FID ZATN follows the DO8 USOC and FIDs TN, PN, Call Appearance (CAPP), and DSNA follow ZATN. | DO8 /ZATN 206 555-1111 /TN 206 555-1111/PN 206 555-1212/DSNA 4.IBSA.123456..PN |
| Additional Shared Call Appearance, per appearance | AS9 |  | Additional shared call appearance used when a shared directory number will have more than one call appearance. | Used in conjunction with the DO8 provisioning USOC when a shared directory number will have more than one call appearance. FIDs TN and PN follow the AS9 USOC. | AS9 /TN 515 555-1212/PN 515 555-1212/ |
| Additional Call Appearances, per appearance | ACS |  | Used to bill for additional call appearances | FIDs TN and PN follow the ACS USOC | ACS/TN 612 555-1111/PN 612 555-1111 |
| Additional Secondary Directory Number (SDN), per SDN | A6QPN |  | Charges apply for each additional SDN provisioned on a terminal. | Used only on TTYP C terminals when more than one SDN will be assigned to a terminal. FIDs TN and the PN follow this billing USOC | A6QPN /TN 515 555-1212 /PN 515 555-1212 |
| Additional Primary Directory Number (PDN) per number | A6PPK |  | Sometimes called the Digital Subscriber Line (DSL) (USOC N2Q). Each terminal is permitted one PDN. When the customer has additional terminals, the additional PDNs are billable. | Used in conjunction with a Logical Terminal USOC (LTH1X, LNM1X) when the customer has more than two terminals per loop FIDs TN and PN follow the A6PPK USOC. | A6PPK /TN 515 555-1212 /PN 515 515-1212 |
| Analog Call Appearance, per number | MAZ |  | Allows an analog TN to appear on an ISDN terminal. | Used when the appearance of an analog TN on an ISDN terminal is requested FIDs TN, PN, and DSNA follow the MAZ USOC. In the Central and Eastern Regions the TN and PN must be different telephone numbers - the TN entry should reflect the actual TN of the shared telephone number and the PN entry should reflect the PDN assigned to the terminal. Western: FID ZATN follows the MAZ USOC and FIDs TN, PN, CAPP, and DSNA follow ZATN. | Central and Eastern: MAZ /TN 505 555-1212/PN 505 533-2345/DSNA 59.IBSA.144958..NW/KEY 9/MADN S N R SILENCE N/CTX CTX300:0 Western: MAZ /ZATN 206 555-1212 /TN 206 555-1212/PN 206 555-1212/DSNA 4.IBSA.123456..PN/KEY 12/MADN S N R SILENCE N/CTX CTX300:0 |
| Appearance Telephone Number |  | ZATN | This FID is used when the telephone number has already been established on another USOC. | Used in Western Region only Use if there is a Shared Call Appearance (DO8) or an Analog Call Appearance (MAZ) FID ZATN follows the MAZ USOC and FIDs TN, PN, CAPP, and DSNA follow ZATN. | MAZ /ZATN 206 555-1212 /TN 206 555-1212/PN 206 555-1212/DSNA 4.IBSA.123456..PN/KEY 12/MADN S N R SILENCE N/CTX CTX300:0 |

| D Channel Optional Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature | USOC | FID | Definition | Data Information | Data Example |
| X.25 Fast Select Acceptance | GXEPN |  | Authorizes incoming packets from a sending data terminal equipped with Fast Select | FIDs DTN, FS, PN and CLS follow this USOC | GXEPN /DTN 719 636-6757 /PN 719 636-0967/FS IB AY /CLS 29.QSZA.123456..MS |
| X.25 Reverse Charging Acceptance | G5BPN |  | Authorizes transmission of incoming call identified as Reverse Charge calls | FIDs Data Terminal Number (DTN), Fast Select (FS), PN and CLS follow this USOC | G5BPN /DTN 719 636-6757 /PN 719 636-0967/FS IB AY /CLS 29.QSZA.123456..MS |
| Additional X.25 Channel – Per Channel | NW9AL |  | If more than two terminals are associated with a line, additional logical channels may be required for use with the D Channel packet. The D Channel can have up to 15 channels. | This USOC bills for the additional X.25 channel. The Logical Channel Layout (LCL) FID and its data following the D channel logical terminal USOC (LTH4X) provisions the channel. | /LCL 2WL.1, 2WH.2 |

| Voice Optional Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature | USOC | FID | Definition | Data Information | Data Example |
| Redirecting Number Delivery | RD7PN |  | Allows for the display of the first and last call forwarded numbers to be displayed when a call is received from a call forwarded terminal. | DMS-100 only. FIDs TN, PN, and Common Language Circuit ID (CLS) for the PDN follow this USOC | RD7PN /TN 612 717-1556 /PN 612 717-1556/CLS 15.IBSA.123456..NW |
| Calling Name ID – per telephone number | NMCPN |  | Provides the name and telephone number of the calling party to be displayed (or unavailable indicator) at the time of a call | Use when LNID *LIRCMNA is present FIDs, PN and CAPP follow this USOC | NMCPN /TN 425 401-1111 /PN 425 401-1111 |
| Call Forwarding Busy Line – Per Number | NQ5PN |  | Allows all calls to a busy telephone number to be forwarded to any other telephone number, either in or out of the serving CO. | The TN and PN follows the NQ5PN USOC | NQ5PN /TN 307 772-1111 /PN 307 772-1111 |
| Call Forwarding – Don’t Answer – Per Number | NQ6PN |  | Allows all calls terminating to an idle telephone number to be forwarded to another telephone number when the called telephone number does not answer after a predetermined number of seconds. | The TN and PN follows the NQ6PN USOC | NQ6PN /TN 307 772-1111 /PN 307 772-1111 |
| Call Forward Number |  | CFN | The telephone number to which calls are forwarded | FID is followed by the 10-digit TN | /CFN 307 772-1111 |
| Call Forward Number Busy Line |  | CFNB | The telephone number to which calls are forwarded when the called telephone number is busy, and when the CFN number is different than the CFNB number. | FID is followed by the TN If customer has both Call Forward Busy Line (NQ5PN) and Call Forward Don’t Answer (NQ6PN), the CFNB FID follows the NQ5PN USOC | /CFNB 307 772-1111 |
| Call Forward Number Don’t Answer |  | CFND | The telephone number or station number to which calls are forwarded when the called telephone number is busy, and when the CFN number is different than the CFND number. | FID is followed by the TN If customer has both Call Forwarding Busy Line (NQ5PN) and Call Forwarding Don’t Answer (NQ6PN), the CFND follows the NQ6PN USOC. | /CFND 307 772-1111 |
| Ring Cycle for Call Forwarding Features |  | RCYC | Tells the switch how many times the line should ring before the Call Forward Don’t Answer is activated. | Used when the customer has the Call Forwarding Don’t Answer feature FID data is numeric and should be one less than the number of times the customer wants the telephone to ring | /RCYC 3 |
| Call Forwarding Variable – All Calls – Per SDN | NZGPN |  | Allows the customer to forward calls to another telephone number by dialing a code, plus the destination telephone number | The TN and PN information follows the NZGPN USOC | NZGPN /TN 208 242-1111 /PN 208 242-1111 |
| Six-Way Conference – Per Terminal | NZ6PK |  | Allows the user to establish a conference call with up to six conferees | Provisioned by FNM *MW16WB2 FIDs TN, PN, and FNM follow this USOC | NZ6PK /TN 605 423-1111 /PN 605 423-1111/FNM *MW16WB2 |
| One-digit Speed Calling – per terminal | NN8PK |  | Allows user to call pre-programmed telephone numbers by dialing a 1-digit code | FIDs TN and PN follow this USOC | NN8PK /TN 701 823-4444 /PN 701 823-4444 |
| Call Pickup, per number | NZHPN |  | Allows a user to answer a call ringing at another terminal, even when the user’s station does not have a call appearance for the called DN | FIDs TN and PN follow the NZHPN USOC | NZHPN /TN 406 440-4444 /PN 406 440-4444 |
| Call Rejection – Per PDN | FKQPN |  | The name and telephone number associated with this line will not be delivered to caller ID subscriber. | One entry is required per affected number FIDs TN and PN follow this FKQPN USOC | FKQPN /TN 320 252-0967 /PN 320 252-0967 |
| Caller ID Blocking – All Calls – Per User | NDD |  | The name and telephone number associated with this line will not be delivered to caller ID subscriber. | 5ESS only. One entry required per affected PDN FIDs TN, PN and CAPP follow this USOC | NDD /TN 541 252-0957/PN 541 252-0957/CAPP CA.1, NBR.1, ADRPAT.NON |
| Continuous Redial – Per PDN | FKAPN |  | Monitors and redials a busy telephone number for a user, signaling with a distinctive ring alert when the telephone number becomes available. | One entry is required per affected number FIDs TN, PN and CAPP follow this USOC | FKAPN /TN 425 401-1111 /PN 425 401-1111 |
| Last Call Return – Per PDN | FKDPN |  | Allows you to automatically redial the last incoming call. | One entry is required per affected number FIDs TN, PN and CAPP follow this USOC | FKDPN /TN 425 401-1111 /PN 425 401-1111 |
| Selective Call Forwarding – Per PDN | FKEPN |  | Calls from a list of up to 15 telephone numbers will be automatically forwarded to a predefined telephone number. | One entry is required per affected telephone number FIDs TN, PN, Key Designation (KEY) and Local Area Signaling Service (LASS) follow this USOC | FKEPN /TN 425 401-1111 /PN 425 401-1111/KEY 4 /LASS SCF BLGOPT.NOAMA, STAT.INACTIVE RGRM.NA |
| Call Forwarding Simultaneous / Screening Options |  | CFSO | Indicates how many calls this feature will forward at one time. | Used when requesting the Selective Call Forwarding (FKEPN) feature. Data is always: SIM N | FKEPN /TN 425 401-1111 /PN 425 401-1111/KEY 7 /LASS SCF BLGOPT.NOAMA, STAT.INACTIVE RGRM.NA/CFSO SIM N |
| Priority Call – Per PDN | NC8PN |  | Allows a distinctive ring when calls are received from telephone numbers assigned to a special list; maximum of 15 telephone numbers | One entry is required per affected telephone number FIDs TN and PN follow this USOC | NC8PN /TN 425 401-1111 /PN 425 401-1111 |
| Non-Standard Configuration Group Applies per button | N3CPB |  | Associates buttons of a terminal with a Call Appearance or a feature. | FID TN follows this billing USOC | N3CPB /TN 320-252-0967 |

| Hunting Optional B Channel Circuit Switched Data Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature | USOC | FID | Definition | Data Information | Data Example |
| Multi-Line Hunt Service –Circular per B Channel | HDT | TN, HML, HTG, TER | Begins with the telephone number dialed and hunts through all remaining telephone numbers until hunting reaches the dialed telephone number or an idle line or terminal. Provisioned as Multi-Line Hunting in the 5ESS CO switch type, and Directory Number Hunt in the DMS-100 CO switch type. | FIDs TN, HML, and TER are used with HDT USOC HDTPG is also required on first terminal in the hunt group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type The HML FID is also with hunt sequence (TER H1-HX) and *MHDC | I1 LTH1X/TN/HML XXXX/TER HX I1 HDT/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHDC |
| Multi-Line Hunt Service –Circular Per group | HDTPG | TN, HML, HTG, TER | See HDT definition above. USOC applies once per hunt group. | FIDs TN, HML, and TER are used with HDTPG USOC HDT also required on each terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also with the hunt sequence (TER H1-HX) and *MHDC | I1 LTH1X/TN/HML XXXX/TER H1 I1 HDTPG/TN/HML XXXX/TER H1 I1 HDT/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHDC |
| Multi-Line Hunt Service – Sequential, Per B Channel | N2D | TN, HML, HTG, TER | Begins with the lead telephone number hunts sequentially in a predetermined order until an idle line or terminal is found or the end of the hunt group is reached. This is called Multi-Line Hunting in the 5ESS switch type, and Directory Number Hunt in the DMS-100 switch type. | FIDs TN, HML, and TER are used with N2D USOC N2DPG is also required on first terminal in group HML is also required with hunt sequence (TER H1-HX) and *MHDR | I1 LTH1X/TN/HML XXXX/TER HX I1 N2D/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHDR |
| Multi-Line Hunt Service – Sequential, Per group | N2DPG | TN, HML, HTG, TER | See definition for N2D above. | FIDs TN, HML, and TER are used with N2DPG USOC N2D also required on each terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHDR | I1 LTH1X/TN/HML XXXX/TER H1 I1 N2DPG/TN/HML XXXX/TER H1 I1 N2D/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHDR |
| Multi-Line Hunt Service – UCD, Per B Channel | H6U | TN, HML, HTG, TER | Provides equal distribution of incoming calls to idle lines or terminals. Provisioned as Multi-Line Hunting in the 5ESS switch type, and Directory Number Hunt in the DMS-100 switch type. | FIDs TN, HML, and TER are used with H6U H6UPG is also required on first terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also with the hunt sequence (TER H1-HX) and *MHDU | I1 LTH1X/TN/HML XXXX/TER HX I1 H6U/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHDU |
| Multi-Line Hunt Service – UCD, Per group | H6UPG | TN, HML, HTG, TER | See definition for H6U above. | FIDs TN, HML, and TER are used with H6UPG USOC H6U is also required on each terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHDU | I1 LTH1X/TN/HML XXXX/TER H1 I1 H6UPG/TN/HML XXXX/TER H1 I1 H6U/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHDU |

| Hunting Voice Features |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Feature | USOC | FID | Definition | Data Information | Data Example |
| Key Short Hunt, Per group | NHGPG | TN, KSH | A hunt group assigned to only one telephone set. Used to simulate call appearances when TNs are being shared on other telephone sets. When TNs are shared they must be MADNs and MADNs can appear only once on a telephone set. One per group. Used with one NHGPN per appearance. | Available in a DMS-100 CO switch type only. FID TN is used with NHGPG. Also requires presence of NHGPN USOC, and FID KSH following PDN USOC. | I1 LTH1X/TN/PIC//LPIC (etc)/KSH 1-4 I1 NHGPG/TN I4 NHGPN/TN |
| Key Short Hunt, Per telephone number | NHGPN | TN, KSH | Key short hunt per appearance of a telephone number. | Available in a DMS-100 CO switch type only. FID TN is used with NHGPN. Also requires presence of USOC NHGPG, and FID KSH following PDN USOC | I1 LTH1X/TN/PIC//LPIC (etc.)/KSH 1-4 I1 NHGPG/TN I4 NHGPN/TN |
| Multi-Line Hunt Service – Circular, Per telephone number | NZS | TN, HML, HTG, TER | Begins with the telephone number dialed and hunts through all remaining telephone numbers until hunting reaches the dialed telephone number or an idle line or terminal. USOC applies once for each terminal (PDN) in the hunt group. Provisioned as Multi-Line Hunting in the 5ESS and Directory Number Hunt in the DMS-100. | FIDs TN, HML, and TER are used with NZS HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type USOC NZSPG also required on first terminal in group HML FID is also required as left-hand FID with the hunt sequence (TER H1-HX) and *MHCBV | I1 LTH1X/TN/HML XXXX/TER HX I1 NZS/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHCBV |
| Multi-Line Hunt Service – Circular, Per group | NZSPG | TN, HML, HTG, TER | See definition for NZS above. USOC applies once per hunt group. Associated with first terminal in the hunt group. | FIDs TN, HML. and TER are used with NZSPG HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type USOC NZS also required on each terminal in group HML is also with the hunt sequence (TER H1-HX) and *MHCBV | I1 LTH1X/TN/HML XXXX/TER H1 I1 NZSPG/TN/HML XXXX/TER H1 I1 NZS/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHCBV |
| Multi-Line Hunt Service, Sequential, Per telephone number | NZQ | TN, HML, HTG, TER | Begins with the lead telephone number hunts sequentially in a predetermined order until an idle line or terminal is found or the end of the hunt group is reached. USOC applies once for each terminal (PDN) in the hunt group. Provisioned as Multi-Line Hunting in the 5ESS and Directory Number Hunt in the DMS-100. | FIDs TN, HML. and TER are used with NZQ USOC NZQPG also required on first terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHRBV | I1 LTH1X/TN/HML XXXX/TER HX I1 NZQ/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHRBV |
| Multi-Line Hunt Service Sequential, Per group | NZQPG | TN, HML, HTG, TER | See definition for NZQ above. USOC applies once per hunt group. Associated with first terminal in the hunt group. | FIDs TN, HML. and TER are used with NZQPG USOC NZQ is also required on each terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHRBV | I1 LTH1X/TN/HML XXXX/TER H1 I1 NZQPG/TN/HML XXXX/TER H1 I1 NZQ/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHRBV |
| Multi-Line Hunt Service- Uniform Call Distribution, Per telephone number | NZT | TN, HML, HTG, TER | Provides equal distribution of incoming calls to idle lines or terminals. USOC applies once for each terminal (PDN) in the hunt group. Provisioned as Multi-Line Hunting in the 5ESS and Directory Number Hunt in the DMS -100. | FIDs TN, HML. and TER are used with NZT USOC NZTPG also required on first terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHUBV | I1 LTH1X/TN/HML XXXX/TER HX I1 NZT/TN/HML XXXX/TER HX HML XXXX TER H1-HX *MHUBV |
| Multi-Line Hunt Service- Uniform Call Distribution,Per group | NZTPG | TN, HML, HTG, TER | See definition for NZQ above. USOC applies once per hunt group. Associated with first terminal in a hunt group. | FIDs TN, HML. and TER are used with NZTPG USOC NZT also required on each terminal in group HML is used in a 5ESS CO switch type HTG is used in a DMS-100 CO switch type HML is also required with the hunt sequence (TER H1-HX) and *MHUBV | I1 LTH1X/TN/HML XXXX/TER H1 I1 NZTPG/TN/HML XXXX/TER H1 I1 NZT/TN/HML XXXX/TER H1 HML XXXX TER H1-HX *MHUBV |
| Series Completion Hunt, Per group | SE3PG | TN, HTG | Begins with the lead telephone number and hunts sequentially in a predetermined order until an idle line or terminal is found or the end of the hunt group is reached. USOC applies once per hunt group. Associated with the lead telephone number in the group. Some CPE will not work with multi-line hunting and requires series hunting. | Available in a 5ESS CO switch type only FID TN is used with SE3PG USOC SE3PN also required on each line in hunt group The SE3PG USOC is followed by the FID HTG and the hunt group letter. HTG is also required with hunt sequence. | I1 LTH1X/TN/HTG A I1 SE3PG/TN/HTG A I1 SEP3N/TN/HTG A HTG A 321-8489-8502 |
| Series Completion Hunt, Per telephone number | SE3PN | TN, HTG | See definition for SE3PG above USOC applies once per telephone number in the hunt group. | FID TN is used with SE3PN USOC SE3PN also required on each line in hunt group FID HTG with the hunt group letter HTG also required with the hunt sequence | I1 LTH1X/TN/HTG A I1 SEP3N/TN/HTG A HTG A 321-8489-8502 |