ATTACHMENT 3:

xDSL CAPABLE LOOP PERFORMANCE PARAMETER TESTS

Note:  As between Attachment 1 and Attachment 3, the terms of Attachment 1 control, should any discrepancy or apparent discrepancy be identified.  See Attachment 1 regarding Conditioning.

| Required Tests | Expected Field Measurement Results | Notes |
| --- | --- | --- |
| Loop Length | Actual (Capacitive) |  |
| Load Coils | None |  |
| Opens | None |  |
| Grounds | None |  |
| Shorts | None |  |
| Bridge Tap | LX-N Maximum: Total Length <2500 ft Single Tap Length < 2000ft No Near End /Far End BT( >1000 ft) LXR- Maximum: Total Length <2500 ft Single Tap Length < 2000 ft No Near End /Far End BT( >1000 ft) Remove All Maximum: None | See Exclusions |
| 1004 Hz Loss | < -8.5dBm |  |
| 196 kHz Loss | Actual Measured Loss (AML): Maximum AML = EML + 5 dB LX-N Maximum dB Loss: 2- wire (e.g., NCI codes of 02QB9.00H and 02QB5.00G) <28.dB 4- wire (e.g, NCI codes of 04QB9.00H, 04QB5.00G, and 04QB9.00F) <31.dB LXR- Maximum dB Loss: LXR- <78.dB | <78 dB if such limit is within test set capability |
| 40 kHz Loss | ISDN BRI <40.dB |  |
| Insulation Resistance | Tip – Ground > 3.3 Meg Ohms Ring – Ground > 3.3 Meg Ohms Tip – Ring > 3.3 Meg Ohms |  |
| Foreign Voltage - DC | Tip – Ground < 8 VDC Ring – Ground < 8 VDC Tip – Ring < 8 VDC |  |
| Foreign Voltage - AC | Tip – Ground <50VAC Ring to Ground <50VAC |  |
| Noise (C – Message) | < 23 dBrnC Far end 600 Ohm Termination | < 20 dBrnC Acceptable, >20 < 30 dBrnC Marginal, > 30 Unacceptable |
| Noise ( C – Notch) | < 45 dB | 1004 Hz, 0 dBm Transmit |
| Line Balance | < to 10% | The length of the Tip side of the line compared to the length of the Ring to 10% difference |
| Longitudinal Balance | 965 Type Meter <= 50 dB @ 196khz Other Meters <= 40 dB @ 196khz |  |
| Power Influence | <=90 dBrnc |  |
| D-Mark Tagged | Yes |  |