MEDIACC/EBTA

CenturyLink™

Implementation Guidelines

December 15, 2011

For questions regarding this document, please contact:

Wholesale System Help Desk
1-888-796-9102

Abstract

This document will assist CLEC/IXCs in understanding and successfully managing the process of implementing MEDIACC/EBTA trading capabilities between their organization and CenturyLink. The information in this document is specific to the use of the MEDIACC/EBTA system and should not be construed as being applicable to other systems available from CenturyLink.

Document History

All revisions made to this document are listed here in chronological order.

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 04/15/2005 | Initial Distribution |
| 2.0 | 02/20/11 | Contact Updates |
| 3.0 | 12/15/11 | Clarify contact phone number. Rebrand from Qwest to CenturyLink. |

Copyright and Trademarks

 2011 CenturyLink, Inc. All Rights Reserved. The CenturyLink mark, pathways logo and certain CenturyLink product names are the property of CenturyLink, Inc. All other marks are the property of their respective owners.

CenturyLink Information Technology Services

Implementation Guidelines – MEDIACC/EBTA

TABLE OF CONTENTS

1.	Introduction	8

Purpose of Document	8

Scope of Document	8

Audience	8

2.	Implementation Activities	10

Initial Communications	10

Initial Communications Activities	11

Initial Communications Phase Completion	11

Project Plan Development/Negotiation	11

Typical Implementation Timeline	12

Requirements Review	12

Requirements Review Activities	12

CenturyLink Documentation Overview	13

Establishing a Dedicated Circuit and Connectivity Testing	13

Establishing a Dedicated Circuit and Connectivity Testing Process Flow	14

Test Plan Development and Approval	16

Test Plan Review Approval Process	16

Interoperability Testing Phase	16

Interoperability Testing Activities	17

Supported Transactions – Interoperability	17

Stack-to-Stack Testing	17

Conformance Testing	17

Controlled Production	18

Network Validation Testing	18

Operational Readiness Testing (ORT)	19

3.	Business Functions	20

Definition of Functions	20

Create Trouble Report (Enter Trouble Report)	20

Request Trouble Report Status (GET)	20

Add Trouble Information (SET)	21

Modify Trouble Information (SET)	21

Status Change Event Notification	21

Close Out	21

Cancel Trouble Report	24

Request MLT Test (Service Test Function)	24

Attribute Implementation Definition	24

Initial Implementation Exceptions	25

Attributes	25

Manual Processes	25

Critical Attributes for Flow-Through	26

Error Conditions	31

UNE/ACCESS/CARRIER only:	32

4.	Communication Protocol	33

Protocol Profile	33

Access Topology	33

CMIP/OSI Implementation Definition	34

Access	34

Security	34

Connectivity	34

Transport Layer	35

Manager/Agent Entity	35

Manager Entity	35

Agent Entity	35

5.	Security	36

CenturyLink/CLEC/IXC Customer Responsibilities	36

Association Mechanism	36

Login/Key Assignment Process	36

6.	Production Availability	38

CenturyLink MEDIACC/EBTA Gateway:	38

LMOS:	38

WFA/C:	38

7.	Recovery Procedures	39

Off-Nominal Status	39

CenturyLink Process	39

CLEC/IXC Process	39

Electronic Recovery	39

CenturyLink Process	39

Manual to Electronic Recovery	40

CLEC/IXC Process	40

EBTA Single Point of Contact	40

8.	Other	42

T1.227 Implementation Levels	42

Name Binding	42

Repeat Report	42

9.	Appendix A - Attribute Definitions	43

A Location Access Address	43

A Location Access Hours	43

A Location Access Person	43

Activity Duration	43

Additional Trouble Information List	44

Additional Trouble Status Information	44

Agent Contact Person	45

Authorization List	45

Called Number	45

Cancel Requested By CLEC/IXC	45

Close Out Narrative	45

Commitment Time	45

Commitment Time Request	46

Customer Trouble Ticket Number	46

Escalation List	46

Maintenance Service Charge	46

Managed Object Access Hours	46

Managed Object Instance	46

Manager Contact Person	47

Managed Object Referring to Test	47

Outage Duration	47

Perceived Trouble Severity	47

Preferred Priority	47

Received Time	48

Repeat Report	48

Restored Time	48

Trouble Clearance Person	48

Trouble Found	48

Trouble Report Format Object Pointer	49

Trouble Report Identifier	49

Trouble Report State	49

Trouble Report Status	49

Trouble Report Status Time	49

Trouble Report Status Window	49

Trouble Type	50

TSP Priority	50

10.	Appendix B - Managed Object Conformance Statements (MOCS)	51

POTS Resale MOCS Table for Initial Deployment	51

Legend for POTS Resale MOCS Table for Initial Deployment	52

UNE/ACCESS/CARRIER MOCS Table for Initial Deployment	52

Legend for UNE/ACCESS/CARRIER MOCS Table for Initial Deployment	54

11.	Appendix C - Fallback Error Conditions	55

POTS Resale and UNE/ACCESS/CARRIER:	55

UNE/ACCESS/CARRIER:	55

12.	Appendix D - EB TA Process Management	56

13.	Appendix E - References to Other Documents	57

References	57

14.	Appendix F - Data Definitions and Mappings	58

Data Definitions and Mappings for both POTS Resale and UNE/ACCESS/CARRIER	58

Standard Abbreviations	58

Data Definitions and Mappings for POTS Resale	66

Status	66

Status Messages	66

Trouble Type to Line In Use	66

Disposition Code to Trouble Found	70

Disposition Code to Maintenance Service Charge	77

Non-Applicable Trouble Types for Local Resale (POTS)	78

CenturyLink will not create an electronic trouble report for these trouble types:	78

Data Definitions and Mappings for UNE/ACCESS/CARRIER	80

Status	80

Status Messages	81

UNE/ACCESS/CARRIER Trouble Found Codes	81

Non-Applicable Trouble Types for UNE/ACCESS/CARRIER	82

Applicable Trouble Types for Local Number Portability (LNP)	83

15.	Appendix F - Glossary/Acronyms	85

16.	Appendix G - Managed Object Instance Formats	87

POTS Resale Formats	87

Telephone Number Format	87

Telephone Number with Terminal Format	87

UNE/ACCESS/CARRIER Formats	87

Unbundled Loop Circuit Formats	87

Unbundled Switch Circuit Formats	87

17.	Service Test Function	88

Implementation	88

Error Messages – See Section 4 of ANSI T1.262-1998	88

# Introduction

Purpose of Document

This document outlines the information to be exchanged as part of the mechanized Electronic Bonding (EB) interface for Trouble Administration and Maintenance (MEDIACC) between CLEC/IXC Customer and CenturyLink. Both parties agree to work in good faith towards a successful implementation. This document is not a contract, but rather the requirements for implementing MEDIACC/EBTA with CenturyLink. This document not only outlines the responsibilities of both companies, but also details the Attributes that will be supported, the business functions that will be allowed, and issues concerning security, support, and fallback at initial deployment.

This Implementation Guide is based on the ANSI T1M1.5 227/228 1995 standards for Trouble Administration for the electronic exchange of information between CLEC/IXC Customer (CLEC/IXC) and CenturyLink MEDIACC system and T1.262-1998 for Service Test Function.

Scope of Document

The scope of this Implementation Guide is the Maintenance and Repair functions supporting POTS (Plain Old Telephone Service) Resale, Unbundled Network Loop Elements and Unbundled Network Switch Elements for residential service in Legacy Qwest’s 14 state territory.

MEDIACC is available for use only in legacy Qwest territories in the following states:

| Arizona Colorado Idaho Iowa Minnesota | Montana Nebraska New Mexico North Dakota Oregon | South Dakota Utah Washington Wyoming |
| --- | --- | --- |

In this document, UNE/ACCESS/CARRIER means designed circuits handled by WFA/C. Also for Carrier and Access customers, this document applies to their services which will mainly be WFA/C (Designed) circuits. The processes for handling UNE/ACCESS/CARRIER will be the same for Carrier and Access circuits.

Audience

This document is intended for organizations wishing to implement wholesale trading capabilities with CenturyLink for the exchange of MEDIACC/EBTA transactions via Common Management Information Protocol (CMIP). It also contains information regarding the CenturyLink test systems and daily production maintenance windows for CenturyLink downstream systems which may impact tickets sent to CenturyLink during those times.

# Implementation Activities

Electronic Bonding is real-time and secure electronic exchange of data between information systems in separate companies. Electronic Bonding is strict adherence to both International and National standards. These standards define the communication and data protocols allowing all organizations in the world to exchange information. The MEDIACC/EBTA Implementation Process will progress according to an agreed upon plan and timeline. Implementation of MEDIACC/EBTA is a detailed and lengthy process involving CenturyLink and the wholesale customer. The steps involved in the implementation process include:

- Initial Communications: During this phase, all activities to initiate a CLEC/IXC implementation are conducted, including the Kickoff conference call.

- Implementation Project Plan Negotiation: During this phase, the Implementation Project Plan is proposed and negotiated.

- Requirements Review: The Requirements Review phase provides an opportunity for a CLEC/IXC to review ANSI T1M1.5 227/228 1995 standards, ANSI T1.262-1998 standards, and CenturyLink’s Attribute definitions/business requirements. This also gives the CLEC/IXC the opportunity ask any questions they may have regarding those requirements.

- Stack to Stack/Network Validation Connectivity Testing: During this phase, the test environment will be established with any necessary software and table changes. The CLEC/IXC connectivity is established and tested for both Interoperability and Controlled Production.

- Interoperability Testing: This phase affords the CLEC/IXC the opportunity to validate their technical development efforts and to quantify MEDIACC/EBTA processing results. Interoperability testing is comprised of Gateway-to-Gateway and End-to-End.

- Controlled Production: This phase consists of the controlled submission of CLEC/IXC requests to the CenturyLink production environment for provisioning as production orders. CenturyLink and the CLEC/IXC use Controlled Production results to determine operational readiness for full Production turn-up.

- Production: The CLEC/IXC is certified and able to submit full volumes of production trouble tickets.

## Initial Communications

This section explains the objectives of the Initial Communications activity regarding MEDIACC/EBTA trading capabilities. The Initial Communications discussion is intended to accommodate:

- A general description of the MEDIACC/EBTA Gateway

- An overview of the MEDIACC/EBTA Implementation Process

- Identification of applicable MEDIACC/EBTA documentation resources

Each CLEC/IXC will be provided with a CenturyLink Implementation Team or a Single Point of Contact (SPOC), who will perform all the roles noted below. The SPOC/Team members will work with the CLEC/IXC throughout the implementation. The Implementation Team roles are:

- Project Manager (PM): Responsible for facilitating and document weekly meetings, coordinating overall Project Plan, and is the primary point of contact for CLEC/IXC implementation process.

- Business Analyst (BA): Responsible for providing assistance with the business rules and processes associated with MEDIACC/EBTA.

- Technical Support Engineer (TSE): Responsible for facilitating the implementation processes from a systems standpoint and testing.

## Initial Communications Activities

When a CLEC/IXC is interested in implementing MEDIACC/EBTA, the CenturyLink Service Manager will put them in contact with the Lead Implementation Project Manager (LPM) who will then assign an Implementation Project Manager (PM) and a kickoff call will be scheduled. The following activities will occur as part of the project initiation meeting (Kickoff call) between CenturyLink and the CLEC/IXC:

- Provide overview of the MEDIACC/EBTA Gateway

- Review of CenturyLink Data Transport Requirements

- Introduce CenturyLink and CLEC/IXC team members and identify roles and responsibilities

- Identify the objectives and scope

- Review the Implementation Guidelines and the Implementation Processes

- Review documentation resources

- Discuss administrative processes

## Initial Communications Phase Completion

The initial communications phase will be considered complete with the initiation of weekly project status calls with the CLEC/IXC.

## Project Plan Development/Negotiation

A formal Implementation Project Plan that represents the milestones and responsibilities of all parties throughout the planned implementation process will be mutually negotiated. Regularly scheduled conference calls will be conducted to review progress, answer questions, and identify and communicate resolution of issues. The respective project managers will be responsible for maintaining and monitoring progress within the Project Plan. A mutual negotiation and approval process will be used to manage changes to schedules as recommended or required by either party. All parties will give sufficient advance notice to the other party to allow adequate analysis of the impact of any proposed schedule changes.

The inability of either party to meet any of the milestones contained in the Implementation Project Plan on the date(s) identified may result in the need to renegotiate a revised Project Plan. All revised plans will be developed based on the availability of resources from within the respective organizations and may contain dates that are substantially different from those described in the original plan.

The development of, and agreement to, a formal Implementation Project Plan is the pre-requisite to beginning the Implementation Process.

### Typical Implementation Timeline

Actual implementation duration will depend on the number of tickets to be sent, resource availability, project scope, and other factors. Cut over to production will be scheduled around CenturyLink minimal change periods.

#### Project Plan Negotiation Phase Completion

This phase of the implementation will be considered complete with the approval of an agreed upon project plan.

## Requirements Review

A critical factor in a successful implementation is a thorough understanding by the CLEC/IXC of the MEDIACC/EBTA Gateway. The requirements review phase of implementation will provide the CLEC/IXC with an opportunity to develop a clear understanding of CenturyLink requirements for MEDIACC. This review is critical to the CLEC/IXC who must:

- Develop and define the business processes and procedures necessary to support the use of their CMIP interface for MEDIACC transactions.

- Develop the appropriate documentation (i.e., Methods and Procedures) necessary to support the use of the CMIP interface by CLEC/IXC personnel.

- Perform any necessary data analysis for the purpose of ensuring that all data fields that carry the EBTA information can be successfully populated.

- Identify appropriate data values.

### Requirements Review Activities

While it is the CLEC/IXC’s responsibility to perform a thorough review of all CenturyLink requirements, CenturyLink will provide support to clarify issues and answer questions regarding the MEDIACC/EBTA business rules, requirements, and the Implementation Process. CenturyLink recommends the following process to ensure that the CLEC/IXC has a thorough understanding of the answers to each question.

- The CLEC/IXC reviews CenturyLink requirements as documented in the Appendix B and ANSI T1M1.5 227/228 1995 standards for Trouble Administration for the electronic exchange.

- Questions arising from the CLEC/IXC’s review of MEDIACC/EBTA requirements will be captured on a Question Log by the CLEC/IXC.

- The Question Log, with questions populated by the CLEC/IXC, will be submitted to CenturyLink.

- CenturyLink will either provide a response to the question or identify those questions requiring further investigation and research. CenturyLink has 5 business days to provide this initial response. Responses are typically communicated during the weekly meeting but may be given via e-mail or phone outside of the meeting.

CenturyLink will provide answers to outstanding questions and the CLEC/IXC will update the log to reflect their understanding of the answers provided.

CenturyLink will review the answers entered by the CLEC/IXC in the log after which the CLEC/IXC may indicate that a question may be closed if it has been adequately answered

If this process becomes cumbersome to a CLEC/IXC, the process can be negotiated to allow the CLEC/IXC and CenturyLink to work together most efficiently while maintaining a CLEC/IXC’s ability to thoroughly understand the answers to the questions.

#### Requirements Review Phase Completion

The Requirements Review phase of the implementation will be considered complete when the CLEC/IXC determines that they have adequately reviewed CenturyLink’s requirements documentation. The CLEC/IXC may decide to continue use of the Question Log throughout the implementation.

### CenturyLink Documentation Overview

The following documents provide additional information on the Implementation Process, MEDIACC requirements, or are designed to capture necessary business and configuration parameters. Distribution of these documents will occur as needed during the course of the implementation project.

Distributed prior to the kickoff call:

- Kickoff Agenda

- Generic Project Plan

- Contact List Template

- CenturyLink and Industry Web site URLs

Distributed after the kickoff call:

- Test Plan Template

Distributed during the Interoperability Testing phase:

- CenturyLink MEDIACC/EBTA Troubleshooting Guide

## Establishing a Dedicated Circuit and Connectivity Testing

A dedicated circuit is needed before implementing the CenturyLink MEDIACC/EBTA interface. This will provide a secure network to send and receive data between CenturyLink and the CLEC/IXC, providing an efficient means to transport high volumes of data.

The same circuit can be used to connect to the production environment, the interoperability environment, and other CenturyLink gateways simultaneously.

It is feasible to share a circuit already being used between CenturyLink and a CLEC/IXC (or a Service Bureau). If a circuit is shared, it is critical for the circuit owner to monitor the transmission volumes as performance issues could be impacted. CenturyLink will require a letter of authorization (LOA) from the circuit owner when a circuit is shared.

### Establishing a Dedicated Circuit and Connectivity Testing Process Flow

Figure: Establishing a Dedicated Circuit and Connectivity Testing Activities

#### Establish a Dedicated Circuit

The dedicated circuit can be ordered from the carrier of the CLEC/IXC’s choice. CenturyLink is able to accommodate a Frame Relay circuit or a point-to-point connection. Currently, CenturyLink does not support a dial up application, VPN or an ISDN circuit for the MEDIACC. The bandwidth requirements for connectivity to the MEDIACC interface are a T-1, fractional T-1, or 56K line. Determining the bandwidth sizing is a function of estimating the number of concurrent users accessing an application, the number of applications riding on the facility, and the contentions for access between those ‘user instances’. CenturyLink recommends that a facility engineer be consulted to determine the bandwidth size required to accommodate the CLEC/IXC’s specific needs.

CenturyLink requires that the CLEC/IXC’s circuit terminate at the CenturyLink data center demarcation point in Omaha, Nebraska. When a CLEC/IXC or Service Bureau requires a new circuit to be installed at the CenturyLink data center, they would need to populate the Data Network Information and Requirements Form (http://www.centurylink.com/wholesale/clecs/electronicaccess.html).

If further circuit information is needed, please contact your CenturyLink Implementation team.

#### Establishing a Dedicated Circuit and Connectivity Testing Phase Completion

This phase will be considered complete with the completion of the connectivity. Each test will be considered complete when all of the exit criteria are satisfied.

## Test Plan Development and Approval

During Interoperability Testing and Controlled Production, the set of required Scenarios that are to be tested will be mutually agreed upon and represented by the CLEC/IXC on a Test Plan. The following bullets indicate the activities to be performed during this process depending upon the testing phase and environment utilized.

For Interoperability Testing and Controlled Production, CLEC/IXCs will submit a Test Plan using an adequate description of the test case and expected results. CenturyLink will review the Test Plan with the CLEC/IXC making the corrections needed for approval. (See ‘Scenario Creation’ in Interoperability and Controlled Production sections).

### Test Plan Review Approval Process

The Test Plan review process for Interoperability Testing and Controlled Production will occur as follows:

- The CLEC/IXC populates the Functional Test Scenarios document for MEDIACC/EBTA and will act as the Test Plan. Only those test cases that pertain to the business needs for the CLEC/IXC will be required. The CLEC/IXC should ensure that the Test Plan includes transactions that will meet the minimum testing criteria for the phase of the test. These criteria are detailed in the Test Plan completion section of the Interoperability Testing or Controlled Production section of this document.

- CenturyLink determines whether the CLEC/IXC has included appropriate scenarios on the Test Plan to meet the minimum requirements for Interoperability Testing, Controlled Production. The BA then reviews the Test Plan comments with the CLEC/IXC.

- The CLEC/IXC corrects the Test Plan based upon any feedback from CenturyLink and resubmits it to CenturyLink for review.

- Tasks 2 and 3 repeat until the Test Plan is correct and approved by CenturyLink.

## Interoperability Testing Phase

The Interoperability Testing Phase affords the CLEC the opportunity to validate their technical development efforts and to quantify MEDIACC/EBTA processing results. Interoperability Testing will identify the CLEC/IXC’s ability to submit correct trouble tickets through the MEDIACC/EBTA system

### Interoperability Testing Activities

The MEDIACC/EBTA interoperability environment is a like production environment of MEDIACC/EBTA

For the scenario to be successfully processed in the Interoperability environment, the CLEC/IXC must supply valid account data. This means that the account information must exist within the CenturyLink systems and be valid in terms of its account owner (CenturyLink or CLEC/IXC), account number, status, end user, related content and format.

In the MEDIACC/EBTA test environment, Interoperability Testing involves the CLEC/IXCs using their own account data for submitting a minimum set of test scenarios for all transaction types they wish to perform in Production.

The accounts used by a CLEC/IXC in the Interoperability Testing Phase will be left intact in CenturyLink production systems technicians will not be sent to work the trouble reports.

### Supported Transactions – Interoperability

All transactions supported by the production environment are supported in the Interoperability environment. The different types of testing that need to be performed are specified below:

### Stack-to-Stack Testing

Stack to Stack testing is only required if a new server is introduced, if new TCP/IP circuitry is provisioned, or if a new Data Center is introduced for the EB Gateway. If an upgrade to an operating system plus the turn-up of CLEC/IXC CUSTOMER Local occurs concurrently, a Stack-to-Stack test will be required. The Stack level to test is TCP/IP data at the Network Level. Both sides must conduct Connect and Server requests by the use of Network Utility Software, the use of CLEC/IXC/CenturyLink Application Software to establish an Association may be requested by either party.

### Conformance Testing

Conformance testing is an internal testing requirement and should be conducted according to the ECIC Trouble Administration Testing Specification.

Interoperability Testing

Interoperability testing consists of the following and may be entered into once the Stack-to-Stack testing is complete:

- Gateway to Gateway

- End-to-End

- Network Validation Testing (NVT)

- Operational Readiness Testing (ORT)

All testing must be coordinated with the CenturyLink TSE/SPOC.

#### Gateway-to-Gateway Testing

The Gateway-to-Gateway interoperability testing verifies that the two OSI gateways can communicate the TA application successfully. The test plans that cover the Gateway-to-Gateway testing scenarios will be jointly developed between CLEC/IXC CUSTOMER and CenturyLink. The use of simulators or test platforms behind the associated interface is permitted.

Gateway-to-Gateway testing is the validation of the basic business functions: Association (including Security), Create, Get, Set (Add-Modify), Set Cancel, Set Closeout, and Event Notifications.

#### End-to-End Testing

Once Gateway-to-Gateway testing is considered complete then the CLEC/IXC may enter in to the End-to-End phase. The End-to-End interoperability testing verifies that the EBTA application information can be successfully exchanged between CLEC/IXC CUSTOMER and CenturyLink. The End-to-End interoperability testing involves the laboratory versions of the CLEC/IXC CUSTOMER OSS and the CenturyLink OSS communicating through the CLEC/IXC CUSTOMER Gateway and the CenturyLink Gateway. The test plans that cover the End-to-End testing scenarios will be jointly developed between CLEC/IXC CUSTOMER and CenturyLink. End-to-End exit criteria will include the completion of a Load and Soak test. The use of simulators is not permitted. Test platforms that are copies of Production systems may be used.

## Controlled Production

Once End-to-End testing is considered complete then the CLEC/IXC may begin the Controlled Production Phase. Controlled Production may last several weeks. All testing must be coordinated with the CenturyLink SPOC.

### Network Validation Testing

The Network Validation Testing (NVT) verifies that there is connectivity in the Production Environment between the CLEC/IXC CUSTOMER OSS - CenturyLink OSS (CLEC/IXC CUSTOMER OSS - CLEC/IXC CUSTOMER Gateway, CLEC/IXC CUSTOMER Gateway - CenturyLink Gateway, CenturyLink Gateway - CenturyLink OSS). Communications can be verified by bringing up the association and performing sanity testing. Participants in Sunny Day testing may include Software Testers and Work Center personnel. Verification of EB functionality can be done by creating a test ticket to verify referral (create), status, and closure to the ticket. NVT exit criteria can be used as a Quality Gate for entrance into Operational Readiness Testing (ORT). The test plans that cover NVT will be jointly developed between CLEC/IXC CUSTOMER and CenturyLink.

### Operational Readiness Testing (ORT)

The ORT testing phase may begin once the NVT testing is considered complete. The Controlled Production verifies EB functionality in the Production Environment between CLEC/IXC CUSTOMER and a CenturyLink Work Center. Controlled Production scenarios will utilize the Referral (create), Modify, Event Notification, and Closeout functions. ORT exit criteria can be used as a Quality Gate for entrance into production.

This phase is designed to trial the Maintenance and Repair process on real trouble reports. The purpose of Controlled Production is to validate the following in a controlled production environment:

- User Requirements

- End-to-End Functionality

- Work Center Methods and Procedures

- Operations Procedures

- Process Validation

- Business Case Metrics

- Data Quality

- Measures of Success

- Production Support Procedures

- System Security Requirements

# Business Functions

This section defines agreements, clarifications, or exceptions for business functions, Attributes, and processes.

## Definition of Functions

This section defines the business transactions which are agreed will be supported via Electronic Bonding. See Appendix B, Managed Object Conformance Statements for details.

### Create Trouble Report (Enter Trouble Report)

POTS Resale: The request to create a trouble report originates from the CLEC/IXC. The CLEC/IXC issues a PT-CREATE operation to create the trouble report in CenturyLink’s OSS. The CLEC/IXC is required to supply mandatory Attributes as defined in TRFD 1. Once the create request is received, CenturyLink is required to respond through the CenturyLink Gateway. If CenturyLink receives the create request and an error condition exists, CenturyLink will respond with the appropriate CMIP defined error message. CenturyLink is required to validate circuit ownership before transmitting the create response. CenturyLink will perform an address verification during the create process and mismatches will be indicated in the OSS. The trouble report will then be routed to the appropriate group for resolution. CenturyLink will perform a TSP comparison during the create process and mismatches will be indicated in the OSS. The trouble report will then be routed to the appropriate group for resolution. The Trouble Type Attribute will determine if an initial MLT test is run (See Appendix F, Data Definitions and Mappings for details).

UNE/ACCESS/CARRIER/Access/Carrier: AutoTest is determined by circuit type. If the circuit is not a candidate for AutoTest, this function will not be performed.

### Request Trouble Report Status (GET)

POTS Resale: This transaction is used by the CLEC/IXC to obtain status information from CenturyLink about a previously entered trouble report. It can also be used to obtain values for certain other Attributes in a trouble report. The CLEC/IXC issues a PT-GET and CenturyLink will respond with either the Attributes that were provided on the list or, if no Attributes were provided, CenturyLink will return the current values of all instantiated Attributes.

All the required data to respond to a customer request, PT-GET, comes from local CenturyLink gateway data stores. Some of the CenturyLink gateway data values are derived from host screens.

UNE/ACCESS/CARRIER: Same as POTS Resale.

### Add Trouble Information (SET)

POTS Resale: A request to add information to an existing trouble report originates from the CLEC/IXC. The CLEC/IXC issues a PT-SET only for add/list type Attributes. When CenturyLink receives the request, it updates the information on the trouble report. The LMOS OSS is severely limited on the Attributes that it can update. Currently, only the Attribute Additional Trouble Info List will be supported.

UNE/ACCESS/CARRIER: Supported according to the UNE/ACCESS/CARRIER MOCS table. See Appendix B, Managed Object Conformance Statements for MOCS table details.

### Modify Trouble Information (SET)

POTS Resale: A request to modify information on an existing trouble report originates from the CLEC/IXC. The CLEC/IXC issues a PT-SET only for modifiable Attributes. When CenturyLink receives the request, it updates the information on the trouble report. The LMOS OSS is severely limited on the Attributes that it can modify. Currently, only the Attribute Perceived Trouble Severity will be supported.

Commitment time will not be changed when electronically modifying a trouble report (a subsequent trouble report). If a commitment is missed, the OSS needs a new commitment to create the subsequent report. As a result, a Modify transaction will not be accepted in this case. CenturyLink handles this manually and negotiates a new commitment with the customer. If a commitment is missed and the CLEC/IXC has requested a SET Modify, a resource limitation/processing error will be returned.

UNE/ACCESS/CARRIER: Supported according to the UNE/ACCESS/CARRIER MOCS table. See Appendix B, Managed Object Conformance Statements for MOCS table details.

### Status Change Event Notification

POTS Resale: A status change notification (Event Notification) originates from CenturyLink. CenturyLink issues an Event Notification when the status of a trouble report changes to notify the CLEC/IXC of the status change. CenturyLink can also issue a “no status change” Event Notification to send text messages (remarks) to the CLEC/IXC. See Appendix F, Data Definitions and Mappings for status details.

UNE/ACCESS/CARRIER: Supported. See Appendix B, Managed Object Conformance Statements for MOCS table details.

### Close Out

#### POTS Resale Closed Out Event Notification

POTS Resale (LMOS) trouble report closure is a two step process (Cleared and Closed). Cleared information is contained in the first event, and Closed information is contained in the second event.

##### POTS Resale Cleared Event

CenturyLink first sends a normal event containing Cleared state (3), or Cleared Customer Advised (CCA) status (25), the time the trouble ticket was restored (Trouble Report Status Time), and a message (Additional Trouble Status Info). The TR-Change-Denied rules will be applied after the Cleared event notification occurs (CCA).

##### POTS Resale Closed Event

The second event is a closed transaction that uses the “cleared awaiting customer verification” data structure, but it is not verification on closeout transaction. CenturyLink sends the second transaction to the CLEC/IXC when the trouble report is closed, and a response is not needed, expected, or waited for to continue processing. The transaction contains Closed state (4), Closed Out status (27), plus other information such as Trouble Found, Maintenance Service Charge, and Outage Duration in separate Attributes. Disposition code and text, and Cause code and text (explaining what CenturyLink found the trouble to be and what caused the trouble) will be found in Close Out Narrative. CenturyLink sends the closed transaction as soon as the information is posted in its OSS, so there may be a time interval between the Cleared and Closed events. Once the Closed transaction is sent to the CLEC/IXC, the object is deleted in the CenturyLink’s gateway. If the CLEC/IXC sends any further transactions on this trouble ticket, CenturyLink will send a “no such object instance” error.

Note: There may be a time interval between the Cleared event and the Closed event. If the CLEC/IXC requests status during this interval, the Cleared status information is returned. If the CLEC/IXC empts to request a PT-SET (e.g., Cancel) during this interval, the TR-Change-Denied rules will be applied.

Details of the closed transaction Attributes are:

| Attribute | Value | Description |
| --- | --- | --- |
| ActivityDuration | ActivityType HH:MM | Zeroes |
| CloseOutNarr | Text | Disposition code and definition, and cause code and definition. |
| MaintServiceCharge | Y/N | Y = based on disposition codes 1230, 1240, 1250, 1260 |
| OutageDuration | HH:MM | restoredTime less receivedTime |
| RestoredTime | GMT | Cleared time |
| TroubleFound | 227 enumerated code value | T1.227 Trouble Found mapped to Disposition Code |
| TroubleReportState | 4 | Closed |
| TroubleReportStatus | 27 | ClosedOut |
| TroubleReportStatusTime | GMT | Closed (CLO) Time |
| AdditionalTroubleStatusInfo | Text | Closed Message “Report is closed in system and no longer electronically bonded” |

#### UNE/ACCESS/CARRIER Closeout Process

UNE/ACCESS/CARRIER (WFA) trouble report closure is a three step process. The first event is a CenturyLink-sent Cleared Awaiting Customer Verification Event. The second event is a CLEC/IXC CloseOut Event Response. The third event is a CenturyLink-sent Closed Event. Closed information is contained in the CloseOut Request event while the second CenturyLink event is a normal Event Notification with closed status.

##### UNE/ACCESS/CARRIER Cleared/Cleared Awaiting Customer Verification Event

A Cleared Event Notification from WFA for UNE/ACCESS/CARRIERs is an OSS request to close a Trouble Ticket. The same Attributes are returned as in the POTS ClosedOut Event, but the State and Status are returned with Cleared and ClearedAwaitingCustVerification values (3/26). The ActivityDuration Attribute contains all of the NoAccess periods (total) and the data structures for Dispatch and DelayedMaintenance (which will be zero). The CloseOutNarrative is a text message from the OSS (WFA) technician and does not correlate to any mapped definition. The MaintenanceServiceCharge relates mainly to whether or not Dispatch charges will apply. The OutageDuration is the total elapsed time minus the ActivityDuration total. The TroubleFound code does not correlate to any mapped definition.

Details of the cleared transaction Attributes (containing closed information) are:

| Attribute | Value | Description |
| --- | --- | --- |
| ActivityDuration | ActivityType DD:HH:MM | For No Access, accumulated data. For Delayed Maintenance and Dispatch, zeroes. |
| CloseOutNarr | Text | Technician-entered close out summary. |
| MaintServiceCharge | Y/N | Primarily based on dispatch. |
| OutageDuration | DD:HH:MM | Elapsed Time less activityDuration |
| RestoredTime | GMT | Technician-entered cleared time (Technician may back date). |
| TroubleFound | 227 enumerated code value | T1.227 Trouble Found |
| TroubleReportState | 3 | Cleared |
| TroubleReportStatus | 26 | ClearedAwaitingCustomerVerification |
| TroubleReportStatusTime | GMT | ClearedTime |
| AdditionalTroubleStatusInfo | Text | Cleared Message “Cleared Awaiting Customer Verification” |

##### CLEC/IXC Responses to the UNE/ACCESS/CARRIER Cleared Event

The CloseOut Request from WFA requires a CLEC/IXC Response. Once the CloseOut Event is confirmed by the CLEC/IXC’s Gateway, a 24 hour timer is started in the CenturyLink Gateway. If no response is received for the CloseOut Request, then the Trouble Ticket will be closed automatically. A normal CloseOut Response (Set/CloseOut) will Verify, Dispute the Closeout data, or Deny that the trouble has been fixed:

- A Verify response will trigger the CenturyLink process to automatically close the Trouble Ticket.

- A Dispute response sends an RMA (Request for Manual Assistance message) to the Work Center with the reason “Closeout Narrative or ActivityDuration” and the Trouble Ticket will close in 90 minutes.

- A Deny response will expire the 24 hour timer and put the Trouble Ticket back into open/active status and will generate an RMA.

##### UNE/ACCESS/CARRIER Closed Event

A Closed Event Notification will be sent when the Trouble Ticket is Closed in WFA, whether it is due to a Set/Closeout or is Closed manually by the WFA technician. This is a normal Event Notification returning TR State, Status, StatusTime, StatusInfo (4, 27, ClosedTime GMT, Closed message). This is also the Event in the CenturyLink Gateway that triggers the deletion of the Trouble Ticket object from the CenturyLink Gateway.

### Cancel Trouble Report

POTS Resale: A request to cancel a trouble report originates from the CLEC/IXC. The CLEC/IXC issues a PT-SET and CenturyLink is required to respond. This response is an echo back to the CLEC/IXC of the information the CLEC/IXC sent. CenturyLink will screen the trouble report to determine where the trouble report is in the trouble resolution process. CenturyLink will either send the trouble report to a closeout process if no work is in progress on the trouble report, or CenturyLink will complete the work in progress on the trouble report in its current trouble state until it can be closed appropriately. In the latter case, billing charges may apply.

UNE/ACCESS/CARRIER: Same as POTS Resale.

### Request MLT Test (Service Test Function)

POTS Resale: A request for an MLT test originates from the CLEC/IXC. The CLEC/IXC issues an M-ACTION. Once the test request is received, CenturyLink is required to respond through the CenturyLink Gateway. If CenturyLink receives the test request and an error condition exists, then CenturyLink will respond with the appropriate CMIP defined error message. CenturyLink is required to validate circuit ownership before transmitting the test request. CenturyLink has implemented all four tests.

UNE/ACCESS/CARRIER: Not available.

## Attribute Implementation Definition

The complete listing of all Attributes that have been identified as part of initial deployment, and the method for handling the Attributes by CenturyLink are identified in Appendix A, Attribute Definitions. Exceptions are noted in Appendix B, Managed Object Conformance Statements (MOCS).

Although all the Attributes listed in Appendix B have been implemented, in some instances there is limited behavior or no behavior in the CenturyLink OSS due to OSS limitations.

## Initial Implementation Exceptions

This section defines Attribute and Process exceptions which will not be supported via Electronic Bonding. Exceptions are due to limitations in the current OSS and drive no behavior.

### Attributes

POTS Resale: The following Attributes are not supported or are partially supported upon initial creation of a trouble report via Electronic Bonding:

- activityDuration

- additionalTroubleInfoList

- aLocationAccessHours

- aLocationAccessPerson

- authorizationList

- committmentTimeRequest

- custTroubleTicketNumber

- escalationList

- managedObjectAccessHours

- troubleReportStatusWindow

The only Attributes fully supported or partially supported upon modification of a trouble report via Electronic Bonding are:

- additionalTroubleInfoList	(partially supported)

- perceivedTroubleSeverity

See Appendix A, Attribute Definitions and Appendix B, Managed Object Conformance Statements for full details.

UNE/ACCESS/CARRIER: The following Attributes are not supported or are partially supported upon initial creation of a trouble report via Electronic Bonding:

- calledNumber

### Manual Processes

The processes listed below are manual processes because the Attributes relating to these processes are not supported via Electronic Bonding:

#### Authorization

POTS Resale: POTS troubles do not support authorization requests electronically. The Attribute authorizationList will not be implemented at initial deployment with the assumption that authorization to dispatch is granted. Access hour restrictions will require manual processing. See Section 2.3.2.3 Scheduled Dispatch for further details.

UNE/ACCESS/CARRIER: Fully supported.

#### Escalation

POTS Resale: POTS troubles do not support escalation requests electronically. Escalation requests must be made manually and are handled manually through the life of the trouble report.

UNE/ACCESS/CARRIER: Fully supported.

#### Scheduled Dispatch

POTS Resale: Upon initial deployment, all dispatches requiring coordination will be handled manually.

UNE/ACCESS/CARRIER: N/A

#### Modifications to Trouble Reports

POTS Resale: Upon initial deployment, modifications to Attributes other than perceivedTroubleSeverity and additionalTroubleInfoList will be handled manually.

UNE/ACCESS/CARRIER: Fully supported. See Appendix B, Managed Object Conformance Statements for MOCS table details.

## Critical Attributes for Flow-Through

The following table lists the critical Attributes for the business processes. It will be used to determine if the trouble report received achieves current flow-through:

| Attribute | Why Critical | Valid Data |
| --- | --- | --- |
| additional Trouble Info List | CenturyLink requests a description of the problem and any additional information further defining the trouble to aid trouble resolution (current process). CenturyLink needs information explaining environmental or physical conditions, or special circumstances that may affect resolution of the trouble. POTS Resale: In the case of a life threatening situation in Idaho, CLEC/IXC must send information describing the situation. In the case of a hunt group trouble, CLEC/IXC must describe the problem. UNE/ACCESS/CARRIER: Fully supported. | Valid text giving a good description of the problem (in addition to the trouble type) and any important additional information that CLEC/IXC has gathered during their trouble isolation with the end-user of the service. For example, dangerous animal in yard, see neighbor next door, or hearing impaired customer. POTS Resale: For a life threatening situation in Idaho, describe the situation, for example, line down in street, open pit in yard, or medical emergency. For series or multi-line hunting, add text describing the type of trouble and any important additional information. Only the first 47 characters are guaranteed to be displayed in LMOS. UNE/ACCESS/CARRIER: 255 characters per transaction. |
| a Location Access Address | POTS Resale and UNE/ACCESS/CARRIER: CenturyLink verifies the premises address (current process) to ensure correct dispatches. If CLEC/IXC does not send this information, incorrect dispatches and charges may result. | POTS Resale and UNE/ACCESS/CARRIER: Valid end-user premises street address. |
| a Location Access Hours | POTS Resale: The Sunday element of the ALocationAccessHours weekmask can be used to specify a four hour appointment for a trouble report requiring dispatch. All other day settings will be ignored. UNE/ACCESS/CARRIER: Fully supported. NoAccess rules will apply. Note: For UBL 2-Wire analog circuits, the Sunday element of the ALocationAccessHours weekmask can be used to specify a four hour appointment for a trouble report requiring dispatch. | POTS Resale: Standard week mask syntax. UNE/ACCESS/CARRIER: Standard week mask syntax. |
| a Location Access Person | POTS Resale: Currently, CenturyLink is not able to process this information in LMOS because the CLEC/IXC Contact Person is used instead. However, CenturyLink expects CLEC/IXC to send valid data in case this can be used in the future. UNE/ACCESS/CARRIER: Fully supported. | POTS Resale: Valid individual or center name and valid phone number. UNE/ACCESS/CARRIER: Valid individual or center name and valid phone number. |
| called Number | POTS Resale: CenturyLink verifies the number being called if this is relevant to the trouble being reported. If CLEC/IXC does not send valid data, CenturyLink cannot respond appropriately and it may impact trouble resolution. UNE/ACCESS/CARRIER: N/A | POTS Resale: Valid phone number. UNE/ACCESS/CARRIER: N/A |
| Managed Object Instance | POTS Resale: CenturyLink expects the line in trouble to be identified correctly to aid trouble resolution (current process). UNE/ACCESS/CARRIER: CenturyLink expects the circuit in trouble to be identified correctly to aid trouble resolution (current process). | POTS Resale: TN, for example, 3031234567 or TN plus TER, for example 3031234567TER12345 UNE/ACCESS/CARRIER: See Appendix G, Managed Object Instance Formats for valid unbundled loop and unbundled switch circuit formats. |
| ManagerContactPerson | POTS Resale and UNE/ACCESS/CARRIER: CenturyLink needs contact name and phone number to prevent further service delays when a telephone call must be made to CLEC/IXC. If CLEC/IXC does not send valid data, CenturyLink cannot respond appropriately and it may impact outage duration. | POTS Resale and UNE/ACCESS/CARRIER: Valid individual or center name and valid phone number. Not VRU info. |
| Perceived Trouble Severity | POTS Resale: CenturyLink requests information about the trouble severity to select commitment time (current process). If CLEC/IXC does not send valid data, CenturyLink defaults the commitment time based on trouble Type. UNE/ACCESS/CARRIER: See Appendix B, Managed Object Conformance Statements for MOCS table details. | POTS Resale: Standard values UNE/ACCESS/CARRIER: Standard values |
| Preferred Priority | POTS Resale: CenturyLink receives “life threatening” value for Idaho lines and denotes this in the LMOS Narrative to identify to the flow-through process that immediate attention should be given to this trouble ticket. If CLEC/IXC does not send this value, CenturyLink cannot respond appropriately. UNE/ACCESS/CARRIER: See Appendix B, Managed Object Conformance Statements for MOCS table details. | POTS Resale: Standard values UNE/ACCESS/CARRIER: Standard values |
| repeat Report | POTS Resale: CenturyLink verifies recent service order activity (current process). CenturyLink receives “recent installation” or “repeat and recent installation” value and denotes this in the LMOS Narrative for the flow-through process. If there has been recent activity and CLEC/IXC does not send one of these values, CenturyLink cannot ensure correct dispatches. In the case of a chronic report, CLEC/IXC sends “chronic” value. CenturyLink denotes this in the LMOS Narrative for the flow-through process. UNE/ACCESS/CARRIER: N/A | POTS Resale: For recent service order activity, “recent installation” (1) or “both installation and repeat” (3) value. For chronic reports, “chronic” (4) value. UNE/ACCESS/CARRIER: N/A |
| Trouble Clearance Person | POTS Resale and UNE/ACCESS/CARRIER: CenturyLink needs clearance name and phone number to identify who within CLEC/IXC has requested cancellation of a trouble ticket (current process). | POTS Resale and UNE/ACCESS/CARRIER: Valid individual or center name and valid phone number. |
| Trouble Type | POTS Resale and UNE/ACCESS/CARRIER: CenturyLink uses trouble Type to derive values necessary to determine line in use, routing, and flow-through. If CLEC/IXC sends a trouble Type that is inappropriate for the type of service, the create transaction will fail. | POTS Resale and UNE/ACCESS/CARRIER: Standard values per the non-applicable (invalid) trouble type lists. See Appendix F, Data Definitions and Mappings for details. POTS Resale: For series or multi-line hunting when exact TN or TER in trouble is known, use the specific trouble type for the situation, if it can be identified. If the type of trouble cannot be identified, use 1118 for Hunting Not Working. For series or multi-line hunting when exact TN or TER in trouble is not known, always use 1118 for Hunting Not Working. |
| tsp Priority | POTS Resale: CenturyLink uses the tsp maintenance value in the LMOS Line Record to identify to the flow-through process that immediate attention should be given to this trouble ticket. CLEC/IXC must provide the tsp maintenance value, if applicable, so CenturyLink can verify the Line Record and ensure proper routing and immediate trouble resolution. UNE/ACCESS/CARRIER: Not critical. Mismatches will be logged in WFA/C. | POTS Resale and UNE/ACCESS/CARRIER: Valid tsp maintenance value. |

## Error Conditions

POTS Resale and UNE/ACCESS/CARRIER: The following ANSI standard T1.227 error messages will be part of the initial deployment:

- 7.3.1 Trouble Report Already Exists - If a trouble report is already open in the CenturyLink OSS, the error Trouble Report Already Exists will be returned.

- 7.3.2 Fall Back Reporting - A trouble report will not be created and an error message will be returned indicating the fall back error, using the ECIC approved list*. See Appendix C, Fallback Error Conditions for further details.

- 7.3.4 Trouble Report Must Be Present Attributes Missing - This error message will be returned to the CLEC/IXC by CenturyLink when the CLEC/IXC fails to provide all required Attributes identified as mandatory.

- 7.3.6 Trouble Report Change Denied - This error message is sent to the CLEC/IXC when the CLEC/IXC attempts to change a trouble report which is not in an appropriate state to accept the change, for example, when the trouble report is in the cleared state.

_____
*Changes to fall back reporting must be agreed to by all CLEC/IXCs before implementation. CenturyLink will determine the changeover date for fall back error updates.

### UNE/ACCESS/CARRIER only:

- 7.3.5 Cannot Verify or Deny At This Time – This error will be returned if a SET CloseOut is submitted by the CLEC/IXC and the trouble report is not in a Cleared Awaiting Customer Verification status.

Some error conditions that have been identified as part of initial deployment for POTS Resale or for UNE/ACCESS/CARRIERs are identified in Appendix C, Fallback Error Conditions. All other error conditions that are defined in the standards can be accepted.

The following CMIP Transaction Errors will be part of the initial deployment for POTS Resale and for UNE/ACCESS/CARRIERs:

- No Such Object Instance - The value of the superior object instance is not recognized (-104).

- Resource Limitation - Processing failure (-103).

- Access Denied - This is an association error due to invalid Account or Network (-101).

- Invalid Parameter/Invalid Value - Invalid or incomplete data required for transaction (-102). This error will be returned for invalid trouble types.

- Missing Attribute - This error is returned due to dependencies (-106).

The following CMIP Transaction Errors will be part of the initial deployment for POTS Resale Service Test Function:

- MistypedTestCategoryInformation – An invalid test type was specified in the request (260).

- NoSuchMort – The phone number sent in on the request can’t be found in the service provider’s database (263).

- Access Denied - This is an association error due to invalid Account or Network (-101).

- MORTNotAvailable – The MLT test system is not available (261).

# Communication Protocol

## Protocol Profile

CenturyLink utilizes the OSI reference model consisting of the TCP/IP transmission protocol for layers one through three, and for layers three through seven the CenturyLink application (SMASE) is accessed via the Retix CMIP/OSI stack (version 0.3).

## Access Topology

Local Access Topology - Initial Implementation

CLEC/IXC Customer Gateway

CLEC/IXC CUSTOMER TCP/IP POI - CenturyLink TCP/IP POI

Point of

Interface

External

IP

Address

CenturyLink

ECOM

Firewall

MEDIACC

Figure: Connections Between CLEC/IXC CUSTOMER Local and CenturyLink

Logical connectivity is offered as illustrated in Figure 1 Connections between CLEC/IXC CUSTOMER Local and CenturyLink. Descriptions of the components and configurations are referenced throughout the remaining sections of this document.

## CMIP/OSI Implementation Definition

### Access

The CenturyLink gateway and associated services can only be accessed through a TCP/IP address at CenturyLink ECOM. Such access is established via CenturyLink's ECOM firewall.

CLEC/IXC CUSTOMER connects to CenturyLink via the assigned external IP address. Connectivity is achieved through a dedicated line terminating at the CLEC/IXC CUSTOMER POI in the CenturyLink Data Center in Omaha, NE.

In the case of recovery from a failed status, see Electronic Recovery in Section 6, Recovery Procedures.

### Security

Detailed specifications for making the necessary connections with TCP/IP are provided during the requirements review process.

### Connectivity

CLEC/IXC CUSTOMER is responsible for all charges incurred in the transmission of communications packets to the Point of Interface (POI) with the CenturyLink TCP/IP Point of Interface (POI). CenturyLink is responsible for all charges incurred in the transmission of data from that location to the CenturyLink GATEWAY system. The POI is the specific location where connectivity is obtained to TCP/IP network.

## Transport Layer

The TP4 transmission class is the only support level that will be supported for X.25 and does not apply to TCP/IP.

## Manager/Agent Entity

Each CLEC/IXC or CenturyLink that requires a unique destination due to business function, load, version control, or security, shall identify itself by a unique Entity Id. Each Entity Id shall have a unique X.25 network address (NSAP) or IP Address or, if on the same host, shall have unique SAPs (TSAP, SSAP, PSAP).

### Manager Entity

Each Manager Entity requires a unique OSI destination to assure the correct routing of all Requests and Event Notifications. If multiple managers are running on the same Server and use the same Network address (NSAP) or IP address, then the upper layer Selectors/SAPs shall be sufficiently unique to route to the correct CLEC/IXC manager. If each CLEC/IXC manager runs on its own Server, then the Network address shall be unique. All Account Ids under each manager shall contain only alpha-numeric characters, e.g. no special characters such as dashes (“-”).

### Agent Entity

Each Agent Entity requires a unique OSI destination to assure the correct routing of all Requests and Event Notifications. If multiple agents are running on the same Server and use the same Network address (NSAP), then the upper layer Selectors/SAPs shall be sufficiently unique to route to the correct CenturyLink agent.  If each agent runs on its own Server, then the Network address shall be unique.

# Security

## CenturyLink/CLEC/IXC Customer Responsibilities

Each company is responsible for ensuring that only authorized users have access to the host systems and gateways to the other company.

## Association Mechanism

CLEC/IXC shall use an independent Manager Entity for CLEC/IXC Customer. The Manager Entity shall have OSI Selectors/SAPs sufficiently unique to allow for independent Associations for each CLEC/IXC and independent CMIP Request and Event Notification routing. Each Manager Entity will use a set of 1000 Encrypted Keys which shall be provided by CenturyLink.

Each CLEC/IXC Manager Entity shall have the ability to Abort or Release association without disrupting the business processes of the other CLEC/IXC. CenturyLink shall allow each CLEC/IXC Manager Entity to conduct business processes independently of any other CLEC/IXC manager.

Access control encryption will be performed using Triple DES CBC.

All association requests will require the current valid key and be within a 2 minute window.

## Login/Key Assignment Process

CenturyLink will be the generator of the keys for Trouble Administration. The keys must be odd parity, 8 character ASCII keys.

The peer-to-peer entity authentication process requires the use of an encryption key. The encryption keys shall be unique for each Manager Entity Id.

There will be two sets of keys provided by CenturyLink, with 1000 keys in a set. Both sets of keys that are generated shall be numbered, and thus identified, from 1 to 1000. Keys shall be referenced by their key identifier (sequence number). Subsequent key sets shall be numbered consecutively in increasing order as a continuation of numbering of the previous list. Every key shall have a unique keyIdentifier.

A backup list of 1000 keys shall be kept on hand by both CLEC/IXC and CenturyLink for use when the primary list has been exhausted or compromised. After 70 percent (700 keys) of a set has been used, or after one year has transpired, whichever comes first, CenturyLink shall generate and provide CLEC/IXC with a new list of 1000 keys.

It is the responsibility of the CLEC/IXC to request the change of the current key on a regular basis. The ECIC Security Requirement suggests that a key will not be used for a period exceeding four days. CenturyLink standards state that a given key shall not be used for a period to exceed 30 days.

The accessControl notation, as specified by ECIC, supports electronic key changes. The change request is made by the CLEC/IXC to expire the current key and increment by one to the next key. When the current key is changed, the old key shall be considered expired.

# Production Availability

## CenturyLink MEDIACC/EBTA Gateway:

Hours of operation is listed on the Wholesale Systems Web site under Change Management Process (CMP) OSS Hours of Availability http://www.centurylink.com/wholesale/cmp/ossHours.html.

### LMOS:

Central Region - AZ, CO, ID, MT, NM, UT, WY (Mountain Time)

00:15 to 01:15 (daily maintenance window)

Eastern Region - IA, NE, MN, ND, SD (Mountain Time)

23:15 to 00:15 (daily maintenance window)

Western Region - OR, WA (Mountain Time)

01:15 to 01:15 (daily maintenance window)

Note: If requests are received for a region during the scheduled downtime then an error (-103) response will be sent to the CLEC/IXC. You should wait until the downtime has expired before sending another transaction.

### WFA/C:

Central Region - AZ, CO, ID, MT, NM, UT, WY (Mountain Time)

03:00 to 04:00 Monday through Friday

22:00 Saturday through 06:00 Sunday

Eastern Region - IA, NE, MN, ND, SD (Mountain Time)

02:00 to 03:00 Monday through Friday

21:00 Saturday through 05:00 Sunday

Western Region - OR, WA (Mountain Time)

04:00 to 05:00 Monday through Friday

23:00 Saturday through 07:00 Sunday

# Recovery Procedures

## Off-Nominal Status

### CenturyLink Process

Off-Nominal Status is an error state in which the software/hardware is capable of recovery on its own. Transaction errors may occur, but no intervention is required. The CenturyLink EB Single Point of Contact/Service Center will not contact the CLEC/IXC.

### CLEC/IXC Process

CLEC/IXC will treat this error state on an instance at a time basis. If an activity returns an error response or no response, the CLEC/IXC will attempt to re-send the transaction. If the subsequent transaction also returns an error response, the CLEC/IXC will handle the activity manually. If an error response indicates a correctable activity, the CLEC/IXC will provide the correction and re-send. If a correction to the error is uncertain, the CLEC/IXC will contact the CenturyLink EB Single Point of Contact/Service Center for clarification of the error response.

## Electronic Recovery

### CenturyLink Process

It is to the advantage of both the CLEC/IXCs and CenturyLink to maintain the electronic data flow for the life of an EB ticket. Minor errors (Off-Nominal Status) should be overcome, and data flow should continue as before without the need for any manual intervention or manual communication between the CLEC/IXCs and CenturyLink. Each EB ticket shall remain electronically bonded.

If disassociation occurs, all Work Center RMA (Request for Manual Assistance) screens receive a “Link Down” message for every CLEC/IXC manager that disassociates. A CenturyLink Work Center technician may use manual Methods and Procedures until the association is reestablished.

During Failed Status or if the link to an OSS is lost, the OSS queues the event notifications until the CenturyLink EB Gateway reestablishes the OSS link and processes the OSS messages (this will not be done until association with all managers is reestablished). If the CLEC/IXC does not request re-association within 30 minutes of recovery, the CLEC/IXC will be contacted. If CLEC/IXC’s manager aborts, event notifications may be lost.

Recovery from a Failed Status may result in the loss of a circuit or an EB server. Recovery from this may require changing the IP address and possibly require a new NSAP.

Failed Status could result in the permanent loss of data flow on existing EB tickets. Once association is reestablished, communication between CLEC/IXCs and CenturyLink may be needed to assess the need for manual intervention to fully recover to a Nominal Status.

### Manual to Electronic Recovery

POTS Resale: If a trouble report is created manually, it remains manual for the life of the trouble report until a mechanism to convert the report to an electronic report is available.

UNE/ACCESS/CARRIER: If trouble is detected on CenturyLink equipment, or if a CLEC/IXC calls in a trouble report (usually due to a Gateway disruption), a Work Center Technician may manually enter a trouble ticket into the appropriate OSS region.

If a CLEC/IXC submits a request to open a trouble ticket on CenturyLink equipment that has had a trouble ticket opened manually, the CenturyLink Gateway (MEDIACC) will convert the trouble ticket to Electronic Bonded status (manual to electronic conversion).

The response returned on the Create request is a TR_Already_Exists error, but the trouble ticket is built up in the CenturyLink Gateway and it is expected that the normal Work Center EB processes will be followed from that point on (authorization/closeout requests and responses, text messages, Status Changes, Set Modify, etc.). The CLEC/IXC can submit a GET Request to obtain the most current data.

### CLEC/IXC Process

When a link is re-established, CLEC/IXC should utilize a supplemental process. CLEC/IXC should send a GET status transaction on all bonded trouble reports. Any no such object error response from CenturyLink would be treated as a closed trouble report and close out details would have to be pursued manually. When a failed link is established, all UNE/ACCESS/CARRIER trouble reports created manually will be resubmitted to CenturyLink electronically for late bonding. If response is a trouble report already exists, CLEC/IXC will submit a GET request to obtain the most current data.

## EBTA Single Point of Contact

In order to manage technical contact between CenturyLink and CLEC/IXC, an EB single point of contact is needed.

CLEC/IXC customers should call the Wholesale Systems Help Desk (WSHD) 888-796-9102. The WSHD will assign out a ticket and assign it to the appropriate SME team.

# Other

## T1.227 Implementation Levels

### Name Binding

CenturyLink currently has Name Binding in production at the T1.227 - 1992 Standard.

### Repeat Report

CenturyLink will support the 1995 T1M1 Standard for the extensions to the Repeat Report Attribute.

# Appendix A - Attribute Definitions

This section includes the Definition for POTS and UNE/ACCESS/CARRIER for Initial Deployment for each Attribute, which will be used by CenturyLink for its development and initial deployment. If a field is marked as mandatory in the ANSI standards documentation then the field should be sent to CenturyLink regardless if CenturyLink backend systems will utilize the data sent.

## A Location Access Address

Definition for POTS for Initial Deployment: The street address received from the CLEC/IXC is compared against the CenturyLink street address contained in the host system. Additionally, CenturyLink will implement a systematic process to prohibit trouble report creation by CLEC/IXC if a CLEC/IXC sends substantial amounts of incorrect information causing incorrect dispatches until the condition is corrected.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## A Location Access Hours

Definition for POTS for Initial Deployment: CenturyLink requires a week mask with current day set to a valid value and hours defaulted to between 0:00 and 23:59. CenturyLink will disregard any A Location Access Hours provided by the CLEC/IXC and assume the default values of current day and 0:00-23:59 for hours. The current OSS generated commitment time based on the current service condition (Affecting Service or Out of Service) is returned. Arranging premises access due to access restrictions is a manual process.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink OSS limitation of three days, one access interval per day. Time is specified in the customer’s (end user’s) local time based on the customer’s address.

## A Location Access Person

Definition for POTS for Initial Deployment: Not supported due to business process agreements and OSS limitations. The CLEC/IXC’s contact information will be used in the CenturyLink OSS as primary customer contact per agreement.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink supports Name (20 characters) and telephone number (10 digits).

## Activity Duration

Definition for POTS for Initial Deployment: Not supported. This Attribute is not applicable for POTS.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Fully supported. Summation of No Access intervals. This value is returned when the trouble report is cleared.

## Additional Trouble Information List

Definition for POTS for Initial Deployment: There is an OSS field size limitation of 102 characters for this Attribute. CenturyLink reserves the first 55 characters to populate information derived from Attributes sent by the CLEC/IXC in the host on a Create. CenturyLink stores 47 characters from the CLEC/IXC in the host on a Create. The CenturyLink and CLEC/IXC information is separated by a delimiter. CenturyLink uses the aLocationAccessAddress, managedObjectInstance, preferredPriority, repeatReport, troubleType, and tspPriority Attributes to derive the information that is stored in its 55 characters. The information sent by the CLEC/IXC in this Attribute should not repeat information derived by CenturyLink from these CLEC/IXC supplied Attributes but further describe the trouble using CenturyLink’s standard abbreviations, where applicable. (See Appendix F, Data Definitions and Mappings for details.) If the CLEC/IXC sends the Attribute preferredPriority with a value of “life threatening,” then the CLEC/IXC also is required to send further information about the situation in Additional Trouble Information List, for Idaho lines. If an add or modify is performed, the new CLEC/IXC information will overlay the old CLEC/IXC information.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink supports 255 characters per transaction. Anything greater than 255 characters is truncated without causing the transaction to fail.

A joint Dispatch Out (DPO) field meet can be requested by populating the Additional Trouble Information List attribute with “DPO Joint Meet” and include date and time (e.g., DPO Joint Meet 07/21/04 3 PM). You may also request a joint Dispatch In (DPI) central office meet by populating the Additional Trouble Information List with “DPI Joint Meet” and include date and time (e.g., DPI Joint Meet 07/21/04 3 PM). To request a joint meet, you must submit your request no later than 3 PM local time the day before the joint meet.

A trouble ticket for IP CENTREX can be submitted by populating the Additional Trouble Information List attribute with “IPCTX” and the telephone number of the line(s) experiencing difficulty.

## Additional Trouble Status Information

Definition for POTS for Initial Deployment: For all status changes, a text message describing the status change is returned in this Attribute. If the status change is due to the initial MLT test, the VER code and VER code definition (a number and the associated text description) are returned. If CenturyLink sends a text message (remarks) to the CLEC/IXC, then the text message is returned in this Attribute. (See Appendix F, Data Definitions and Mappings for details.)

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink supports up to 250 characters.

## Agent Contact Person

Definition for POTS for Initial Deployment: The responsible CenturyLink Work Center name and phone number are returned when the trouble report is created.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as for POTS Resale.

## Authorization List

Definition for POTS for Initial Deployment: Not supported. This Attribute is not applicable for POTS.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Fully supported for After Hours Repair, Test, Dispatch, No Access, and Delayed Maintenance for request, approve, and deny. CenturyLink maps the CLEC/IXC’s Release Authorization to Test Authorization.

## Called Number

Definition for POTS for Initial Deployment: This Attribute is stored in a specific host field.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Not supported.

## Cancel Requested By CLEC/IXC

Definition for POTS for Initial Deployment: A specific host message is logged in the host system, so the technician can close/cancel the trouble report as appropriate.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Close Out Narrative

Definition for POTS for Initial Deployment: The disposition code (a number) and its associated text description, and the cause code (a number) and its associated text description entered when the trouble report is closed are returned.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: The text description entered by a technician when the trouble report is cleared is returned. There is no mapping of this text.

## Commitment Time

Definition for POTS for Initial Deployment: The OSS generated commitment time based on the current service condition is returned when the trouble report is created. The commitment time attribute is provided with each standard status event notification.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: The OSS generated Objective Time.

## Commitment Time Request

Definition for POTS for Initial Deployment: Not supported.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## Customer Trouble Ticket Number

Definition for POTS for Initial Deployment: Not supported.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## Escalation List

Definition for POTS for Initial Deployment: Not supported.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Fully supported.

## Maintenance Service Charge

Definition for POTS for Initial Deployment: The disposition code entered when the trouble report is closed is used to set the Maintenance Service Charge, which is then returned. (See Appendix F, Data Definitions and Mappings for details.)

Definition for UNE/ACCESS/CARRIER for Initial Deployment: The value is set to either Yes or No by the technician when the trouble report is cleared. There is no mapping of this value.

## Managed Object Access Hours

Definition for POTS for Initial Deployment: Not supported. This Attribute is not applicable for POTS. CenturyLink believes this is not an issue because currently a technician monitors a line to see if the line is in use. Work is not performed until the line is not in use.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink OSS limitation of three days, one access interval per day.

## Managed Object Instance

Definition for POTS for Initial Deployment: This is the CenturyLink circuit ID, facility ID or telephone number. This Attribute is the key element used by the host system when resolving the trouble report.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: This is the Network ID:Account ID:Circuit ID or Trouble Report ID.

## Manager Contact Person

Definition for POTS for Initial Deployment: The CLEC/IXC contact name and phone number are stored in specific host fields as primary customer contact.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: CenturyLink supports Name (20 characters) and telephone number (18 characters).

## Managed Object Referring to Test

Definition for POTS Resale Initial Deployment: MORT is the CenturyLink telephone number. This Attribute identifies the managed object instance(s) which identifies the resource being or to be tested. This parameter is mandatory in all notifications concerning the test invocation when the parameter is also used in the test initiation request.

## Outage Duration

Definition for POTS for Initial Deployment: Since Activity Duration (no access or delayed maintenance) is not applicable for POTS, Outage Duration is calculated as the amount of time between when the trouble report is cleared (Restored Time) and the trouble report Received Time. This value is returned when the trouble report is closed.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Restored Time minus Received Time minus Activity Duration. This value is returned when the trouble report is cleared.

## Perceived Trouble Severity

Definition for POTS for Initial Deployment: This information is stored in the host system. If a modify is performed, the severity can only be raised, not lowered. If it was originally set to Affecting Service, then it can be modified to Out of Service. If it was originally set to Out of Service, then it cannot be modified.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Out of Service is set to Yes or No in the OSS.

## Preferred Priority

Definition for POTS for Initial Deployment: The CLEC/IXC is required to send Preferred Priority to indicate a “life threatening” situation for Idaho lines. This information is stored in the host system.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## Received Time

Definition for POTS for Initial Deployment: The CenturyLink host system generates the value of this Attribute, which is returned when the trouble report is created.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Repeat Report

Definition for POTS for Initial Deployment: Repeat Report is used to derive if there is recent service order activity, defined to be activity within the last seven days, based on the values of Recent Installation (1) or Repeat and Recent Installation (3). If there is recent service order activity, this information is stored in the host. Repeat Report is also used to derive if there is a chronic trouble report situation based on the Chronic value (4) and this information is stored in the host. Repeat Report is not used to indicate a repeat report because this information resides in the host system.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## Restored Time

Definition for POTS for Initial Deployment: The value of this Attribute is entered in the host system when the trouble report is cleared.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Trouble Clearance Person

Definition for POTS for Initial Deployment: The person name and phone number are stored in the trouble report in the host during cancellation of a trouble report.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Trouble Found

Definition for POTS for Initial Deployment: The disposition code entered when the trouble report is closed is mapped to a T1.227 standard Trouble Found code. Trouble Found is returned when the trouble report is closed. (See Appendix F, Data Definitions and Mappings for details.)

Definition for UNE/ACCESS/CARRIER for Initial Deployment: The WFA/C trouble found code entered when the trouble report is cleared is mapped to a T1.227 standard Trouble Found code. (See Appendix F, Data Definitions and Mappings for details.)

## Trouble Report Format Object Pointer

Definition for POTS for Initial Deployment: This field refers to the Trouble Report Format that applies to the definition for the CREATE transaction. TRFD 1 is the format in use.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: TRFD 1 and TRFD 2 are the formats in use.

## Trouble Report Identifier

Definition for POTS for Initial Deployment: The CenturyLink Trouble Report Identifier that is generated by the host system is returned when the trouble report is created.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Trouble Report State

Definition for POTS for Initial Deployment: Values of Open/Active, Deferred, Cleared, and Closed are supported by the host system.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Trouble Report Status

Definition for POTS for Initial Deployment: Host statuses will be mapped to the T1.227 statuses. (See Appendix F, Data Definitions and Mappings for details.)

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale. (See Appendix F, Data Definitions and Mappings for details.)

## Trouble Report Status Time

Definition for POTS for Initial Deployment: The CenturyLink host system generates the value of this Attribute.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Same as POTS Resale.

## Trouble Report Status Window

Definition for POTS for Initial Deployment: Not supported.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

## Trouble Type

Definition for POTS for Initial Deployment: Host fields have been identified to store this Attribute. CenturyLink has identified specific trouble types that will indicate Line In Use to prohibit the initial test. Additionally, CenturyLink has identified specific trouble types that are not applicable trouble types for Local Resale (POTS). In this case, CenturyLink will not create an electronic trouble report for these trouble types and will return an Invalid Attribute Value error. (See Appendix F, Data Definitions and Mappings for details.)

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS. CenturyLink has identified specific trouble types that are not applicable trouble types for UNE/ACCESS/CARRIER. In this case, CenturyLink will not create an electronic trouble report for these trouble types and will return an Invalid Attribute Value error.

## TSP Priority

Definition for POTS for Initial Deployment: The TSP Priority received from the CLEC/IXC is compared against the CenturyLink TSP Priority contained in the host system. If the two values are different, the TSP value sent by the CLEC/IXC is stored in the host and this trouble report is flagged for manual screening.

Definition for UNE/ACCESS/CARRIER for Initial Deployment: Stored in the OSS.

# Appendix B - Managed Object Conformance Statements (MOCS)

This section includes the MOCS tables which will be used by CenturyLink for its development and initial deployment.

## POTS Resale MOCS Table for Initial Deployment

| Attribute | Create |  | Get |  | Set Add/Mod |  | Set/ Cancel |  | Events |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Man | Agt | Man | Agt | Man | Agt | Man | Agt | Man | Agt |
| ActivityDuration | P | D | O | NR | P | N/A | P | N/A | N/A | C |
| AdditionalTroubleInfoList | M* | NR | O | NR | O* | NR | O | NR | N/A | P |
| AdditionalTroubleStatusInfo | P | M | O | NR | P | N/A | P | N/A | NR | E |
| AgentContactPerson | P | M* | O | NR | P | N/A | P | N/A | N/A | P |
| ALocationAccessAddress | M | NR | O | NR | O | NB | P | N/A | N/A | P |
| ALocationAccessHours | M* | NR | O | NR | O | NB | P | N/A | N/A | P |
| ALocationAccessPerson | M | NB | O | NR | O | NB | P | N/A | N/A | P |
| AuthorizationList | M | NB | O | NR | O | NB | P | N/A | N/A | N/A |
| CalledNumber | O | NR | I | NR | O | NB | P | N/A | N/A | P |
| CancelRequestedByManager | P | D | O | NR | P | N/A | M | NR | N/A | P |
| CloseOutNarr | P | D | O | NR | P | N/A | P | N/A | NR | C |
| CommitmentTime | P | M | O | NR | P | N/A | P | N/A | N/A | N/A |
| CommitmentTimeRequest | O | NB | I | NR | O | NB | P | N/A | N/A | P |
| CustTroubleTikNum | M | NB | O | NR | P | N/A | P | N/A | N/A | P |
| EscalationList | M | NB | O | NR | O | NB | P | N/A | N/A | N/A |
| MaintServiceCharge | P | D | O | NR | P | N/A | P | N/A | NR | C |
| ManagedObjectAccessHours | M* | NR | O | NR | O | NB | P | N/A | N/A | P |
| ManagedObjectInstance | M | M | O | NR | P | P | P | N/A | N/A | P |
| ManagerContactPerson | M | NR | O | NR | O | NB | P | N/A | N/A | P |
| OutageDuration | P | D | O | NR | P | N/A | P | N/A | NR | C |
| PerceivedTroubleSeverity | M* | NR | O | NR | O* | NR | P | N/A | N/A | P |
| PreferredPriority | O* | NR | O | NR | O | NR | P | N/A | N/A | P |
| ReceivedTime | P | M | O | NR | P | N/A | P | N/A | N/A | P |
| RepeatReport | O* | NR | I | NR | O | NB | P | N/A | N/A | P |
| RestoredTime | P | D | O | NR | P | N/A | P | N/A | NR | C |
| TroubleClearancePerson | P | D | O | NR | P | N/A | M | NR | N/A | P |
| TroubleFound | P | D | O | NR | P | N/A | P | N/A | NR | C |
| TroubleReportFormatObjectPtr | M | NR | O | NR | P | P | P | N/A | N/A | P |
| TroubleReportID | P | M | O | NR | P | P | P | N/A | N/A | P |
| TroubleReportState | P | M | O | NR | P | N/A | P | N/A | NR | E |
| TroubleReportStatus | P | M | O | NR | P | N/A | P | N/A | NR | E |
| TroubleReportStatusTime | P | M | O | NR | P | N/A | P | N/A | NR | E |
| TroubleReportStatusWindow | O | NB | I | NR | O | NB | P | N/A | N/A | P |
| TroubleType | M | NR | O | NR | P | P | P | N/A | N/A | P |
| TspPriority | O | NR | O | NR | O | NB | P | N/A | N/A | P |

## Legend for POTS Resale MOCS Table for Initial Deployment

| C | Required on ClosedOut event |
| --- | --- |
| D | Defaulted on Create, but may be null |
| E | Required on all status change events |
| I | Optional, if this Attribute was instantiated on the Create |
| M | Mandatory Attribute for this transaction |
| NB | No behavior, the Attribute will be stored in the Gateway only |
| O | Optional for this transaction |
| P | Prohibited Attribute for this transaction |
| * | Attribute requires new behavior or agreed-to implementation |
| NR | Normal response |
| N/A | Not applicable |

## UNE/ACCESS/CARRIER MOCS Table for Initial Deployment

| Attribute | Create |  | Get |  | Set Add/Mod |  | Set/ Cancel |  | Set/ Closeout |  | Close Out Request | Events |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Man | Agt | Man | Agt | Man | Agt | Man | Agt | Man | Agt | Agt | Agt |
| activityDuration | P | D | O | NR | P | N/A | P | N/A | P | N/A | M | N/A |
| additionalTroubleInfoList | M | NR | O | NR | O | NR | O | NR | O | NR | N/A | N/A |
| additionalTroubleStatusInfo | P | M | O | NR | P | N/A | P | N/A | P | N/A | E | NR |
| AgentContactPerson | P | M | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| aLocationAccessAddress | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| aLocationAccessHours | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| aLocationAccessPerson | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| authorizationList | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | E |
| cancelRequestedByManager | P | D | O | NR | P | NR | M | NR | P | N/A | N/A | N/A |
| closeOutNarr | P | D | O | NR | P | NR | P | N/A | P | N/A | M | NR |
| closeOutVerification | P | D | O | NR | P | N/A | P | N/A | M | NR | N/A | N/A |
| commitmentTime | P | M | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| commitmentTimeRequest | O | NR | I | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| custTroubleTikNum | M | NR | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| escalationList | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | E |
| maintServiceCharge | P | D | O | NR | P | N/A | P | N/A | P | N/A | M | N/A |
| managedObjectAccessHours | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| managedObjectInstance | M | M | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| managerContactPerson | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| outageDuration | P | D | O | NR | P | N/A | P | N/A | P | N/A | M | N/A |
| perceivedTroubleSeverity | M | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| preferredPriority | O | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| receivedTime | P | M | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| repeatReport | O | NR | I | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| restoredTime | P | D | O | NR | P | N/A | P | N/A | P | N/A | M | N/A |
| troubleClearancePerson | P | D | O | NR | P | NR | M | NR | M | NR | N/A | N/A |
| troubleFound | P | D | O | NR | P | N/A | P | N/A | P | N/A | M | N/A |
| troubleReportFormatObjectPtr | M | NR | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| troubleReportID | P | M | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| troubleReportState | P | M | O | NR | P | N/A | P | N/A | P | N/A | E | E |
| troubleReportStatus | P | M | O | NR | P | N/A | P | N/A | P | N/A | E | E |
| troubleReportStatusTime | P | M | O | NR | P | N/A | P | N/A | P | N/A | E | E |
| troubleReportStatusWindow | O | NR | I | NR | O | NR | P | N/A | P | N/A | N/A | N/A |
| troubleType | M | NR | O | NR | P | N/A | P | N/A | P | N/A | N/A | N/A |
| tspPriority | O | NR | O | NR | O | NR | P | N/A | P | N/A | N/A | N/A |

## Legend for UNE/ACCESS/CARRIER MOCS Table for Initial Deployment

| C | Required on ClosedOut event |
| --- | --- |
| D | Defaulted on Create, but may be null |
| E | Required on all status change events, or OSS functions |
| I | Optional, if this Attribute was instantiated on the Create |
| M | Mandatory Attribute for this transaction |
| NB | No behavior, the Attribute will be stored in the Gateway only |
| O | Optional for this transaction |
| P | Prohibited Attribute for this transaction |
| * | Attribute requires new behavior or agreed-to implementation |
| NR | Normal response |
| N/A | Not applicable |

# Appendix C - Fallback Error Conditions

Fall Back Report Errors: This section lists the fall back errors which will be implemented by CenturyLink for its initial deployment for POTS Resale and UNE/ACCESS/CARRIER. There are no Fallback Errors for the Service Test Function.

## POTS Resale and UNE/ACCESS/CARRIER:

| Code | Error | Definition |
| --- | --- | --- |
| 0 | Circuit Mismatch | Circuit not found or invalid circuit ID format |
| 1 | Create in progress | Create Transaction in Progress (Currently in process of creating a ticket) |
| 2 | Circuit Ownership | Invalid MCN or RSID |
| 9 | Circuit Disconnected | Circuit is not in a valid working status |

## UNE/ACCESS/CARRIER:

| Code | Error | Definition |
| --- | --- | --- |
| 6 | New Service Pending | Circuit is designed and/or provisioned, but service is Not In Effect |

# Appendix D - EB TA Process Management

This section lists trouble isolation and trouble resolution processes.

- All trouble isolation processes will be followed by CLEC/IXC before CLEC/IXC sends an electronic trouble report to CenturyLink.

# Appendix E - References to Other Documents

This document is intended to contain a list of all publications and release dates for all documents referenced in this Joint Implementation Agreement.

## References

T1M1.5/96-216, 5/1/96, Operations, Administration, Maintenance and Provisioning (OAM&P) - Basic extensions to CLEC/IXC Network Information Model for Interfaces Between Operations Systems Across Jurisdictional Boundaries to Support a CLEC/IXC Set of Management Services

ANSI T1.227 -1995, American National Standards for Telecommunications - Operations, Administration, Maintenance, and Provisioning (OAM&P) - Extension to CLEC/IXC Network Model for Interfaces Between Operations Systems Across Jurisdictional Boundaries to Support Fault Management - Trouble Administration

ANSI T1.228 -1995, American National Standards for Telecommunications - Operations, Administration, Maintenance, and Provisioning (OAM&P) - Services for Interfaces Between Operations Systems Across Jurisdictional Boundaries to Support Fault Management - Trouble Administration

ANSI T1.262 –1998, American National Standards for Telecommunications - Operations, Administration, Maintenance, and Provisioning (OAM&P) - Services for Interfaces Between Operations Systems Across Jurisdictional Boundaries to Support Fault Management – Service Test Function

Electronic Communications Implementation Committee (ECIC) Trouble Administration (TA)

Testing Standard Application (ANSI T1.262-1998) Implementation Guidelines, ECIC Document Number: TRA/98-03

ECIC Trouble Administration Test Cases for ANSI T1.262-1998 Testing Standard Application, ECIC Document Number TCIF-ECIC-TRA-099-032

CLEC/IXC Electronic Bonding Functional Requirements and Design document Version 5.08.13, dated 12/15/03(“CLEC/IXC Spec”)

ISO/IEC CMIP/OSI Standards:

10164, parts 1-18

10165, parts 1-7

Other supporting ISO/IEC CMIP/OSI Standards

# Appendix F - Data Definitions and Mappings

## Data Definitions and Mappings for both POTS Resale and UNE/ACCESS/CARRIER

### Standard Abbreviations

| ABBREVIATIONS | MEANING |
| --- | --- |
| AAB | ALL ACCESS BUSY |
| ACC | ACCESS |
| ADL | ADDITIONAL |
| ADV | ADVISED |
| ALCLS | ALL CALLS |
| ALIT | ALI (AUTOMATIC LINE TEST) |
| ALL ONES | GETTING ALL ONES |
| ALM | ALARM |
| ALPHS | ALL PHONES |
| ANA TST | ANALOG TEST LINE |
| ANI TMOUT | ANI TIMEOUT |
| APT | APARTMENT |
| ASAP | AS SOON AS POSSIBLE |
| ASW | AERIAL SERVICE WIRE |
| /T | AT TIMES OR INTERMITTANT |
| CLEC/IXCN | CLEC/IXCENTION |
| B4 | BEFORE |
| BAD BAL | BAD BALANCE |
| BAD ERL | BAD ERL |
| BAD FREQ | DOES NOT PASS FREQ RESPONSE |
| BAD LVLS | LEVELS OUT OF LIMITS |
| BDR | BELL DOESN’T RING |
| BDR EXT | BELL DOES NOT RING2 |
| BIPOL ERR | BIPOLAR VIOLATIONS |
| BO | BUSINESS OFFICE |
| BR | BELL RINGS |
| BRAA | BELL RINGS AFTER ANSWERED |
| BRCA | BELL RINGS CAN’T ANSWER |
| BRKN | BROKEN |
| BRWD | BELL RINGS WHILE DIALING |
| BSW | BURIED SERVICE WIRE |
| BUS | BUSINESS |
| BUSY | BUSY |
| BSY | ALWAYS BUSY/CANNOT BE CALLED BUSY |
| BSY TRIPS | RINGS THEN GOES BUSY |
| C/RUN CSU | CANNOT RUN TO CSU |
| C/RUN OSU | CANNOT RUN TO OSU |
| CALL RET | CALL RETURN NOT WORKING |
| CANCEL | CANCEL |
| CANT MEET | CANNOT MEET |
| CBC | CAN’T BE CALLED |
| CBDT | CAN’T BREAK DIAL TONE |
| CBDT | CAN’T BREAK DIAL TONE |
| CBH | CAN’T BE HEARD |
| CBR | CAN’T BE REACHED |
| CBS | CANNOT BE SIGNALLED |
| CC 700 | CANNOT CALL 700 |
| CC 800/888 | CANNOT CALL 800/888 |
| CC 900 | CANNOT CALL 900 |
| CC 911 | CANNOT CALL 911 |
| CC DA | CANNOT CALL DA |
| CC LD | CANNOT CALL LONG DISTANCE |
| CC LLD | CANNOT CALL INTERLATA TOLL |
| CC TRACE | CALL TRACE NOT WORKING |
| CCE | CUSTOMER CHECKED EQUIPMENT |
| CCF | CAN’T CALL FORWARD |
| CCO | CAN’T CALL OUT |
| CD | COMPLETION DATE |
| CF | CALL FORWARDING |
| CFBL | CALL FORWARDING BUSY LINE |
| CFDA | CALL FORWARDING DON’T ANSWER |
| CFWN | CALL FORWARDING WRONG NUMBER |
| CH | CAN’T HEAR |
| CHK | CHECK |
| CHNG | CHANGE |
| CID | CALLER ID |
| CKD | CIRCUIT DEAD/DEAD AT CIRCUIT |
| CKT DOWN | CIRCUIT DOWN |
| CKT FAIL | FAILING CIRCUIT |
| CKT SZD | SEIZURE ON CIRCUIT |
| CL BLK | CALL BLOCKING NOT WORKING |
| CL RET BLK | CALLER RETURN BLOCK NOT WORKING |
| CL XFER | CALL TRANSFER PROBLEM |
| CLB4 | CALL BEFORE |
| CLD | CALLED |
| CLICKING | CLICKING |
| CLIPPING | CLIPPING |
| CLR | CALLER |
| CNTX | CENTREX |
| COMM | COMMITMENT |
| CONV | CONVERSATION |
| CPH | CALLING PARTY HOLD |
| CPIW | CUSTOMER PROVIDED INSIDE WIRE |
| CPU | CALL PICK UP |
| CR | CALL REJECT |
| CRC | CAN’T RELEASE CIRCUIT |
| CRDA | CANNOT RECEIVE DATA |
| CRT | CATHODE RAY TUBE |
| CSDA | CANNOT SEND DATA |
| CSO | CANNOT SIGNAL |
| CUT CABLE | CUT CABLE |
| CUT OFF | CUT OFF |
| CW | CALL WAITING |
| CWD | CALL WAITING DELUXE |
| CXX | NEW OR ADDING INFORMATION |
| 2DY | TODAY |
| DAMAGE | DAMAGE |
| DATA EQUIP | DATA EQUIPMENT |
| DATA SET | DATA SET |
| DD | DUE DATE |
| DEAD PC | CANNOT ACTIVATE PC |
| DEL | DELAY |
| DIAL BLK | REPEAT DIAL BLOCK NOT WORKING |
| DIE | DISCONNECT IN ERROR |
| DIG TST | DIGITAL TEST LINE |
| DISC | DISCONNECT |
| DISP | DISPATCH |
| DISTANT | DISTANT |
| DL TMOUT | PARTIAL DIAL TIMEOUT |
| DMARC | DEMARCATION POINT |
| DROP OUTS | DROPOUTS |
| DT | DIAL TONE |
| DTAD | DIAL TONE AFTER DIALING |
| DTWT | DIAL TONE WHILE TALKING |
| ECO | ECHO |
| EQUIP | EQUIPMENT |
| ERR | ERRORS |
| EST | ESTIMATE |
| ETIR | EVERY TIME IT RAINS |
| EXT | EXTENSION |
| FAC ALM | FACILITY ALARM |
| FADE IN/OUT | FADES IN AND OUT |
| FADES | FADING |
| FEAT | FEATURES |
| FEAT BRK | CUSTOM CALL FEATURES DO NOT WORK |
| FLS PULSE | FALSE KEY PULSE |
| FRI | FRIDAY |
| FRM ERR | FRAME ERRORS |
| FRM ERR/H | FRAME ERRORS HICAP |
| FRM SLPS | FRAME SLIPS |
| FSR | FALSE RINGS |
| FST BSY | FAST BUSY |
| GBI | GETS BUSY IN |
| GCO | GETS CUT OFF |
| GRBL | GARBLED |
| GRBL DATA | GARBLED DATA |
| GRND | GROUND |
| GRND HUM | GROUND HUM |
| GWN | GETS WRONG NUMBER |
| HAE | HARMONIC DISTORTION |
| HAZARD | HAZARD |
| HI ROLL | HIGH END ROLLOFF |
| HI/DRY | HIGH AND DRY |
| HICAPDOWN | HICAP DOWN |
| HID | HIGH DISTORTION |
| HIL | HIGH LEVELS |
| HIS | HISTORICAL REPORTS |
| HITS | MULTIPLE SHORT DURATION HIT |
| HOL | HOLLOW |
| HOOL | HEARS OTHERS ON LINE |
| HOT LVL | HOT LEVELS |
| HTG | HUNTING |
| HUM ON LN | HUM ON LINE |
| HUNG UP | HUNG UP |
| IMP NSY | IMPULSE NOISE |
| IN FAIL | HIGH RATE INCOMPLETE INCOMING |
| IN LPBK | CIRCUIT IN LOOPBACK |
| IN OK | INCOMING OK |
| INF | INFORMATION |
| INFO | INFORMATION |
| INSP | INSPECT |
| INV DATA | INVALID DATA |
| IW | INSIDE WIRE |
| JITTER | PHASE JITTER |
| LBO | LIGHT BURNED OUT |
| LCR | LAST CALL RETURN |
| LD | LONG DISTANCE |
| LINE LOSS | LINE LOSS |
| LOC | LOCATION |
| LOL | LOW LEVELS |
| LONG LVL | LONG LEVELS |
| LOOSE JK | LOOSE JACK |
| LOUD | LOUD |
| LOW ROLL | LOW END ROLLOFF |
| LOW SF | LOW SF |
| LOW SIG | LOSS EPSCS OR SWITCHED |
| LTR | LOST TIME REPORT |
| LXD | LINE CROSSED |
| MEM | MEMORY |
| MIS PULSE | MISPLACED START PULSE |
| MISC | MISCELLANEOUS |
| MISS COMM | MISSED COMMITMENT |
| MKBSY | MAKE BUSY |
| MODEM | MODEM |
| MON | MONDAY |
| MONT CKT | MONITOR CIRCUIT |
| MSC | MAINTENANCE OF SERVICE CHARGE |
| MUT DIG | MUTILATED DIGIT GROUP |
| # | NUMBER |
| #CHG | NUMBER CHANGED |
| ND EQLZD | NEEDS EQUALIZED |
| NDT | NO DIAL TONE |
| NDT ORDER | NEW SERVICE NOT WORKING |
| NDT/T | NO DIAL TONE AT TIMES |
| NI | NETWORK INTERFACE |
| NO ANI | ANI FAILURE |
| NO ANSBK | NO ANSWER BACK |
| NO CONT | NO CONTINUITY |
| NO CXR | NO CARRIER |
| NO D CHNL | D CHANNEL DOWN |
| NO LPBK | NO LOOP BACK |
| NO NTWK | NETWORK FAILURE |
| NO POLL | NOT POLLING |
| NO PULSE | NO KEY PULSE |
| NO REG | NO REGISTER |
| NO RESP | NO RESPONSE |
| NO SF | NO SF |
| NO SYNC | LOSS OF SYNC |
| NRNA | NO RING NO ANSWER |
| NSY | NOISY |
| NSY EPS | NOISE EPSCS OR SWITCHED SERVICES |
| NSY/T | INTERMITTENT NOISE |
| NTR | CANNOT TRANSMIT CANNOT RECEIVE |
| NWNK | NO WINK START |
| OD | OUT OF ORDER |
| OG OK | OUTGOING OK |
| OHOP | ONLY HAS ONE PHONE |
| OO FRM | OUT OF FRAME |
| OO SPEC | OUT OF SPECIFICATION |
| OOS | OUT OF SERVICE |
| OPEN | OPEN |
| OPEN EPSC | OPEN EPSCS OR SWITCHED SERVICES |
| OPENDMARC | OPEN TO DEMARC |
| OUT FAIL | OUTGOING FAILURE AFTER WINK |
| OVERSEAS | CANNOT CALL OVERSEAS |
| PERM SIG | PERMANENT SIGNAL |
| PH | PHONE |
| PHYS | PHYSICAL |
| PLZ | PLEASE |
| PRC DEAD | PROCESSOR DEAD |
| PRN | HIGH SPEED PRINTER |
| PROT | PROTECTOR |
| PTY | PARTY |
| PTYLN | PARTY LINE |
| QTD | QUOTED |
| RCF | REMOTE CALL FORWARDING |
| REQ ASSIST | REQUEST TEST ASSIST |
| REQ DISP | REQUEST DISPATCH |
| RES | RESIDENCE |
| RLS | RELEASE |
| RLS CKTEC | RELEASE CKT REQUESTED BY EC |
| RLS CKTIC | RELEASE CKT REQUESTED BY IC |
| RLS FACEC | RELEASE FACILITY REQUESTED BY EC |
| RLS FACIC | RELEASE FACILITY REQUESTED BY IC |
| RMV | REMOVE |
| RNA | RING NO ANSWER |
| RNA CKL | CANNOT RAISE A CIRCUIT LOCATION |
| RNTRBL | OTHER RING TROUBLE |
| RO CKT | RECORDING ON CIRCUIT |
| ROH | RECEIVER OFF HOOK |
| ROL | RECORDING ON LINE OR INTERCEPT |
| ROUTINE | REQUEST FOR ROUTINE |
| RPR | REPAIR |
| RPT DIAL | REPEAT DIAL NOT WORKING |
| RPTD | REPORTED |
| RPTR | REPEATED REPORT |
| RTF | ROUTINE TEST FAILURE |
| RVY RPT | RECOVERY REPORT |
| SAT | SATURDAY |
| SC8/30 | SPEED CALLING 8 OR 30 |
| SFTWR ALM | SOFTWARE GROUP ALARM |
| SGD ERR | STATION GROUP DESIGNATION DIGIT FAILURE |
| SIG ERR | SIGNALING NETWORK FAILURE INCOMING |
| SLIP | SLIP |
| SLOW RESP | SLOW RESPONSE |
| SO | ORDER WORK |
| SOL | STATIC ON LINE |
| STAT | STATUS |
| STK SNDR | STUCK SENDER |
| STRMR | STREAMER |
| SUN | SUNDAY |
| SUP | SUPERVISION/IMPROPER SUPERVISION |
| SUPV | SUPERVISOR |
| SWITCH | SWITCH OR TRUNK RELATED |
| SYS | SYSTEM |
| T1 ERRS | DEGRADATION OF T1.5 |
| TAG LINE | LINES NEED TAGGING |
| TC | TRANSFER OF CALLS |
| THURS | THURSDAY |
| TIC | TROUBLE ISOLATION CHARGE |
| TKT | TICKET |
| TN | TELEPHONE NUMBER |
| TRACE BLK | CALL TRACE BLOCK NOT WORKING |
| TRAN | TRANSMISSION |
| TRBL | TROUBLE |
| TRK BLKD | TRUNK BLOCKED FAR END |
| TTY | TTY SET |
| VIDO EQUIP | VIDEO EQUIPMENT |
| VM | VOICE MESSAGE |
| VOIC EQUIP | VOICE EQUIPMENT |
| 3WC | THREE WAY CALLING |
| W/O | WITHOUT |
| WATS FSR | OUT WATS RINGING IN |
| WED | WEDNESDAY |
| WKG | WORKING |
| XFER | TRANSFER |
| XMD | CROSS MODULATION |
| XTLK | CROSS TALK |
| XTR DIGIT | EXTRA DIGIT |
| XTR PULSE | EXTRA PULSE |

## Data Definitions and Mappings for POTS Resale

### Status

| TR State | TR Status |
| --- | --- |
| 1 (openActive) | 1 (screening) |
| 1 | 3 (dispatchedIn) |
| 1 | 4 (dispatchedOut) |
| 1 | 6 (bulkDispatchedOut) |
| 1 | 9 (pendingDispatch) |
| 2 (deferred) | 13 (noAccessOther) |
| 2 | 14 (startNoAccess) |
| 2 | 16 (startDelayedMtce) |
| 3 (cleared) | 20 (temporaryOK) |
| 3 | 25 (clearedCustAdvised) |
| 4 (closed) | 27 (closedOut) |

### Status Messages

| State | Status |  | Status Message |
| --- | --- | --- | --- |
| 1 | 1 | Pending Screen |  |
| 1 | 3 | Dispatched In |  |
| 1 | 4 | Dispatched Out |  |
| 1 | 6 | Bulk Dispatched Out, Report one of several routed to a Technician |  |
| 1 | 9 | Pending Dispatch |  |
| 2 | 13 | No Access Other, Technician had no access to TELCO equipment in order to restore service |  |
| 2 | 14 | No Access Subscriber, Outside tech had no access to customer premises |  |
| 2 | 16 | Hold, Report is on hold pending further action |  |
| 3 | 20 | Temporary OK, Service is restored but is temporary |  |
| 3 | 25 | Cleared Customer Advised, Service has been restored and customer advised |  |
| 4 | 27 | Closed, Report is closed in system and no longer electronically bonded |  |

## Trouble Type to Line In Use

If the line is in use (LIU indicator is set to Y), then no initial MLT test is run. If the line is not in use, (LIU indicator is set to N), then the initial MLT test is run.

| Trouble Type | TA Code | LIU |
| --- | --- | --- |
| NoDialToneGroup | 0100 | N |
| NoDialTone | 0101 | N |
| SlowDialTone | 0102 | N |
| CircuitDead | 0103 | N |
| CanNotCallOutGroup | 0200 | N |
| CanNotCallOut | 0201 | N |
| CanNotBreakDialTone | 0203 | N |
| DialToneAfterDialing | 0204 | N |
| HighandDry | 0205 | N |
| CanNotRaise | 0206 | N |
| AllAccessBusy | 0207 | N |
| canNotCallOut2 | 0208 | N |
| CanNotCallLongDistance | 0209 | N |
| CanNotCallOverseas | 0210 | N |
| SpeedCall | 0211 | N |
| cannotCall911 | 0212 | N |
| cannotCall700 | 0213 | N |
| cannotCall800/888 | 0214 | N |
| cannotCall900 | 0215 | N |
| CannotCallDA | 0216 | N |
| CannotCallInterLATAToll | 0217 | N |
| CanNotBeCalledGroup | 0300 | N |
| CanNotBeCalled | 0301 | N |
| CanNotBeCalledBusy | 0302 | N |
| DoNotGetCalled | 0303 | N |
| CanNotTripRing | 0304 | N |
| FalseRings | 0305 | N |
| DoNotAnswer | 0306 | N |
| ReachRecording | 0307 | N |
| CanNotRaiseAStation | 0308 | N |
| CanNotRaiseAdrop | 0309 | N |
| CanNotRaiseACircuitLocation | 0310 | N |
| RingNoAnswer | 0311 | N |
| Reorder | 0312 | N |
| AlwaysBusy | 0313 | N |
| BellDoesNotRing | 0314 | N |
| bellDoesNotRing2 | 0315 | N |
| BellRingsCannotAnswer | 0316 | N |
| bellRingsAfterAnswer | 0317 | N |
| noRingNoAnswer | 0318 | N |
| otherRingTrouble | 0319 | N |
| receivesCallsForWrongNumber | 0320 | N |
| recordingOnLine | 0321 | N |
| ringsThenGoesBusy | 0322 | N |
| canNotBeHeardGroup | 0400 | N |
| canNotBeHeard | 0401 | N |
| canNotHear | 0402 | N |
| fading | 0403 | N |
| distant | 0404 | N |
| reachedWrongNumberGroup | 0500 | N |
| reachedWrongNumber | 0501 | N |
| circuitOperationGroup | 0600 | N |
| open | 0601 | N |
| falseDisconnect | 0602 | N |
| grounded | 0603 | N |
| canNotBeSignalled | 0604 | N |
| canNotSignal | 0605 | N |
| permanentSignal | 0606 | N |
| improperSupervision | 0607 | N |
| supervision | 0608 | N |
| canNotMeet | 0609 | N |
| canNotReleaseCircuit | 0610 | N |
| hungUp | 0611 | N |
| noWinkStart | 0612 | N |
| noSF | 0613 | N |
| lowSF | 0614 | N |
| noContinuity | 0615 | N |
| openToDEMARC | 0617 | N |
| noRingGenerator | 0618 | N |
| badERL | 0619 | N |
| echo | 0620 | N |
| hollow | 0621 | N |
| circuitDead | 0622 | N |
| circuitDown | 0623 | N |
| failingCircuit | 0624 | N |
| noSignal | 0625 | N |
| seizureOnCircuit | 0626 | N |
| lossEPSCSorSwitched | 0627 | N |
| monitorCircuit | 0628 | N |
| newServiceNotWorking | 0629 | N |
| openEPSCSorSwitchedServices | 0630 | N |
| otherVoiceDescribeAdditInfo | 0631 | N |
| trunkBlockedFarend | 0632 | N |
| badBalance | 0633 | N |
| highRateIncompleteIncoming | 0634 | N |
| outgoingFailureAfterWink | 0635 | N |
| cutOffsGroup | 0700 | N |
| cutOff | 0701 | N |
| noiseProblemGroup | 0800 | N |
| intermittentNoise | 0801 | N |
| noisy | 0802 | N |
| foreignTone | 0803 | N |
| clipping | 0804 | N |
| crossTalk | 0805 | N |
| staticOnLine | 0806 | N |
| groundHum | 0807 | N |
| hearsOtherOnLine | 0808 | N |
| humOnLine | 0809 | N |
| clicking | 0810 | N |
| noiseEPSCSorSwitchedServices | 0811 | N |
| levelTroublesGroup | 0900 | N |
| lowLevels | 0901 | N |
| highLevels | 0902 | N |
| longLevels | 0903 | N |
| hotLevels | 0904 | N |
| highEndRollOff | 0905 | N |
| lowEndRollOff | 0906 | N |
| needsEqualized | 0907 | N |
| lineLoss | 0908 | N |
| doesNotPassFreqResponse | 0909 | N |
| levelsOutOfLimits | 0910 | N |
| miscellaneousTroublesGroup | 1000 | Y |
| carrierDown | 1002 | Y |
| recordingOnCircuit | 1010 | Y |
| outwatsRingingIn | 1012 | Y |
| remoteAccess | 1013 | Y |
| multipleShortDurationHit | 1016 | Y |
| networkFailure | 1022 | Y |
| memoryServiceProblemGroup | 1100 | Y |
| picTrouble | 1101 | Y |
| callTransferProblem | 1102 | Y |
| callWaitingProblem | 1103 | Y |
| customCallFeature | 1104 | Y |
| threeWayCalling | 1105 | Y |
| callTraceNotWorking | 1106 | Y |
| callTraceBlockNotWorking | 1107 | Y |
| repeatDialNotWorking | 1108 | Y |
| repeatDialBlockNotWorking | 1109 | Y |
| callReturnNotWorking | 1110 | Y |
| calledReturnBlockNotWorking | 1111 | Y |
| callerIdentificationNotWorking | 1112 | Y |
| callBlockingNotWorking | 1113 | Y |
| voiceMessagingServicesProblem | 1114 | Y |
| callForwardingNotWorking | 1115 | Y |
| callForwardingBusyLineNotWorking | 1116 | Y |
| callForwardingNoAnswerNotWorking | 1117 | Y |
| huntingNotWorking | 1118 | Y |
| selectiveCallForwardingNotWorking | 1119 | Y |
| cannotSetupUniqueRingID | 1120 | Y |
| callerIDBlockNotWorkingPerLine | 1121 | Y |
| callerIDBlockNotWorkingPerCall | 1122 | Y |
| cannotRemoveBlockingOnASingleCall | 1123 | Y |
| dataTroubleGroup | 1200 | N |
| canNotReceiveData | 1201 | N |
| canNotSendData | 1202 | N |
| canNotTransmitCanNotReceive | 1203 | N |
| noReceive | 1204 | N |
| noResponse | 1205 | N |
| delay | 1206 | N |
| noCarrier | 1212 | N |
| errors | 1224 | N |
| garbledData | 1225 | N |
| invalidData | 1226 | N |
| slowResponse | 1228 | N |
| otherDataDescribeAdditInfo | 1229 | N |
| wireBrokSetBrokePoleDown | 1415 | Y |
| otherCaseGroup | 1500 | N |
| callTransferProblem | 1501 | Y |
| callWaitingProblem | 1502 | Y |
| customCallFeatureDoNotWork | 1503 | Y |
| information | 1504 | N |
| threeWayCallingProblem | 1505 | Y |
| orderWork | 1506 | N |
| recovery | 1600 | Y |
| recoveryReport | 1601 | Y |

### Disposition Code to Trouble Found

| Disposition Code | Definition | T1.227 Code | Trouble Found Definition |
| --- | --- | --- | --- |
| 01 | Public Segment - Applies To All Troubles Outside The Phone |  |  |
| 0110 | External Components not listed below | 24 | Public Service Coin Set |
| 0111 | Coin/Coinless Housing | 24 | Public Service Coin Set |
| 0112 | Rotary/TT Dial | 24 | Public Service Coin Set |
| 0114 | Loud button assembly | 24 | Public Service Coin Set |
| 0115 | Handset/Handset cord | 24 | Public Service Coin Set |
| 0117 | Switch-hook/Cradle/Intl Ckt | 24 | Public Service Coin Set |
| 0119 | Instruction Card/Number plate | 24 | Public Service Coin Set |
| 0150 | Enclosure misc. components | 24 | Public Service Coin Set |
| 0151 | Glass | 24 | Public Service Coin Set |
| 0152 | Lights | 24 | Public Service Coin Set |
| 0154 | Shelf | 24 | Public Service Coin Set |
| 0155 | Enclosure | 24 | Public Service Coin Set |
| 0156 | Power | 24 | Public Service Coin Set |
| 0158 | Directory | 24 | Public Service Coin Set |
| 0159 | Signage | 24 | Public Service Coin Set |
| 0160 | Miscellaneous Assemblies | 24 | Public Service Coin Set |
| 0168 | TDD Unit | 24 | Public Service Coin Set |
| 0170 | Clean Equipment | 24 | Public Service Coin Set |
| 0171 | Preventive Maintenance/Routine NTF | 24 | Public Service Coin Set |
| 02 | Public Segment - Applies To All Troubles Inside The Phone |  |  |
|  | Set Internal/Shelf/Book |  |  |
| 0210 | Set internal components not listed below | 24 | Public Service Coin Set |
| 0211 | Chassis | 24 | Public Service Coin Set |
| 0212 | Coin chute | 24 | Public Service Coin Set |
| 0213 | Escrow/Hopper/Relay Assembly | 24 | Public Service Coin Set |
| 0216 | Full Coin Box | 24 | Public Service Coin Set |
| 0217 | Signal Unit/Totalizer | 24 | Public Service Coin Set |
| 0219 | Validator | 24 | Public Service Coin Set |
| 0231 | Software/Reprogram | 24 | Public Service Coin Set |
| 0232 | BCLEC/IXCery/Power/Transformer | 24 | Public Service Coin Set |
| 0233 | PCB (Printed Circuit Board) | 24 | Public Service Coin Set |
| 0234 | Card Reader Assembly | 24 | Public Service Coin Set |
| 0237 | LCD Display | 24 | Public Service Coin Set |
| 0240 | Coin Collection Misc. | 24 | Public Service Coin Set |
| 0246 | Prevent Full Box | 24 | Public Service Coin Set |
| 0247 | Tripped Box, Box not Seated | 24 | Public Service Coin Set |
|  | Miscellaneous |  |  |
| 0290 | Other Miscellaneous | 24 | Public Service Coin Set |
| 0291 | Foreign coin grabber | 24 | Public Service Coin Set |
| 0292 | Tel-Cart bCLEC/IXCery | 24 | Public Service Coin Set |
| 0293 | Lock or key | 24 | Public Service Coin Set |
| 0294 | Coin on ledge | 24 | Public Service Coin Set |
| 0295 | Box not open, not seated | 24 | Public Service Coin Set |
| 0296 | String cutter | 24 | Public Service Coin Set |
| 0297 | Telecart circuitry | 24 | Public Service Coin Set |
| 0298 | Board/Card - Inmate Service | 24 | Public Service Coin Set |
| 0299 | Conduit | 24 | Public Service Coin Set |
| 03 | Regulated Station Wiring |  |  |
| 0331 | Simple Network terminating Wire- for coin phone and/or wire between terminal and demarc | 19 | Station Wiring |
| 0341 | Network Interface, Registration Jack, NCTE | 16 | Station Product Terminal |
| 0371 | Protection | 30 | Protective Connecting Arrangement |
| 0381 | Drop or BSW bad, replaced or repaired | 44 | Outside Wire |
| 0382 | *Broadband drop replaced or repaired | 44 | Outside Wire |
| 0383 | Buried Service Wire bad, placed temp Drop on Grnd | 44 | Outside Wire |
| 0385 | Temp Drop On Grnd damaged; replaced or repaired | 44 | Outside Wire |
| 0387 | Temp Drop On Grnd, cut over to permanent BSW | 44 | Outside Wire |
| 04 | Outside Plant |  |  |
|  | Cable Not Repaired |  |  |
| 0401 | Pair Transferred/Cut to clear | 5 | Facility |
| 0402 | Bridge tap removed/Cut dead ahead | 5 | Facility |
| 0403 | Pair transposed | 5 | Facility |
|  | Copper Cable |  |  |
| 0410 | Intermittent/Came clear | 1 | Came Clear |
| 0411 | Fault in sheath, conductor or pair | 5 | Facility |
| 0421 | Splice case, closure | 5 | Facility |
| 0431 | Terminal, Cross Box, GAI | 5 | Facility |
| 0434 | Encap plant; unable to clear; laid temp drop | 5 | Facility |
| 0441 | Rural, Open or Urban wire | 5 | Facility |
| 0461 | Analog pair gain system | 46 | Outside Plant Equipment |
| 0471 | Digital pair gain system | 46 | Outside Plant Equipment |
|  | Associated Trouble |  |  |
| 0481 | All pole, guy, anchor, trench or other miscellaneous trouble | 46 | Outside Plant Equipment |
|  | Fiber Optic Cable |  |  |
| 0483 | Buried or aerial fiber optic cable | 47 | Outside Plant Fiber Optic |
|  | Broadband |  |  |
| 0492 | Hut, replacing broadband cards | 46 | Outside Plant Equipment |
| 0493 | ONU-COAX power associated trouble | 46 | Outside Plant Equipment |
| 0494 | ONU-Fiber | 47 | Outside Plant Fiber Optic |
| 0495 | ONU-Protection | 46 | Outside Plant Equipment |
| 05 | Central Office |  |  |
|  | Central Office Equipment |  |  |
| 0505 | Wireless port into to Wireline | 9 | Information |
| 0509 | Wireless port out to Wireless | 9 | Information |
| 0510 | Other/Came clear | 1 | Came Clear |
| 0511 | Common equipment processor | 2 | Central Office |
| 0512 | Common equipment peripheral | 2 | Central Office |
| 0513 | Line Equipment | 2 | Central Office |
| 0514 | Billing Equipment | 2 | Central Office |
| 0515 | Trunk, includes calling party hold | 6 | Central Office Facility |
| 0516 | Trunk, Customer/IEC | 6 | Central Office Facility |
| 0518 | Held by traffic | 2 | Central Office |
| 0519 | SS7 network failure | 35 | Service Node |
|  | Central Office Translations |  |  |
| 0520 | Other | 3 | Switch Trouble |
| 0521 | Software CLEC/IXC error | 3 | Switch Trouble |
| 0522 | Parameter error | 3 | Switch Trouble |
| 0523 | Translations Questionaires (TQs) and ASRs | 3 | Switch Trouble |
| 0524 | Routing | 3 | Switch Trouble |
| 0525 | Marketing Centrex forms | 3 | Switch Trouble |
| 0526 | Other marketing forms | 3 | Switch Trouble |
| 0527 | CenturyLink translations letters | 3 | Switch Trouble |
| 0528 | Trunking software | 3 | Switch Trouble |
| 0529 | Special Requests | 3 | Switch Trouble |
|  | Distributing Frames |  |  |
| 0530 | Other/Came Clear | 1 | Came Clear |
| 0531 | Cross-Connect missing | 50 | CO Equipment Frames |
| 0532 | Cross-Connect broken | 50 | CO Equipment Frames |
| 0533 | Cross-Connect work error | 50 | CO Equipment Frames |
| 0534 | Intercept work error (EM only) | 50 | CO Equipment Frames |
| 0535 | Protection/reversing device/coils | 50 | CO Equipment Frames |
| 0536 | Improper connection | 50 | CO Equipment Frames |
| 0537 | Cross-Connect document error | 50 | CO Equipment Frames |
| 0538 | Request for Assistance | 50 | CO Equipment Frames |
| 0539 | Customer change request/Not ready | 50 | CO Equipment Frames |
|  | Line Translations |  |  |
| 0540 | Other/Came clear | 1 | Came Clear |
| 0541 | RCMAC work error | 3 | Switch Trouble |
| 0542 | Flowthrough system error | 3 | Switch Trouble |
| 0543 | Inaccurate missing work document: HPS | 3 | Switch Trouble |
| 0544 | Inaccurate missing work document: SBS | 3 | Switch Trouble |
| 0545 | Inaccurate missing work document: BGS | 3 | Switch Trouble |
| 0546 | Inaccurate missing work document: Other MU | 3 | Switch Trouble |
| 0547 | Inaccurate missing work document: LPC/MPAC | 3 | Switch Trouble |
| 0548 | Inaccurate missing work document: CDAC/BDAC/BCSC | 3 | Switch Trouble |
| 0549 | Inaccurate missing work document: Network Tech | 3 | Switch Trouble |
|  | Power |  |  |
| 0550 | Other/Came clear | 1 | Came Clear |
| 0551 | DC Power Equipment | 2 | Central Office |
| 0552 | AC Power Equipment | 2 | Central Office |
| 0554 | Standby Emergency Power | 2 | Central Office |
|  | Miscellaneous |  |  |
| 0560 | Other/Came clear | 1 | Came Clear |
| 0561 | Subscriber line carrier | 46 | Outside Plant Equipment |
| 0562 | Line testing equipment | 51 | CO Concentrator |
| 0563 | Concentrator | 51 | CO Concentrator |
| 0564 | Range extender | 2 | Central Office |
| 0565 | Carrier system | 46 | Outside Plant Equipment |
|  | Special Services Equipment |  |  |
| 0570 | Other/Came clear | 1 | Came Clear |
| 0571 | Design | 2 | Central Office |
| 0573 | Carrier channel | 2 | Central Office |
| 0574 | Signaling | 35 | Service Node |
| 0575 | Repeater | 2 | Central Office |
|  | Radio System |  |  |
| 0580 | Other/Came Clear | 1 | Came Clear |
| 0581 | Maritime | 2 | Central Office |
| 0582 | Ship to Shore | 2 | Central Office |
| 0583 | G2 Control | 2 | Central Office |
| 0584 | Enhanced Mobile Telephone Service control | 2 | Central Office |
| 0585 | Enhanced Mobile Telephone Service/B | 2 | Central Office |
| 0586 | Telephone Mobile Radio Service manual | 2 | Central Office |
| 0587 | Telephone Mobile Radio Service aitp, atoc | 2 | Central Office |
| 0588 | Advanced Mobile Phone System | 2 | Central Office |
|  | Data Base for Driven Services |  |  |
| 0590 | Other/Came clear | 1 | Came Clear |
| 0591 | Calling Card Services (ABS-LIDB) | 3 | Switch Trouble |
| 0592 | Automatic Intercept System | 3 | Switch Trouble |
| 0593 | Expanded 911 Service | 3 | Switch Trouble |
| 0596 | 800 Data Base | 3 | Switch Trouble |
| 0597 | 700 Services | 3 | Switch Trouble |
| 0598 | Expanded 800 Service | 3 | Switch Trouble |
| 0599 | Dial it Services | 3 | Switch Trouble |
| 06 | Customer Action/Public Services/Customer Use Of Equipment |  |  |
| 0611 | Receiver Off Hook (ROH) | 25 | CLEC/IXC Operation Instructions |
| 0612 | Calling Party Hold (CPH) | 25 | CLEC/IXC Operation Instructions |
| 0613 | User Dialing Wrong | 25 | CLEC/IXC Operation Instructions |
| 0615 | Foreign or Bent Coins/Objects, Pennies | 25 | CLEC/IXC Operation Instructions |
| 0616 | Canadian | 25 | CLEC/IXC Operation Instructions |
| 0617 | Remove for Remodeling | 25 | CLEC/IXC Operation Instructions |
| 0618 | Can't Be Called (On Outgoing Only) | 25 | CLEC/IXC Operation Instructions |
| 0619 | Line Is In Use | 25 | CLEC/IXC Operation Instructions |
| 0690 | Service Order Activity | 9 | Information |
|  | Customer Action/All Classes Of Service/Non-Dispatched |  |  |
| 0620 | Service Order | 9 | Information |
| 0640 | Suspended or Disconnected | 9 | Information |
| 0650 | Cancel Report | 42 | Cancel Excluded |
| 0651 | Request to Cancel | 42 | Cancel Excluded |
| 0660 | Not Currently in Trouble | 9 | Information |
| 0661 | Customer Requests Information | 9 | Information |
| 07 | Test OK, Verify OK |  |  |
| 0701 | Verified OK with Customer | 26 | Tested OK, Verified OK |
| 0702 | Customer does not answer (DA) | 23 | Test OK |
| 0703 | Central Office Overload | 23 | Test OK |
| 0704 | *Broadband TOK | 23 | Test OK |
| 0710 | Operator report TOK | 23 | Test OK |
| 0720 | PRED report TOK non-dispatched | 23 | Test OK |
| 0747 | Test OK via front end close out | 23 | Test OK |
| 0751 | TOK, verified OK with Customer | 26 | Tested OK, Verified OK |
| 0752 | TOK, Customer does not answer | 23 | Test OK |
| 08 | Found OK In |  |  |
| 0810 | Test made - No Trouble Found | 27 | CO Facility Tested, Found OK |
|  | Data Base for Data Base Driven Services |  |  |
| 0880 | Other C. O. based services | 27 | CO Facility Tested, Found OK |
| 0891 | Calling Card Services | 27 | CO Facility Tested, Found OK |
| 0892 | Automatic Intercept Services | 27 | CO Facility Tested, Found OK |
| 0893 | Expanded 911 Service | 27 | CO Facility Tested, Found OK |
| 0897 | 700 Services | 27 | CO Facility Tested, Found OK |
| 0898 | Expanded 800 Service | 27 | CO Facility Tested, Found OK |
| 0899 | Dial It Service | 27 | CO Facility Tested, Found OK |
| 09 | Found OK Out |  |  |
| 0910 | Dispatched and Found OK | 13 | No Trouble Found |
| 0920 | *Broadband dispatched and found OK | 13 | No Trouble Found |
| 0950 | Unable to isolate trouble In or Out because of no access to demarc/protector | 13 | No Trouble Found |
| 0970 | Public Services Trouble reported and found OK | 13 | No Trouble Found |
| 10 | Referred Out |  |  |
| 1001 | CenturyLink Toll Cable Cut/Damaged | 29 | Referred Out to Other Department |
| 1002 | other outside CenturyLink | 29 | Referred Out to Other Department |
| 1005 | Move drop, deliver/place drop | 29 | Referred Out to Other Department |
| 1006 | Other non-network within CenturyLink | 29 | Referred Out to Other Department |
| 1015 | Fiber to Premise ( FTTP) | 29 | Referred Out to Other Department |
| 1016 | VOIP | 29 | Referred Out to Other Department |
| 11 | Non-Telco Plant |  |  |
|  | Non-CenturyLink, dispatched out |  |  |
| 1150 | Wires down, broken poles, etc., that are other than CenturyLink plant or equipment, and a dispatch was required to make this determination | 9 | Information |
|  | Non-CenturyLink, not dispatched |  |  |
| 1151 | Wires down, broken poles, etc., that are other than CenturyLink plant or equipment, and a dispatch was not required to make the determination | 9 | Information |
| 12 | Trouble Beyond The Network Interface |  |  |
| 1210 | Customer has a maintenance contract and CenturyLink does not bill | 31 | CPE CLEC/IXC Responsibility |
| 1211 | *Linebacker Coverage of non-standard inside wire | 31 | CPE CLEC/IXC Responsibility |
| 1220 | Customer has trouble beyond the NI and CenturyLink does not bill | 31 | CPE CLEC/IXC Responsibility |
| 1230 | Customer does not have a maintenance contract, or has only WMR, and we bill Trouble Isolation Charge (TIC). | 31 | CPE CLEC/IXC Responsibility |
| 1231 | Idaho non-complex customer and/or Iowa customer where CenturyLink does not bill | 31 | CPE CLEC/IXC Responsibility |
| 1240 | Customer does not have any maintenance contract and CenturyLink bills both TIC and Time & Materials | 31 | CPE CLEC/IXC Responsibility |
| 1250 | Customer does not have a maintenance contract and CenturyLink bills Trip Charge and Time & Materials | 31 | CPE CLEC/IXC Responsibility |
| 1260 | Carrier/End-user Trouble where CenturyLink bills TIC and possibly Time & Materials | 31 | CPE CLEC/IXC Responsibility |
| 13 | Miscellaneous Non-Dispatched Reports For Non-CenturyLink Trouble |  |  |
| 1310 | CPE Trouble | 31 | CPE CLEC/IXC Responsibility |
| 1320 | Reported by Other | 31 | CPE CLEC/IXC Responsibility |
| 1330 | Test Assist | 9 | Information |
| 1340 | Carrier Trouble | 8 | Interexchange Carrier |
| 1341 | Carrier/CenturyLink ISP Referral | 8 | Interexchange Carrier |
| 1350 | Carrier /ISP Trouble Report | 8 | Interexchange Carrier |
| 1370 | Customer Instructions | 25 | CLEC/IXC Operation Instructions |

## Disposition Code to Maintenance Service Charge

| Disposition Code | Definition | MSC |
| --- | --- | --- |
| 1230 | Customer does not have a maintenance contract, or has only WMR, and CenturyLink bills Trouble Isolation Charge (TIC) | Y |
| 1240 | Customer does not have any maintenance contract and CenturyLink bills both TIC and Time & Materials | Y |
| 1250 | Customer does not have a maintenance contract and CenturyLink bills Trip Charge and Time & Materials | Y |
| 1260 | Carrier/End-user Trouble where CenturyLink bills TIC and possibly Time & Materials | Y |

## Non-Applicable Trouble Types for Local Resale (POTS)

### CenturyLink will not create an electronic trouble report for these trouble types:

| Trouble Type | TA Code | Extra Rules |
| --- | --- | --- |
| cutCable | 0616 |  |
| (miscellaneousTroublesGroup) |  |  |
| hiCapDown | 1001 |  |
| biPolarViolations | 1003 |  |
| frameErrorsHiCap | 1004 |  |
| outofFrame | 1005 |  |
| lossOfSync | 1006 |  |
| frameSlips | 1007 |  |
| noLoopBack | 1008 |  |
| canNotLoopbackDEMARC | 1009 |  |
| linesNeedTagging | 1011 |  |
| other | 1014 |  |
| alarm | 1015 |  |
| frameErrors | 1017 |  |
| facilityAlarm | 1018 |  |
| softwareGroupAlarm | 1019 |  |
| dChannelDown | 1020 |  |
| degadationOfT1.5 | 1021 |  |
| (dataTroubleGroup) |  |  |
| impulseNoise | 1207 |  |
| phaseJitter | 1208 |  |
| harmonicDistortion | 1209 |  |
| highDistortion | 1210 |  |
| noDataLoopback | 1211 |  |
| notPolling | 1213 |  |
| dataFramingErrors | 1214 |  |
| dropOuts | 1215 |  |
| hits | 1216 |  |
| noAnswerBack | 1217 |  |
| streamer | 1218 |  |
| outOfSpecification | 1219 |  |
| canNotRunToCSU | 1220 |  |
| canNotRunToOSU | 1221 |  |
| deadDataCircuit | 1222 |  |
| circuitInLoopback | 1223 |  |
| crossModulation | 1227 |  |
| gettingAllOnes | 1230 |  |
| slip | 1231 |  |
| stationTroubleGroup | 1300 |  |
| voiceEquipment | 1301 |  |
| dataEquipment | 1302 |  |
| videoEquipment | 1303 |  |
| otherEquipment | 1304 |  |
| stationWiring | 1305 | N/A for all states except Colorado, Minnesota, and Oregon. |
| physicalTroubleGroup | 1400 |  |
| lightBurnedOut | 1401 |  |
| dataset | 1402 |  |
| ttySet | 1403 |  |
| highSpeedPrinter | 1404 |  |
| ALI | 1406 |  |
| canNotActivatePC | 1407 |  |
| modem | 1408 |  |
| cathodeRayTube | 1409 |  |
| looseJack | 1410 | N/A for all states except Colorado, Minnesota, and Oregon. |
| OffHook | 1411 |  |
| processorDead | 1413 |  |
| NoRegister | 1416 |  |
| stuckSender | 1417 |  |
| otherStationTrouble | 1418 |  |
| (otherCaseGroup) |  |  |
| releaseCktRequestedByIC | 1507 |  |
| releaseCktRequestedByEC | 1508 |  |
| releaseFacilityRequestedByIC | 1509 |  |
| releaseFacilityRequestedByEC | 1510 |  |
| requestForRoutine | 1511 |  |
| release | 1512 |  |
| requestMonitorOfCircuit | 1514 |  |
| routineTestFailure | 1515 |  |
| lostTimeReport | 1516 |  |
| historicalReports | 1517 |  |
| switchOrTrunkRelated | 1518 |  |
| requestTestAssist | 1519 |  |
| analogTestLine | 1520 |  |
| digitalTestLine | 1521 |  |
| manualInterventionRequested | 1522 |  |
| switchedNetworkGroup | 1700 |  |
| aNITimeout | 1701 |  |
| extraDigit | 1702 |  |
| extraPulse | 1703 |  |
| falseKeyPulse | 1704 |  |
| misplacedStartPulse | 1705 |  |
| mutilatedDigitGroup | 1706 |  |
| noKeyPulse | 1707 |  |
| partialDialTimeout | 1708 |  |
| signalingNetworkFailureIncoming | 1709 |  |
| stationGroupDesignationDigitFailure | 1710 |  |
| aNI Problem | 1711 |  |
| oSPSEqualAccessSignaling | 1712 |  |
| missingANI | 1713 |  |
| vacantCodeAnnouncement | 1714 |  |
| invalidDigit | 1715 |  |
| highandWet | 1716 |  |
| payPhoneProblemsGroup | 1800 |  |
| noCoinReturn | 1801 |  |
| coinStuck | 1802 |  |
| cannotDepositCoin | 1803 |  |
| coinsFallThrough | 1804 |  |
| coinsDoNotRegister | 1805 |  |
| payPhoneDamage | 1806 |  |

## Data Definitions and Mappings for UNE/ACCESS/CARRIER

### Status

| TR State | TR Status |
| --- | --- |
| 1 (openActive) | 1 (screening) |
| 1 | 3 (dispatchedIn) |
| 1 | 4 (dispatchedOut) |
| 1 | 11 (referMtceCenter) |
| 1 | 15 (stopNoAccess) |
| 1 | 17 (stopDelayedMtce) |
| 2 (deferred) | 14 (startNoAccess) |
| 2 | 16 (startDelayedMtce) |
| 3 (cleared) | 26 (clearedAwaitingCustVerification) |
| 4 (closed) | 27 (closedOut) |

### Status Messages

| State | Status | Status Message |
| --- | --- | --- |
| 1 | 1 | Pending Screen |
| 1 | 3 | Handoff to Central Office |
| 1 | 4 | Handoff to Dispatch |
| 1 | 11 | Handoff to Facility |
| 1 | 15 | No Access Canceled or No Access Expired |
| 1 | 17 | Delayed Maintenance Canceled or Delayed Maintenance End |
| 2 | 14 | No Access Start |
| 2 | 16 | Delayed Maintenance Start |
| 3 | 26 | Cleared Awaiting Customer Verification |
| 4 | 27 | Closed |

## UNE/ACCESS/CARRIER Trouble Found Codes

| Trouble Found | T1.227 Code |
| --- | --- |
| pending | 0 |
| cameClear | 1 |
| centralOffice | 2 |
| switchTrouble | 3 |
| customerProvidedEquipment | 4 |
| facility | 5 |
| centralOfficeFacility | 6 |
| ICfacility | 7 |
| interexchangeCarrier | 8 |
| information | 9 |
| nonplanClassified | 10 |
| nonplanClassifiedIC | 11 |
| nonplanClassifiedEA | 12 |
| noTroubleFound | 13 |
| station | 14 |
| stationProductData | 15 |
| stationProductTerminal | 16 |
| stationProductVideo | 17 |
| stationProductVoice | 18 |
| stationWiring | 19 |
| otherStationEquipment | 20 |
| foundOKStation | 21 |
| servingBureau | 22 |
| testOK | 23 |
| publicServicesCoinSet | 24 |
| customerOperatingInstructions | 25 |
| testedOKVerifiedOK | 26 |
| coFacilityTestedFoundOK | 27 |
| outsideFacilityTestedFoundOK | 28 |
| referredOutToOtherDept | 29 |
| protectiveConnectiveArrang | 30 |
| cpeCustomerResponsibility | 31 |
| preService | 32 |
| preServiceIC | 33 |
| preServiceEA | 34 |
| serviceNode | 35 |
| data | 36 |
| customerReferredToVendor | 37 |
| exchangeAccess | 38 |
| international | 39 |
| otherProvidedAccess | 40 |
| existingReport | 41 |
| cancelExclude | 42 |

## Non-Applicable Trouble Types for UNE/ACCESS/CARRIER

CenturyLink will not create an electronic trouble report for these trouble types:

| Trouble Type | TA Code |
| --- | --- |
| (stationTroubleGroup) |  |
| voiceEquipment | 1301 |
| dataEquipment | 1302 |
| videoEquipment | 1303 |
| otherEquipment | 1304 |
| (physicalTroubleGroup) |  |
| lightBurnedOut | 1401 |
| dataset | 1402 |
| ttySet | 1403 |
| highSpeedPrinter | 1404 |
| canNotActivatePC | 1407 |
| modem | 1408 |
| cathodeRayTube | 1409 |
| physicalProblem | 1412 |
| processorDead | 1413 |
| noRegister | 1416 |
| stuckSender | 1417 |
| otherStationTrouble | 1418 |
| (otherCaseGroup) |  |
| manualInterventionRequested | 1522 |
| recovery | 1600 |
| recoveryReport | 1601 |
| payPhoneProblemsGroup | 1800 |
| noCoinReturn | 1801 |
| coinStuck | 1802 |
| cannotDepositCoin | 1803 |
| coinsFallThrough | 1804 |
| coinsDoNotRegister | 1805 |
| payPhoneDamage | 1806 |

## Applicable Trouble Types for Local Number Portability (LNP)

CenturyLink will create an electronic trouble report for these trouble types only on LNP telephone numbers:

| Trouble Type | TA Code |
| --- | --- |
| CanNotCallOut | 0201 |
| canNotCallOut2 | 0208 |
| CanNotBeCalled | 0301 |
| CanNotRaiseAStation | 0308 |
| receivesCallsForWrongNumber | 0320 |

# Appendix F - Glossary/Acronyms

Acronyms either used in this document or useful for discussions about this document are listed below.

| Acronym | Definition |
| --- | --- |
| ACSE | Association Control Service Element |
| ANSI | American National Standards Institute |
| AOM | Application of OSI Management |
| AVC | Attribute Value Change |
| BAS | Business Application Services |
| CCITT | International Telegraph and Telephone Consultative Committee |
| CLEC | Certified Local Exchange Carrier |
| CME | Conformant Management Entity |
| CMIP | Common Management Information Protocol |
| CMIS | Common Management Information Service |
| CMISE | Common Management Information Service Element |
| CTR | Customer Trouble Reporting |
| CUG | Closed User Group |
| DCS | Data Communications Services |
| DES CBC | Data Encryption Standard/Cipher Block Chain |
| DTN | Digital Telephone Number |
| EB | Electronic Bonding |
| ECIC | Electronic Communications Implementation Committee |
| EFD | Event Forwarding Discriminator |
| FU | Functional Units |
| IEC | International Electrotechnical Commission |
| ISO | International Organization for Standardization |
| ISP | International Standardized Profiles |
| LEC | Local Exchange Carrier |
| LMOS | Loop Maintenance Operations System |
| MAPDU | Management Application Protocol Data Unit |
| MEDIACC | Mediated Access |
| MLT | Mechanized Loop Test |
| MOCS | Managed Object Conformance Statements |
| MOI | Managed Object Instance |
| MORT | Managed Object Referring to Test |
| NM | Network Management |
| NSAP | Network Service Access Point |
| OBF | Ordering and Billing Forum |
| OID | Object Identifier |
| OIW | OSI Implementation Workshop |
| ORT | Operational Readiness Test |
| OSI | Open Systems Interconnection |
| OSS | Operations Support Systems |
| PDU | Protocol Data Unit |
| PICS | Protocol Implementation Conformance Statements |
| POI | Point of Interface |
| POP | Point of Presence |
| POTS | Plain Old Telephone Service |
| PSAP | Presentation Service Access Point |
| RMA | Request for Manual Assistance |
| ROSE | Remote Operations Service Element |
| ROER | ROSE Error |
| RORJ | ROSE Error Reject |
| SAP | Service Access Point |
| SMFU | System Management Functional Unit |
| SSAP | Session Service Access Point |
| SYAD | Systems Administration |
| TA | Trouble Administration |
| TACEN | Trouble Administration Configuration Event Notification |
| TMN | Telecommunications Management Network |
| TR | Telecommunications Local Trouble Report |
| TRFD | Trouble Report Format Definition |
| TSAP | Transport Service Access Point |
| TT | Trouble Ticket |
| TT | Trouble Tracking |
| TTR | Telecommunications Trouble Report |
| WFA/C | Work Force Administration/Control |

# Appendix G - Managed Object Instance Formats

Sample POTS Resale and UNE/ACCESS/CARRIER (only unbundled loop and unbundled switch network elements) circuit formats are listed below.

## POTS Resale Formats

### Telephone Number Format

3031234567

### Telephone Number with Terminal Format

3031234567TER12345

## UNE/ACCESS/CARRIER Formats

### Unbundled Loop Circuit Formats

24/LXFU/111111/MS (serial numbered)

24/UBFU/111111/MS

24/HCFU/111111/MS

### Unbundled Switch Circuit Formats

29/SNNU/303/555/1111 (telephone numbered)

A02/T1/CLSPCOMADS0/CLSPCOMAHJ3 (carrier)

97/PH55IEKEYY/CLSPCOMAHJ3/77/CLSPCOMADS0 (message trunk)

# Service Test Function

## Implementation

CenturyLink has implemented all four MLT Test types as specified by the standards. See Electronic Communications Implementation Committee (ECIC) Trouble Administration (TA) Testing Standard Application (ANSI T1.262-1998) Implementation Guidelines, ECIC Document Number: TRA/98-03 for specifics.

## Error Messages – See Section 4 of ANSI T1.262-1998