911/E911 Process Flow

TASK 3A-1:

Process Summary:	Evaluate new and/or augment ES trunk requirements provided  from CLEC.

TASK 3A-2:

Process Summary: Issue Trunk Group Service Request (TGSR) form for ES trunk augments. This form is an internal notification.

TASK 3A-3:

Process Summary:	Complete RG29-0029 form for new trunk requirements.  Additionally, the Service Order Memo to be completed for billing.

TASK 3A-4:

Process Summary:	Distribute RG29-0029 form and Service Order Memo to CP Trunk Forecasting, E911 Center, and Complex Translations.

TASK 3A-5:

Process Summary:	(Minnesota Only)  Initiate State Certification Process.  This process is required for the state to approve additional CenturyLink billing process.

TASK 3A-6:

Process Summary:	Evaluate information on RG29-0029.

TASK 3A-7:

Process Summary:	Create circuit ID, message circuit id, TSC associated with ES trunks.

TASK 3A-8:

Process Summary:	Build trunk information into TIRKS and E-Mail information to E911 center for design.

TASK 3A-9:

Process Summary:	Initiate trunk orders into TIRKS and the Service Order Process.

TASK 3A-10:

Process Summary:	Evaluate end office switch type (5E, DMS, 1A).

TASK 3A-11:

Process Summary:	Provide E911 designer with translations information.

TASK 3A-12:

Process Summary:	If switch type is a 1AESS, designer checks TIRKS for trunk inventory.

TASK 3A-13:

Process Summary:	Complete Trunk Group Number (TGN) translations

TASK 3A-14:

Process Summary:	Continue with TIRKS trunk design process.

TASK 3A-15:

Process Summary:	Notify CP to initiate the trunk capacity process.  This step is initiated if existing trunk facilities are not satisfactory.

TASK 3A-16:

Process Summary:	The WIO Account Team is notified of trunk availability. The Account Team determines new due date.

TASK 3A-17:

Process Summary:	Complete and issue word doc into WFA and provides information to Complex Translations in order to assign trunk members.

TASK 3A-18:

Process Summary:	Assign trunk members.

TASK 3A-19:

Process Summary:	Verify completion of trunk group and trunk group member translations

TASK 3A-20:

Process Summary:	Perform CO x-connects.

TASK 3A-21:

Process Summary:	Perform electronic DACS as required.

TASK 3A-22:

Process Summary:	Verify CO work is complete.

TASK 3A-23:

Process Summary:	Coordinate CO to E911 router testing.

TASK 3A-24:

Process Summary:	Coordinate CO to E911 router to PSAP testing.

TASK 3A-25:

Process Summary:	Complete order in WFA and TIRKS.

TASK 3A-26:

Process Summary:	Generate OCY report.

TASK 3A-27:

Process Summary:	Update and prime the data collection process.

TASK 3A-28:

Process Summary:	Complete Service Order Memo to initiate billing.

TASK 3B-1:

Process Summary:	Request EM trunks and/or data links to the ALI database node.

TASK 3B-2:

Process Summary:	Evaluate request and issue RG29-0151 form to E911 center.

TASK 3B-3:

Process Summary:	Verify accuracy of information.

TASK 3B-4:

Process Summary:	Input service order into SOPAD/SOLAR/IFE.

TASK 3B-5:

Process Summary:	Check and resolve Service Order Input errors.

TASK 3B-6:

Process Summary:	Initiate manual input into TIRKS.

TASK 3B-7:

Process Summary:	Assign local loop if in an independent office is not involved.

TASK 3B-8:

Process Summary:	Complete LAM date

TASK 3B-9:

Process Summary:	Assign trunks into TIRKS.

TASK 3B-10:

Process Summary:	Verify and check TIRKS for record accuracy.

TASK 3B-11:

Process Summary:	Design EM trunks from 911 tandem to PSAP.

TASK 3B-12:

Process Summary:	Issue word document into TIRKS.

TASK 3B-13:

Process Summary:	WFA assigns to 911 implementation and downstream organizations.

TASK 3B-14:

Process Summary:	Verify completion of trunk group and trunk group member translations

TASK 3B-15:

Process Summary:	Perform CO x-connects.  Tag circuits as 911.

TASK 3B-16:

Process Summary:	Perform electronic DACS as required.

TASK 3B-17:

Process Summary:	Verify CO work is complete.

TASK 3B-18:

Process Summary:	Coordinate CO to E911 router testing.

TASK 3B-19:

Process Summary:	Coordinate CO to E911 router to PSAP testing.

TASK 3B-20:

Process Summary:	Complete order in WFA and TIRKS.

TASK 3B-21:

Process Summary:	Generate OCY report.

TASK 3B-22:

Process Summary:	Update and prime the data collection process.

TASK 3B-23:

Process Summary:	Account Team notifies customer of completed service.

TASK 3B-24:

Process Summary:	Complete Service Order Memo to initiate billing.