CenturyLink 911 Resale

CLEC end-users served by a CenturyLink end office switch, through resale of CenturyLink’s retail services, will have their 911 calls routed from the CenturyLink end office switch in the same manner that CenturyLink routes it’s 911 calls.

Trunking

CenturyLink 911 Resale will utilize the same 911/E911 trunking to the E911 Selective Router as CenturyLink’s retail end-users.

Refer to Figure 1.  CenturyLink is the 911 service provider. The Emergency Service (ES) trunks are sized and ordered by CenturyLink and translated to provide the NENA standard CAMA signaling format. The Emergency (EM) trunks are ordered by the PSAP and follow the same CAMA signaling standard.

Figure 1

911 Retail/Resale

ES and EM Trunk E911 Configuration

Dedicated

ES Trunks

(CAMA)

EM Trunks

(CAMA)

CenturyLink

End Office

CenturyLink

911 Selective

Router

PSAP

911

Originated

Calls

Intrado

911

Database

Subscriber Data

ALI

Updates

ALI Queries

Table Updates

CenturyLink

Updates

Database Updates

CenturyLink has established a business relationship with Intrado, Inc.  CenturyLink will provide the CLECs end-user information (database updates) to the E911 Database for CenturyLink 911 Resale Interconnection.  CenturyLink shall use its standard process to update and maintain CLECs end-user service information in the E911 Database on the same schedule that it uses for its own end-users.  CenturyLink assumes no liability for the accuracy of the end-user information provided by the CLEC.