PS/ALI for Resale Based CLECs - Download

Description

Private Switch/Automatic Location Identification (PS/ALI) is an additional capability of E911 emergency response systems that provides end-user’s using a private telephone switch, such as Resale Private Branch Exchange (PBX) Integrated Services Digital Network Primary Rate Interface (ISDN PRI) or some Centrex/Centrons with the Selective Routing (SR) and/or ALI feature(s) of E911 for individual telephone stations served by the PBX/Centrex/Centron (private switch).  This additional capability allows the private switch to identify individual subscriber address and location information to the Public Safety Answering Point (PSAP) operator.  CenturyLink’s retail service, PS/ALI, is available for resale by you to your end-users.  Additional information about resale of CenturyLink’s retail services can be found in the Resale General Product Catalog (PCAT).

SR and ALI are defined as follows:

- Selective Routing (SR) – allows 911 calls to route to a designated PSAP on the basis of the Automatic Number Identification (ANI) number of the station used to place the call.  Calls from different stations within the same prefix could route to different PSAPs.

- Automatic Location Identification (ALI) – identifies and forwards the end-user’s address and location (and related information) to the PSAP for display on their consoles.

PS/ALI is ordered dependent upon the capabilities of the end-user’s private switch.  There are three ordering options and the PSAP is the primary decision-maker for these options:

- If the end-user is not Resale ISDN PRI/Centrex Plus/Centrex Prime/Centron, Resale PS/ALI Centralized Automatic Message Accounting (CAMA) trunk/circuits are ordered.

- If your end-user has Resale ISDN PRI served from a 5ESS switch, then the SR and/or ALI must be ordered as a feature of the Resale ISDN PRI.

- If your end-user has Resale Centrex Plus/Centrex Prime/Centron and they have extended their station lines to another premises using their own facilities, the SR and/or ALI must be ordered as a feature of the Resale Centrex Plus/Centrex Prime/Centron.

Your end-user’s private switch Resale PBX/Centrex/Centrex Prime/Centron system will be viewed as a serving wire center/Central Office (CO) within the E911 network.  The ANI generated by the private switch will be read, processed and utilized as if it were a typical end office in the E911 system.

The E911 SR will route the E911 PS/ALI call to the appropriate PSAP based on the ANI it receives or the default Emergency Service Number (ESN) assigned to the trunk group.  Upon receipt of the information, the PSAP forwards the ANI information to the ALI database over an existing data network where it is then used to retrieve the stored station address and location information.  The PSAP monitor then displays the station name, address and location information for handling by the emergency response personnel.

The PS/ALI feature capability is offered separately from E911 services.  PS/ALI is available through the terms of Commission-approved Interconnection Agreements that include PS/ALI or through the PS/ALI Service Amendment.

The PS/ALI feature capability consists of ALI database updates (i.e. the ability of your Resale PBX/Centrex Plus/Centrex Prime/Centron end-user to update the ALI database) and the transport of PS/ALI calls (the Resale PS/ALI CAMA trunks/circuits to an E911 SR).

PS/ALI Requirements

You must work directly with your end-user to ensure that the following requirements are met.

- Requirements that must be met for PS/ALI:

- For the ALI database update portion of the service, you, or your end-user, are responsible for providing the private switch ALI information directly to Intrado, Inc., which includes the area code, telephone number, name, address, and location information.  The accuracy of the database records is based on the information you, or your end-user, provide to Intrado, Inc.

- End-user’s private switch must have Direct Inward Dialing (DID) station numbers.

- Provide CenturyLink and Intrado Inc., with the telephone number ranges used by your end-user’s Resale PBX/Centrex Plus/Centrex Prime/Centron systems.  The E911 PS/ALI Service Provider Authorization form is used to complete this activity.  Ensure that the E911 PS/ALI Service Provider Authorization form is completed and updated for end-user changes such as additions or deletions to DID ranges.

- Ensure 911 calls are routed directly into an existing 911 network through the SR

- Ensure your end-user routes only 911 dialed call to the SR

- Ensure that the ANI associated with the individual station line is passed

- Your end-user’s switch must provide a full 7-digit or 10-digit numbering system and the associated ANI for every station within the private switch.  If the 7-digit or 10 digit number is not dialable, you are responsible to identify the associated call back number to be populated in the database.

- When the station user dials 911, the private switch system must be able to recognize the digits as a completed dialing code.  (In some systems, it may be necessary to dial a single digit network access code before dialing 911, i.e., dial “9” to make a call outside of the private switch, prior to dialing 911).

Requirements unique to Resale PS/ALI CAMA trunk/circuits include:

- Resale PS/ALI CAMA trunk/circuit requirements will be discussed at the Joint Planning Meeting with the PSAP.  The PSAP is the primary decision-maker for trunk group design, size and routing.  A minimum of two dedicated Resale PS/ALI CAMA trunks/circuits must be established per end-user’s private switch for the sole purpose of sending 911 calls.  These Resale PS/ALI CAMA trunks/circuits must be provisioned to conform to the standard CAMA signaling format.  Standard traffic engineering methods must be used to determine the number of transmission paths required to achieve the design blocking objective of P.01 (Probability of 1%) grade of service.

- Resale PS/ALI CAMA trunks/circuits are not available with Digital Switched Service (DSS)

- Requirements unique to Resale ISDN PRI include:

- Resale Primary Rate Interface (PRI) trunks, served in a 5ESS Central Office, must order SR and/or ALI as a feature of the Resale ISDN PRI.  Resale PS/ALI CAMA trunks/circuits may not be required if your end-user uses Resale ISDN PRI to provide PS/ALI.

Requirements unique to Resale Centrex Plus/Centrex Prime/Centron include:

- Subscribe to the ALI service feature of PS/ALI for any additional locations beyond the Minimum Point of Presence (MPOP), where private facilities are used.

- Request for Resale PS/ALI CAMA trunks/circuits is not required as each station is already assigned a telephone number with ANI capabilities.

Any other requirements will be discussed at the Joint Planning Meeting with CenturyLink.

Pricing

Rate Structure

CenturyLink retail rates, rate elements, and how they apply to PS/ALI can be found in the state specific Tariffs/Catalogs/Price Lists.

When placing your request for PS/ALI, you will be charged for establishing the service, for the Resale PS/ALI CAMA trunks/circuits, if applicable, service features, if applicable, and storage of the ALI Resale PBX/Centrex Plus/Centrex Prime/Centron station records.  These are independent charges from the actual Resale PBX/Centrex Plus/Centrex Prime/Centron product charges.

Resale PS/ALI service is sold on a month-to-month basis.  Nonrecurring and monthly recurring rates are billed to the Resale PS/ALI CAMA trunks/circuits (rates are on a per trunk/circuit basis).  The rate elements will include:

- Network Access Channel (NAC) – per end-user PBX location.

- Channel Connection (CC) – same as NAC (Iowa Only)

- Channel Performance (CP) –  per end-user PBX location, none applies if PS/ALI rides a DS1

- Channel Transmission Parameter (CTP) – same as CP (Iowa Only)

- Transport Mileage (TM) – interoffice mileage applies only between different serving wire center and the 911 SR is composed of mileage bands based on a fixed or a per mile rate.

- Incoming SR Trunk – per trunk/circuit

- Selective Router (SR)

- Automatic Location Identification (ALI)

- Combination of SR and ALI

Resale ISDN PRI/Centrex Plus/Centrex Prime/Centron end-user’s not using dedicated Resale PS/ALI CAMA trunk/circuits will only be billed the service feature charge (SR, ALI, combination of SR and ALI).  The NAC, CP, CC, CTP and TM rates do not apply.

Nonrecurring and monthly recurring rates, relating to service features (SR, ALI, combination of SR and ALI), are rated per 1,000 access lines served, except in the states of Nebraska, North Dakota, South Dakota, Idaho-Southern, and Wyoming, which are per 100 lines served.

Rates

Retail rates can be found in the state specific Tariffs/Catalogs/Price Lists.  CenturyLink’s retail rates for PS/ALI apply.

Pre Ordering

Schedule a face-to-face joint planning meeting with the PSAP Authority Board and the CenturyLink Service Manager.

PS/ALI information required for the Joint Planning Meeting will include:

- E911 PS/ALI Service Provider Authorization form that includes the telephone number ranges allowed for Resale ISDN/PRI/PBX/Centrex Plus/Centrex Plus/Centron end-users.

- Complete name, address and location information for each Resale ISDN-PRI/PBX/Centrex Plus/Centrex Prime/Centron station.

More information on scheduling the face to face Joint Planning Meeting is available in the Access to Emergency Services (911/E911) PCAT.

Ordering

It is important to understand the Resale General and Access to Emergency Services (911/E911) procedures before ordering PS/ALI.

General ordering activities are identified in the Ordering Overview.

The PS/ALI database will be activated at Intrado, Inc., after receiving a valid and accurate station/address file.  It is imperative that you work closely with your CenturyLink Service Manager to determine when the request for the applicable Resale PS.ALI CAMA trunks/circuits and features can be submitted.

.

A minimum interval applies to the establishment of new CAMA trunks/circuit groups and it is applicable to all service providers.  The timeline begins with the issuance of a complete RG29-OO29 internal form and is based on 12 business days:

It is important to note that the 12 business day interval listed above is a minimum.  PS/ALI is a complex service involving multiple centers and work groups.  PSAPs and other governance agencies must approve any PS/ALI serving arrangement.  More lead-time than the minimum greatly enhances the ability of CenturyLink to address contingencies and still meet DDs.  While minor adjustments to some of the intermediate dates may occasionally be required, any reduction of the overall 12 business day interval should be considered an expedite to be negotiated on an ICB.  This interval also applies to the augmentation of existing trunks groups.  If multiple trunk groups are requested simultaneously, longer intervals may be appropriate and critical dates should be negotiated on a project basis.

The LSOG forms used for your request will include:

- Local Service Request (LSR)

- Resale Private Line (RPL)

Detailed information describing field entry requirements are available on the LSOG web page.

Required in the Remarks field in the LSR form is:

- PS/ALI

- Required in the Remarks field of the RPL, the PS/ALI feature must be noted as follows:

- SR

- ALI

- Both

When requesting PS/ALI features for Resale /Centrex Plus/Centrex Prime/Centron, you will need to submit the following LSOG forms using CenturyLink LSOG:

- LSR

- End User Form (EU)

- Centrex Resale Services (CRS)

The requested features need to be noted in the Feature Details section of the CRS:

| # USOC | # Description |
| --- | --- |
| 9DM | Automatic Location Identification (ALI) (Per 1,000 Station Lines) |
| 9DS | Automatic Location Identification (ALI) (Per 100 Station Lines) |
| 9DW | Automatic Location Identification (ALI), Selective Routing (SR) (Per 1,000 Station Lines) |
| 9NW | Automatic Location Identification (ALI), Selective Routing (SR) (Per 100 Station Lines) |
| 9D2 | Selective Routing (SR) (Per 1,000 Station Lines) |
| 9C2 | Selective Routing (SR) (Per 100 Station Lines) |

When requesting PS/ALI features for Resale ISDN PRI, you need to submit the following LSOG forms:

- LSR

- EU

- Resale Service (RS)

The requested features need to be noted in the Feature Details section of the RS form.

| # USOC | # Description |
| --- | --- |
| 9DM | Automatic Location Identification (ALI) (Per 1,000 Station Lines) |
| 9DS | Automatic Location Identification (ALI) (Per 100 Station Lines) |
| 9DW | Automatic Location Identification (ALI), Selective Routing (SR) (Per 1,000 Station Lines) |
| 9NW | Automatic Location Identification (ALI), Selective Routing (SR) (Per 100 Station Lines) |
| 9D2 | Selective Routing (SR) (Per 1,000 Station Lines) |
| 9C2 | Selective Routing (SR) (Per 100 Station Lines) |

Use of Universal Service Order Codes (USOCs) and Field Identifiers (FIDs) are described in the USOCs and FIDs Overview.  Using this overview will assist you in identifying USOCs and FID requirements.

Provisioning

General Provisioning and Installation activities can be found in Provisioning in the Access to Emergency Services (911/E911).

A jeopardy occurs on an LSR if a condition exists that threatens timely completion. Jeopardy notification information is described in the Provisioning and Installation Overview.

For information on procedures for updating the ALI Database, refer to the 911 Database Services section of the Access to Emergency Services (911/E911).

Billing

On a monthly basis, CenturyLink will provide you with billing information that will provide summary account information as well as end-user sub-account information.

Detailed information regarding the Customer Records and Information System (CRIS) Summary Bill, Inquiry and Disputes is described in Billing Information – Customer Records and Information System.