Centrex Line Class Codes (LCC) Job Aid

# Description

Line Class Code (LCC) screening enables control over the types of calls individual lines can make or receive.  Each line requires an LCC. It is not necessary for each line to have the same LCC.

The tables below are separated and labeled for the following Centrex products:

- Centrex 21

- Centrex Plus

- Centron

- Centrex Prime

- The tables are provided for informational purposes only and are a sampling of the most commonly used LCCs for each of the products.  The appearance of an LCC in one of the tables does not mean that it is available to be assigned to your Centrex accounts.  If you have any questions regarding the availability of a particular LCC, please contact your Service Manager.

For additional information on the Terminal Station Restriction (/TGS) FID for 5ESS central office switch types, see specific information at the end of this document.

## Centrex 21

### Blocking Terms

Definitions of common blocking terms used with Centrex 21 service for both incoming and outgoing calls are as follows:

#### Incoming Calls

- Non-Restricted - No restrictions are placed on the line, may receive all incoming calls.

- Fully Restricted - Can receive all incoming local calls, all incoming toll calls will be blocked.

#### Outgoing Calls

- Non-Restricted - No restrictions are placed on the line, may place calls to any destination.

- Fully Restricted - Can place outgoing local calls, but can not place outgoing toll calls.

- Toll Restricted - Can place outgoing local calls, can not dial any 1+ or 0+ calls.

- Enhanced Toll Restricted - Can place outgoing local calls, can not dial 1+ call, and can dial 0+, 1+800, and 1+950.

- Block various destinations - Can place toll calls, but, as specified for the particular LCC to use, Directory Assistance (DA) and/or 900/976/960 destinations will be blocked.

#### 900/976/960 (Pay per call restriction)

This type of blocking is part of Centrex 21 pricing. No additional USOCs are applicable.

#### CustomNet

When converting from a business line with CustomNet to Centrex 21 with a CustomNet type of blocking, the CustomNet billing USOC is used on the service order; however, the Centrex 21 LCC should be used instead of the CustomNet LCC.

### Centrex 21 LCC Chart

The following line class codes should be used for Centrex 21.

Note:  The Terminal Station Restriction (/TGS) FID is required in 5ESS™ -

| Originating Service (Outgoing) | Terminating Service (Incoming) | Centrex Station LCC | Multi-Line Hunting LCC (5ESS only) | TGS (Terminal Group/Station Restriction FID Entries (5ESS) |
| --- | --- | --- | --- | --- |
| Non-Restricted: A station may originate calls to intra-group stations, tie lines, foreign exchange (FX) lines, and the message network as well as call the attendant | Non-Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCN | GNN | *TGUUA |
| Non-Restricted: A station may originate calls to intra-group stations, tie lines, FX lines, and the message network as well as call the attendant | Semi-Restricted: A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive direct inward dial (DID™) calls. | NNS | GCS |  |
| Toll Restricted (411 allowed): A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCT | GNT | *TGUUA |
| Toll Restricted (411 allowed): A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Fully Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NTE | GTE | *TGUFA |
| Toll Restricted (Deny all DA): A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance | Non-Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCB | GNB | *TGUUA |
| Enhanced Toll Restricted: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCR | GNR | *TGUUA |
| International Restricted 1+ (*) | Non-Restricted | NIN |  |  |

| Originating Service (Outgoing) | Terminating Service (Incoming) | Single Line LCC | TGS FID Entries (5ESS) |
| --- | --- | --- | --- |
| CustomNet/Restricted Toll Deny - Allows local, toll free (800), DA, 911 | Non-Restricted | N68 | *TGUUA |
| CustomNet/Non-Restricted - Blocks 900 and 976, Allows 1+ and 0+ | Non-Restricted | N69 | *TGUUA |

Note:  The above LCCs for CustomNet are not your normal 1FB CustomNet LCC. There is not a one-for-one comparison that would give the Centrex 21 user the same exact restrictions that are in Option 1 or 2. Please look at other LCC in this matrix. There may be another LCC that will give needed. LCC N66 and N68 are fairly unique with a limited application.

## Centrex Plus

### Blocking Terms

Definitions of blocking terms used with Centrex Plus service for both incoming and outgoing calls are as follows:

#### 900/976/960 (Pay per call restriction)

Pay Per Call Blocking prevents outgoing calls to informational 900/976/960 numbers.

#### Block

Block various destinations - For example, can place toll calls, but, as specified by the particular LCC, may block or allow Directory Assistance and/or 900/976/960 destinations.

#### CustomNet

CustomNet service provides operator screening when originating a call by dialing 0/0+, thereby restricting the types of calls billed to third number, collect, or calling card depending on the Call Screening Code (/CAS) FID used .  Line restrictions are a standard offering with Centrex Plus; do not include a CustomNet billing USOC.

#### Denied

Denied Originating/Denied Terminating can neither originate nor terminate any outgoing or incoming calls

#### Enhanced toll restricted

Enhanced Toll restricted consists of the following rules:

- Can place outgoing local calls

- Can dial 0+, 1+ 800, 1 + 888, and 1+ 950

- Can not dial 1+ calls

#### Fully restricted

Fully restricted allows the line to originate/terminate calls within the intra-group only.  A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls.  It may not receive DID calls.  A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant or use the call transfer feature.

#### Intra-group

Intra-group means calls made to or received from numbers within the Centrex Plus system.

#### Non-restricted

Non-restricted means no restrictions are placed on the originating/terminating aspects of the line.  A station may originate/receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network.

#### Semi-restricted

Semi-restricted allows the line to originate/terminate intra-group calls or from the attendant console, or private facilities.  A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls.

#### Toll restricted

Toll restricted can place outgoing local calls, but can not dial any 1+ or 0+ calls.

### Centrex Plus LCC Chart

The following line class codes should be used for Centrex Plus.

Note:  The Terminal Station Restriction (/TGS) FID is required in 5ESS™.  If the station line is in a Multi-line Hunt Group, use the column Centrex Plus Multi-line Hunt LCC (5ESS Only).

| Originating (Outgoing) Treatment | Terminating (Incoming) Treatment | Centrex Plus LCC | Centrex Plus Multi-Line Hunt LCC (5ESS Only) | TGS FID Entries (5ESS Only) |
| --- | --- | --- | --- | --- |
| Non-restricted | Non-restricted | NCN | GNN | *TGUUA |
| Non-restricted | Semi-restricted | NNS | GCS | *TGUSA |
| Semi-restricted | Non-restricted | NCS | GNS | *TGSUA |
| Semi-restricted | Semi-restricted | NSS | GSS | *TGSSA |
| Semi-restricted | Deny Termination | NSD | GSD | *TGSFA |
| Fully Restricted | Semi-restricted: A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NES | GES | *TGFSA |
| Fully Restricted | Fully Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NCE | GNE | *TGFFA |
| Denied: Restricts a line from initiating a call. The subscriber will not receive dial tone. | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NDN | GDN | *TGFUA |
| Denied: Restricts a line from initiating a call. The subscriber will not receive dial tone. | Deny Termination: A station is not allowed to receive calls. | NDD | GID | *TGFFA |
| Toll Restricted - 411 Allowed: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCT | GNT | *TGUUA |
| Toll Restricted - 411 Allowed: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Semi-restricted: A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NTS | GTS | *TGUSA |
| Toll Restricted - 411 Allowed: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Fully Restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NTE | GTE | *TGUFA |
| Toll Restricted - Deny all DA: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCB | GNB | *TGUUA |
| Non-restricted - Block 976,960, 900: A station may originate calls to intra-group stations, tie lines, FX lines, and the message network as well as call the attendant. | Semi-restricted: A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | D3S | M3S | *TGUSA |
| Enhanced Toll Restricted: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | NCR | GNR | *TGUUA |
| Enhanced Toll Restricted: A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Semi-restricted: A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NRS | GRS | *TGUSA |
| CustomNet - Non-restricted | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | N66 | M66 | *TGUUA |
| CustomNet - Semi-Restricted | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | N67 | M67 | *TGUUA |
| CustomNet - Toll Restricted - 411 OK | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | N68 | M68 | *TGUUA |
| CustomNet - Non-Restricted - Block 976. 960, 900 | Non-restricted: A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network | N69 | M69 | *TGUUA |

#### LCC Chart

The following LCCs are used with Centrex Plus and later Centrex Prime:

| Originating Service (Outgoing) | Terminating Service (Incoming) | Flat Rate LCC | Multi-Line Hunting Flat LCC |
| --- | --- | --- | --- |
| Non-restricted | Non-restricted | NCN** | GNN** |
| Non-restricted | Semi-restricted | NNS | GCS |
| Semi-restricted | Non-restricted | NCS** | GNS** |
| Semi-restricted | Semi-restricted | NSS | GSS |
| Semi-restricted | Denied | NSD | GSD |
| Fully Restricted | Semi-restricted | NES | GES |
| Fully Restricted | Fully restricted | NCE | GNE |
| Denied | Non-restricted | NDN | GDN |
| Denied | Denied | NDD | GID |
| Toll Restricted - 411 OK | Non-restricted | NCT** | GNT** |
| Toll Restricted | Semi-restricted | NTS | GTS |
| Toll Restricted | Fully Restricted | NTE | GTE |
| Toll Restricted - Deny all DA | Non-restricted | NCB | GNB |
| Non-restricted - Block 976/960/900 | Semi-restricted | D3S | M3S |
| 800 Service – Block 960 | Non-restricted | DDI | MDI |
| Enhanced Toll Restricted | Non-restricted | NCR** | GNR** |
| Enhanced Toll Restricted | Semi-restricted | NRS | GRS |
| CustomNet – Non -Restricted | Non-restricted | N66 | M66 |
| CustomNet – Semi-Restricted | Non-restricted | N67 | M67 |
| CustomNet – Toll Restricted, 411 OK | Non-restricted | N68 | M68 |
| CustomNet – Non-restricted, Block 976/960/900 | Non-restricted | N69 | M69 |

## Centrex Prime

### Blocking Terms

Definitions of blocking terms used with Centrex Prime service for both incoming and outgoing calls are as follows:

#### 900/976/960 (Pay per call restriction)

Pay Per Call Blocking prevents outgoing calls to informational 900/976/960 numbers.

#### Block

Block various destinations - For example, can place toll calls, but, as specified by the particular LCC, may block or allow Directory Assistance and/or 900/976/960 destinations.

#### CustomNet

CustomNet service provides operator screening when originating a call by dialing 0/0+, thereby restricting the types of calls billed to third number, collect, or calling card depending on the Call Screening Code (/CAS) FID used .  Line restrictions are a standard offering with Centrex Plus; do not include a CustomNet billing USOC-

#### Denied

Denied Originating/Denied Terminating can neither originate nor terminate any outgoing or incoming calls

#### Enhanced toll restricted

Enhanced Toll restricted consists of the following rules:

- Can place outgoing local calls

- Can dial 0+, 1+ 800, 1 + 888, and 1+ 950

- Can not dial 1+ calls

#### Fully restricted

Fully restricted allows the line to originate/terminate calls within the intra-group only.  A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls.  It may not receive DID calls.  A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant or use the call transfer feature.

#### Intra-group

Intra-group means calls made to or received from numbers within the Centrex Plus system.

#### Non-restricted

Non-restricted means no restrictions are placed on the originating/terminating aspects of the line.  A station may originate/receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, and the message network.

#### Semi-restricted

Semi-restricted allows the line to originate/terminate intra-group calls or from the attendant console, or private facilities.  A station may receive calls from intra-group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls.

#### Toll restricted

Toll restricted can place outgoing local calls, but can not dial any 1+ or 0+ calls.

| Originating (Outgoing) | Terminating (Incoming) | Centrex STATION LCC | Centrex Multi-line Hunt Group LCC -5ESS |  |
| --- | --- | --- | --- | --- |
| Non-restricted A station may originate calls to intra-group stations, tie lines, FX lines, and the message network as well as call the attendant | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NCN | GNN |  |
| Non-restricted A station may originate calls to intra-group stations, tie lines, FX lines, and the message network as well as call the attendant | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NNS | GCS |  |
| Semi-restricted A station may originate calls to intra-group stations, tie lines, FX lines as well as call the attendant | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NCS |  | GNS |
| Semi-restricted A station may originate calls to intra-group stations, tie lines, FX lines as well as call the attendant | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NSS |  | GSS |
| Semi-restricted A station may originate calls to intra-group stations, tie lines, FX lines as well as call the attendant | Deny Termination A station is not allowed to receive calls. | NSD |  | GSD |
| Fully Restricted A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant, nor use the call transfer feature. | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NES |  | GES |
| Fully Restricted A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant, nor use the call transfer feature. | Fully Restricted A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NCE |  | GNE |
| Denied Restricts a line from initiating a call. The subscriber will not receive dial tone. | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NDN |  | GDN |
| Denied Restricts a line from initiating a call. The subscriber will not receive dial tone. | Deny Termination A station is not allowed to receive calls. | NDD |  | GID |
| Toll Restricted - 411 Allowed A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NCT |  | GNT |
| Toll Restricted - 411 Allowed A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NTS |  | GTS |
| Toll Restricted - 411 Allowed A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Fully Restricted A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NTE |  | GTE |
| Toll Restricted - Deny All Directory Assistance (DA) A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NCB |  | GNB |
| Non-restricted - Block 976,960,900 Stations may originate calls to intra-group stations, tie lines, FX lines, and the message network as well as call the attendant. | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | D3S |  | M3S |
| Enhanced Toll Restricted A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | NCR |  | GNR |
| Enhanced Toll Restricted A station may originate calls to intra-group stations, tie lines, FX lines and the message network. These stations are denied access to operator services and 1+ long distance. | Semi-restricted A station may receive calls from intra group stations, the Centrex attendant tie lines, FX lines, call transfer and call forwarded calls. It may not receive DID calls. | NRS |  | GRS |
| CustomNet -Non-restricted | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | N66 |  | M66 |
| CustomNet -Semi-restricted | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | N67 |  | M67 |
| CustomNet -Toll Restricted - 411 OK | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | N68 |  | M68 |
| CustomNet - Non-restricted - Block 976, 960, 900 | Non-restricted A station may receive calls from intra group stations, Centrex attendant, tie lines, FX lines, and the message network | N69 |  | M69 |

## Centron

### Blocking Terms

Definitions of blocking terms used with Centron service for both incoming and outgoing calls are as follows:

#### Incoming Calls

- Unrestricted - No restrictions are placed on the line, may receive all incoming calls

- Partially Restricted - Restricts access from the PSTN (Public Switched Telephone Network) to a station

- Fully Restricted - Allows the line to originate/terminate calls within the intra-group only.  A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls.  It may not receive DID calls.  A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant or use the call transfer feature.

#### Outgoing Calls

- Unrestricted - No restrictions are placed on the line, may place calls to any destination

- Partially Restricted - Restricts 1 + direct dialed access to the PSTN from a station

- Fully Restricted - Allows the line to originate/terminate calls within the intra-group only.  A station may receive calls from intra-group stations, Centrex attendant, tie lines, FX lines, call transfer and call forwarded calls.  It may not receive DID calls.  A station may originate calls to intra-group stations, tie lines, and FX lines. It cannot call the attendant or use the call transfer feature.

### Centron LCC Chart

The following line class codes should be used for Centron.

Note:  If the station line is in a Multi-line Hunt Group, use the column Multi-line Hunting LCC (5ESS Only).

| Line Class Code & Description | Unrestricted | Partially Restricted | Fully Restricted | Multi-Line Hunting LCC (5ESS only) |
| --- | --- | --- | --- | --- |
| C3N - Full Access to exchange | X |  |  | G3N |
| C2D* - No direct dial (DD) calls (9+1), receives no incoming calls |  | X |  |  |
| C2F* - No DD calls (9+1), receives intercom and tie line calls |  | X |  |  |
| C2N* - No DD calls (9+1), receives all incoming calls |  | X |  |  |
| C2S* - No DD calls (9+1), receives no DID calls |  | X |  |  |
| C3D - Make all calls outgoing, receives no incoming calls |  | X |  |  |
| C3F - Make all calls, receives intercom and tie line calls |  | X |  |  |
| C3S - Make all calls, receives no DID calls |  | X |  |  |
| C4D* - Make no DD calls (9+1 & 9+0), receives no calls |  | X |  |  |
| C4N* - Make no DD calls (9+1 & 9+0), receives all calls |  | X |  | G4N |
| C4R* - Make no DD calls (9+1 & 9+0) or DA, receives all calls |  | X |  |  |
| C5N - Make no DD calls, receives all calls |  | X |  |  |
| C5S - Make no calls, receives no DID calls |  | X |  |  |
| CFN - Make intercom calls, receives all calls, deny access to attendant |  | X |  |  |
| CSD - Make intercom calls and calls to attendant, receives no calls |  | X |  |  |
| CSF - Make intercom calls and calls to attendant, receives intercom and tie line calls |  | X |  |  |
| CSN - Make intercom calls, receives all calls |  | X |  |  |
| CSS - Make intercom calls and calls to attendant, receives no DID calls |  | X |  |  |
| CFD - Make intercom calls, receives no calls |  |  | X |  |
| CFF - Make and receive intercom calls only |  |  | X |  |
| CFS - Make intercom calls, receives no DID calls |  |  | X |  |
| CTS – Deny originating and terminating |  |  | X |  |
| D3N* - (976 block) Full dial access to exchange | X |  |  | GSN |
| D3S - (976 block) Make all calls, receives no DID calls |  | X |  |  |

## Terminal Station Restriction (TGS) - 5ESS only

In the 5ESS switch, the Terminal Station Restriction (TGS) is required in addition to the LCC.  The /TGS indicator is required on every main station line in the 5ESS central office switch type. The /TGS allows or restricts access to and from the network, whereas the LCC allows further restrictions such as toll and DA.

- The FID /TGS is followed by *TG (Terminal Group) plus restriction elements, e.g. /TGS *TGXXX

- The /TGS identifies a terminal as a member of a terminal group

- Additional data is: *TG(1)(2)(3) (See tables below for valid values for (1), (2), and (3))

(1) - Required originating station restriction

| U | Unrestricted – originate calls to: - outside common block - inside common block - attendant console - private facilities Example: /TGS *TGU++ |
| --- | --- |
| S | Semi-restricted - originate calls to: - inside common block - attendant console - private facilities Example: /TGS *TGS++ |
| F | Fully restricted - originate calls to: - inside common block - private facilities Example: /TGS *TGF++ |

(2) - Required terminating station restriction

| U | Unrestricted - terminate calls from: - outside common block - inside common block - attendant console - private facilities Example: /TGS *TG+U+ |
| --- | --- |
| S | Semi-restricted - terminate calls from: - inside common block - attendant console - private facilities Example: /TGS *TG+S+ |
| F | Fully restricted - terminate calls from: - inside common block - private facilities Example: /TGS *TG+F+ |

(3) - Required station error treatment

| A | Announcement Example: /TGS *TG++A |
| --- | --- |
| T | Touch Tone Example: /TGS *TG++T |

Last Update: January 13, 2012

5ESS™ is a trademark of Lucent Technologies Inc.

DMS™ is a trademark of Nortel Networks

DID™ is a trademark of CenturyLink Communications International Inc.