Circuit ID (ECCKT)

Unbundled Local Loop service requests are identified by using Common Language Circuit Identification (CLCI).  The CLCI is in Serial Number format and is provided to you on your FOC.  Once the Circuit Identification is assigned, all subsequent requests must carry the Circuit Identification in the ECCKT field of the LSR.

IMA will prohibit the ECCKT from being populated on an initial LSR when ACT=N, LNA=N and ReqType = AB, until CenturyLink makes the initial ECCKT assignment.

Once the Circuit Identification is assigned, all subsequent requests must carry the Circuit Identification in the ECCKT field of the LSR.

The existing Private Line CLCI will be changed to an Unbundled Local Loop CLCI and is provided to you on your FOC.  Once the CLCI is assigned, all subsequent design changes will require the Unbundled Local Loop CLCI, and should be compliant with the design requirements identified in Technical Publication, Interconnection – Unbundled Loop 77384.  Private Line to Unbundled Loop Pricing Conversion CLEC Job Aid (See Unbundled Local Loop – General Information, Request Scenarios section)  is a document designed to provide guidance in performing service request entries for the Private Line to Unbundled Loop Price Conversion.

Following is a description of the CenturyLink Circuit Identification Number format:

- Characters 1 and 2 are the state code (Refer to Circuit ID by State Table below.)

- Characters 3 through 6 are the Service Code indicating what product is being ordered.  (Refer to the Service Code Table below.)

- Characters 7 through 12 are numeric

- Characters 13 and 14 represent; PN equates to Western, MS equates to Central, and NW equates to Eastern

29.LXFU.123456..MS is an example of a Circuit Identification Number in Serial Format.

### Circuit ID by State Table

| Central MS |  | Eastern NW |  | Western PN |  |
| --- | --- | --- | --- | --- | --- |
| State | State Code | State | State Code | State | State Code |
| Arizona | 19 | Iowa | 1 | Oregon | 5 |
| Colorado | 29 | Minnesota | 3 | Washington | 4 |
| Idaho | 39 | Nebraska | 7 |  |  |
| Montana | 49 | North Dakota | 5 |  |  |
| New Mexico | 59 | South Dakota | 9 |  |  |
| Utah | 69 |  |  |  |  |
| Wyoming | 79 |  |  |  |  |

### SERVICE CODE TABLE

| Product | Service Code |
| --- | --- |
| Analog type loops, Voice Grade, Non-Loaded ADSL Compatible Loops | LXFU |
| DS1 Capable Loop | HCFU |
| DS3 Capable Loop | HFFU |
| ISDN BRI Capable Loop | UBXU |
| OC3 Capable Loop | OBFU |
| OC12 Capable Loop | ODFU |
| OC48 Capable Loop | OFFU |
| OC192 Capable Loop | OGFU |
| xDSL-I Capable Loop | AGFU |

*UBXU service code replaces UBCU, UBCU is a grandfathered code for ISDN BRI Capable Loop.