# 5ESS: Ordering Information

| Feature Package Entry | USOC | FID | Definition | Data Information | Data Example |
| --- | --- | --- | --- | --- | --- |
| CenturyLink Standard Feature Package | BAFHB-All states, except WA. BAKHB-WA only ACBHB-UT only, when ordered as an unbundled package |  | CenturyLink Standard Feature package provided with ISDN BRI. | This entry should agree with the Class of Service entry (flat or measured) | The TN FID and its data is not used with this USOC |

| ISDN BRI Loop Entries | USOC | FID | Definition | Data Information | Data Example |
| --- | --- | --- | --- | --- | --- |
| ISDN BRI loop USOC, sometimes called the DSL (Digital Subscriber Line) | N2Q |  | Carries 2 B Channels and one D Channel | Required whenever installing a new ISDN BRI loop Several FIDs must be follow this USOC –LSO, FSO, BS, MTER, MDPK, and DCLS The TN FID is not used with this USOC |  |
| Local Serving Office |  | LSO | The switching entity used for a circuit termination. | Required entry on the N2Q NPA and NXX must be specific to the customer’s usual serving office Obtain from pre-order address validation | /LSO NPA NXX |
| Foreign Serving Office |  | FSO | Indicates the basic services provided on the B and D channels | Used when dial tone and telephone number are being supplied by a CenturyLink backhaul or customer designated serving office NPA and NXX must be specific to the central office supplying the dial tone and TN | /FSO NPA NXX |
| Bearer Service |  | BS | Indicates the basic services provided on the B and D channels DMD = On demand service – indicates the B channel will support both voice and data SX = Signaling/Packet – indicates the D channel will support normal signaling and packet data | The required entry will always be: DMD DMD SX | /BS DMD DMD SX |
| Maximum Number of Terminals |  | MTER | Indicates the maximum number of terminals on the ISDN BRI loop | Required entry Can be 1 to 8 terminals in the 5ESS. This entry should be specified on the request by a specific entry or by the Capabilities Package that is ordered | /MTER 2 |
| Maximum D Channel Packet users |  | MDPK | Indicates the maximum number of D-packet devices that will be using the ISDN BRI loop | Required entry in the 5ESS, this can be 0 to 8 devices. | /MDPK 1 |
| Digital Subscriber Line Class of Service |  | DCLS | Tells the switch this a National ISDN protocol | Required entry will always be STD | /DCLS STD |

| B Channel Logical Terminal Entries | USOC | FID |  | Definition | Data Requirements (+ = variable data) | Data Example |
| --- | --- | --- | --- | --- | --- | --- |
| Logical Terminal on the B Channel – also called the PDN USOC | LTHIX |  |  | Sets the Primary Directory Number (PDN) associated with the terminal and provides information to support use of the terminal in an ISDN environment | Required |  |
| Telephone Number |  | TN |  | Indicates the number assigned to the terminal | Required following the logical terminal USOC | /TN NPA NXX-XXXX |
| Primary Number |  | PN |  | Indicates the primary telephone number of the terminal | Required data is the 10-digit number | /PN NPA NXX-XXXX |
| Service Profile Identifier |  | SPID |  | Allows the CPE to identify itself to the central office switch | Required Each LTH1X/ will have its own identifier Data is the 10-digit PDN plus a suffix of 11-11 (no spaces) | /SPID NPANXXXXXX11-11 |
| Digital Services Network Assignment |  | DSNA |  | Common language of the ISDN BRI loop associated with the service. | Required to follow the logical terminal USOC and all associated USOCs except the N2Q and optional features Same data as the CLS assigned to the ISDN BRI loop | Western Region = /DSNA 4.IBSA.123456..PN Central Region = /DSNA 19.IBSA.123456..MS Eastern Region = /DSNA 15.IBSA.123456..NW |
| Terminal Type |  | TTYP |  | Provides the characteristics of the logical terminal | Required for terminal adapters (TTYP A) the entry will always be DISP For ISDN terminals (TTYP C) the entry can be DISP or CACH | /TTYP A DISP /TTYP C DISP; EKTS.CACH |
| Message Service System |  | MSS | Message group number to use when end user has audible or visual message waiting on their voice messaging |  | Visual = /MSS *MSAV MSSGRP.++++ Audible =/MSS *MSAAD MSSGRP.++++ ++++ = voice message provider message group number | /MSS *MSAV MSSGRP.1234 /MSS *MSAAD MSSGRP.1234 |
| Circuit Switched Channel Options |  | CSO |  | Tells the switch: - if DN has voice and/or data ability - how many B channels to use | CSV.+ (Circuit Switched Voice) 0 = voice not supported 1 = voice on 1 B channel CSVC.+++ (Circuit Switched Voice Channel) ANY = either channel NO = None CSD.+ (Circuit Switched Data) 0 = data not supported 1 = data on 1 B channel 2 = data on both B channels CSDC.+++ (Circuit Switched Data Channel) ANY = either channel NO = None | /CSO CSV.1, CSVC.ANY; CSD.1, CSDC.ANY |
| Line Identification |  | LNID |  | Identifies the type of line identification the terminal will display. Delivers the calling line ID. | Required when a DN is associated with a display set. Options include: *LIDLXA – incoming call information will be sent to all terminals sharing the called DN *LIRCNMA – delivers the calling party’s name and number – must also have the BFG (Business /Residence Feature Name) FID The NMCPN (Calling Party ID) USOC must also be present, followed by the /TN, /PN, and /CAPP data | /LNID *LIDLXA /LNIC *LIRCNMA/BFG 1SD002 NMCPN /TN NPA NXX-XXXX/PN NPA NXX-XXXX/CAPP CA.1, NBR.1, ADRPAT.NOR |
| Additional Call Offering Service |  | ACOS |  | Indicates the total number of calls that can be supported over a B channel at one time | Used on terminal adapters (TTYP A) or an ISDN terminal (TTYP C) if the terminal has data capability Data is as follows: CSVACO – Circuit Switched Voice Additional Call Offering (for TTYP A only) – always U CSVLIMIT – Circuit Switched Voice Call Limit = the total number of incoming /outgoing calls – data can be 1-16 (If CSVLIMIT is on the order, then CSVACO with the data of U must also be on the order) CSDLIMIT – Circuit Switched Data Call Limit is the total number of incoming /outgoing data calls – data can be 1-16 | /ACOS CSVACO.U, CSVLIMIT.3; CSDACO.U, CSDLIMIT.1 |
| Multiple Appearance of a TN |  | MADN |  | Used to indicate whether a shared number is the primary appearance or a subsequent appearance of the number | Required if a PDN or SDN appears on another terminal. This FID is used on the LTH1X/ or the DO6 USOCs Note: Do not use the this FID on the Shared Directory Number USOC DO8 | /MADN P |
| Call Appearance Button and Number |  | CAPP |  | Indicates: - which buttons on the terminal have telephone numbers associated with them -whether the terminal has call exclusion - the ringing pattern associated with the call appearance of each number (Call exclusion allows call privacy when a PDN appears as a shared call appearance on another terminal) In addition, the CAPP FID and the CA and NBR data must appear after each feature USOC. The CA and NBR numbers will match the CA and NBR assigned to the PDN with which the feature works. | Required for processing (whether there is a call appearance or not) Possible entries for TTYP C terminals are: CA.NNN – where the number is the first appearance of the number on the terminal – usually 1 for the PDN NBR.NN – the number of call appearances to a maximum of 16 CALLEXCL.M – will be present only if there is call exclusion: M = Manual Exclusion A – Automatic exclusion N = No exclusion ADRPAT. +++ - indicates whether the ringing is: NOR = Normal ABR = Abbreviated DEL = Delayed Ringing NON = Call Appearance does not ring, but light will flash ADRTIME.++ = the number of seconds to abbreviate or delay the ring (0 to 255). Only used if ABR or DEL is used for ADRPAT. Possible entries for TTYP A terminals are: CA.1 – when the number is the first appearance of the number on the terminal NBR.1 = the maximum number of call appearances ADRPAT.+++ indicates whether the ringing is: NOR = Normal NON = Delayed | TTYP C: /CAPP CA.1, NBR.3, CALLEXCL.M, ADRPAT.DEL, ADRTIME.12 TTYP A: /CAPP CA.1, NBR.1, ADRPAT NOR/CAPP CA.1, NBR.1 /ADRPAT.NON |
| Maximum B channels |  | MAXB |  | Signifies how many B channels the terminal can access simultaneously | Standard entry is 2 unless otherwise specified or line is Multi-Line hunt | /MAXB 2 |
| Terminal Group / Station Restriction |  | TGS |  | Identifies the terminal group assigned to the terminal | Part of BFG ISD001 or ISD002. If BFG FID is not present, TGID FID is required. Data entry will always be *TGUUA | /TGS *TGUUA |
| Terminal Group Identification |  | TGID |  | Used to indicate the terminal group number | The TGID feature name is included in the BFG FID data If the BFG FID is not present, the TGID FID is required. Data entry will always be 1000 | /TGID 1000 |
| Terminal Configuration Group Number |  | TCGN |  | Indicates the terminal configuration group assigned to the terminal | Required | /TCGN NSCG015 |
| Feature Name |  | FNM |  |  | Data is the actual feature name typed into the switch Each optional feature must be followed by the /PN, /TN, and/or /DTN, and the /CAPP | /FNM *MWICTIA, *CPCOFA, *RND, *CFVFB /TN NPA NXX-XXXX/PN NPA NXX-XXXX/CAPP CA1, NBR.1, ADRPAT.NOR |
| Business/Residence Feature Group Name |  | BFG |  | Specific set of custom features assigned in a 5ESS office | Used when a specific standard configuration group is indicated by the customer |  |
| Individual Dialing Plan |  | IDP |  | Provides the dialing plan number | This information is included in the BFG data – if BFG is not used on the order, the IDP FID must be shown on the order Data is the feature name IDISDN | /IDP IDISDN |
| Intercom Group |  | ICMC |  | Indicates: - intercom group name - type of intercom - auto or 2-digit - auto dial TN - TN intercom number - call appearance button used for the intercom and number | Data is: ICMGRP.+++++, where + is 1-8 SOURCE.++++, where + is AUTO or DIAL TNPOS.++, where + is 0-9 (for AUTO) or 00-99 (for DIAL) CA.+++ where +++ is 1 to 3 digit call appearance button number NBR.++ where ++ is 1 to 2 digit number of call appearances | /ICMC ICMGRP.50234, SOURCE.DIAL, TNPOS.02, CA.5, NBR.1; ICMGRP.50244, SOURCE.AUTO, AUTODP.0, TNPOS.1, CA.6, NBR.1 |
| Calling Party Number Billing |  | CPNB |  | Delivers calling line ID information to the CPE | Conditional The customer must specifically request this. It is used when the customer’s CPE will not accept normal calling line ID information. Data is *CPCOFA floated after the FID | /CPNB *CPCOFA |
| Intercom Dialing Plan Group |  | IDGP |  | Provides the TN list of an intercom group | Used with AUTO intercom and 2-digit DIAL intercom This feature should be specified on the request. Data is: - Intercom Group Name assigned by NSAC - TN or dialed position number from 0-9 and the primary DN for Auto Intercom - TN or 2-digit position number from 00-99 and the primary DN for 2-digit Dial Intercom TGID 1000 | /IDGP 2-50244 20.6884000, 21.6884009, 22.6884010, 23.6884011, TGID 1000 |

| D Channel Logical Terminal Entries | USOC | FID | Definition | Data Requirements (+ = variable data) | Data Example |
| --- | --- | --- | --- | --- | --- |
| D Channel Packet Logical Terminal USOCs | LTH4X |  |  |  |  |
| Public Packet Line Class Code |  | PLCC | Line Class Code for the packet switched services | Data is IPS | /PLCC IPS |
| Logical Channel Layout |  | LCL | Indicates the maximum number of channels and the allocation of the channels associated with an X.25 access line Two logical channels are provided per ISDN BRI loop. If more than 2 are requested, one NW9AL charge USOC must be added to the order for each additional X.25 logical channel | Data is: 2WL – Lowest 2-way logical channel number – will always be 1 2WH – Highest 2-way logical channel number of channels – can be 1-15 | /LCL 2WL.1, 2WH.2 |
| Data Terminal Number |  | DTN | Used to indicate the data terminal number for dialing the device | Used with D Channel packets only – no voice. Data consists of the NPA, prefix and line number. May be the same number as the PDN, or can be a different number | /DTN 719 636-1234/DSNA 29.IBSA.123456..MS |
| Destination Network Identification Code |  | DNIC | Permits routing over a particular interface providing gateways to packet switched networks or databases. Corresponds to the PIC code. | Data is 4 numeric | /DNIC 3147 |
| Throughput Class and Flow Control |  | TWN | Gives the throughput, window size, and packet size negotiation for the packet switched data | Data is: - T (Throughput Negotiation) Y = Yes, N = No - W (Window Size Negotiation) Y = Yes, N = No P (Packet Negotiation) Y = Yes, N = No The SPID, CLS and PN should be floated following this entry | /TWN T.Y, W.Y, P.Y/SPID 303555121211-11/CLS 29.QZSA.24919..MS/PN 303 555-3245 |
| Fast Select Options |  | FS | Indicates the fast select options available on the X.25 packet service | To Initiate Fast Select, data entries are: - IB – Both restricted and unrestricted response - IR – Restricted response - IU – Unrestricted response To Accept Fast Select. data entries are: - AY – Accept Fast Select - AN – Fast Select Not Accepted The CLS and DTN are floated following this entry | /FS IB AY/CLS 19.QSZA.123456..MS/DTN 602 235-6789 |