Local Interconnection Service (LIS) Mid Span Meet Point of Interconnection (POI)

Interconnection Agreements allow for a negotiated POI midway between you and CenturyLink.  A Mid Span Meet POI (MSMP) establishes a physical location where CenturyLink’s network is connected to your network to provide transport for LIS or Wireless Type II trunk groups. The Point of Interface is in CenturyLink QC.  Each provider has the operational responsibility for their portion of the shared facility and for the equipment provided on their side of the POI.

To get started, you need to furnish a network drawing of your proposal to your CenturyLink Service Manager. Normally a meeting is necessary to determine the appropriate MSMP arrangements and mutually agreeable termination points. Your CenturyLink Service Manager should be contacted to arrange a meeting between you and representatives from CenturyLink.

There are three MSMP arrangements: (See exception notice for Arizona, Colorado and Washington, below)

- You use an affiliate ILEC’s switch that has been partitioned with a portion dedicated to you.

- In this instance, you request the use of CenturyLink and an affiliate ILEC existing facilities.  A new LIS trunk group is necessary.  A T1U/T1UZF facility designation is named by CenturyLink Network Engineering which identifies each channel of the existing shared facility that will be used for the new trunk group.  You provide a Letter of Authorization (LOA) from the ILEC to CenturyLink.  An Access Customer Terminal Location (ACTL) for your equipment is named at the CenturyLink office where the existing shared facility terminates.  The T1U/T1UZF facility will be referenced on the design of the facility for the LIS trunks.  If you request one-way trunking, there must be sufficient facilities for CenturyLink to build one-way trunking to you.   CenturyLink engineering will work with you to verify whether or not adequate facilities are available.

- SHARED FACILITY

- CENTURYLINK

- SWITCH

- EO/TANDEM

- ILEC/

- CLEC

- CENTURYLINK

- END   OFFICE

- T1U/T1UZF

- (Named by CenturyLink

- Network Engineering)

- TRUNKS & FACILITY

- (Named from ACTL to switch)

- ACTL NAMED

- (x-connect)

- You request a new MSMP in the CenturyLink Serving Wire Center (SWC) boundary.

- The CenturyLink SWC is generally the CenturyLink central office building nearest your POI/switch.  CenturyLink will establish a MSMP with you approximately halfway between your POI/switch and the CenturyLink SWC.  CenturyLink may not be willing to negotiate such an arrangement if you and CenturyLink do not share equally in the cost of establishing the new MSMP facility.  When CenturyLink agrees to the MSMP request, your CenturyLink Service Manager will contact you within 25 business days.  You need to sign a “Letter of Acceptance” to authorize CenturyLink to start the MSMP construction which is provided to you by your CenturyLink Service Manager.  If CenturyLink does not approve the MSMP proposal, your CenturyLink Service Manager may suggest alternative interconnection arrangements to you.

- MID POINT

- CENTURYLINK

- SWITCH

- EO/TANDEM

- SHARED FACILITY BUILD

- Direct Trunked Transport (DTT)

- (Trunks & facility named from ACTL to CenturyLink Switch)

- YOUR

- LOCATION

- CENTURYLINK

- SWC

- ACTL NAMED

- (All in CenturyLink

- QC Territory)

- You request a MSMP arrangement within CenturyLink QC’s ILEC territory that is outside of your CenturyLink SWC boundary.

- The CenturyLink SWC is generally the CenturyLink central office building nearest to your POI/switch.  CenturyLink will establish a MSMP with you approximately halfway between your POI/switch and any CenturyLink Wire Center (WC) in the LATA.  CenturyLink will meet with you and negotiate an entrance into a CenturyLink WC near the mid-point of the span or at another technically feasible access point within CenturyLink QC’s ILEC territory.  CenturyLink may not be willing to negotiate such an arrangement if you and CenturyLink do not share equally in the cost of establishing the MSMP.  When CenturyLink agrees to the MSMP proposal, your CenturyLink Service Manager will contact you within 25 business days. You need to sign a “Letter of Acceptance” to authorize CenturyLink to start the MSMP construction which is provided to you by your CenturyLink Service Manager.  If CenturyLink does not approve of the MSMP proposal, your CenturyLink Service Manager may suggest alternative interconnection arrangements to you.

- MID POINT

- YOUR

- LOCATION

- CENTURYLINK

##### SWITCH

- EO/TANDEM

- CENTURYLINK

- WC

- DTT

- (Trunks & facility named from ACTL to CenturyLink Switch)

- SHARED FACILITY BUILD

- ACTL NAMED

- (All in CenturyLink

- QC Territory)

- Arizona, Colorado and Washington Exception Notice:

- In Arizona, Colorado and Washington four separate fiber MSMP options are available.  The option chosen must be mutually acceptable by both parties before implementation.  Available options include:

- Joint Meet – 50% Build

- Four fibers from each provider terminating at a mutually agreed upon POI location with approximately a 50% - 50% build.  The fibers terminate at a Fiber Distribution Frame (FDF) at this MSMP.  Each provider will supply their mutually agreed upon fiber termination equipment at their location. Each provider is responsible for their own cost and maintenance to the POI location.

- CLEC Fiber Build

- You supply the four fibers to a mutually agreed upon vault/manhole location outside the CenturyLink tandem or end office. You must provide enough extra cable to allow CenturyLink to extend the fiber to a FDF in the tandem or end office. Each provider will supply their mutually agreed upon fiber termination equipment at their location. Each provider is responsible for their own cost and maintenance from the agreed upon vault/manhole.

- CenturyLink Fiber Build

- CenturyLink will supply four fibers to a mutually agreed upon vault/manhole outside your location (POI/switch).  CenturyLink will provide enough extra cable to allow you to extend the fiber to a FDF in your location (POI/switch). Each provider must supply their mutually agreed upon fiber termination equipment at their location (POI/switch).  Each provider is responsible for their own cost and maintenance from the agreed upon vault/manhole.

- Joint Build – 100%

This option is a combination of the three above.  Each provider constructs fiber to the other provider’s agreed upon vault/manhole allowing extra cable to allow each provider to extend the fiber to the FDF.  You must provide two fibers to the CenturyLink vault/manhole outside the tandem or end office.  CenturyLink would also provide two fibers to the CLEC’s manhole outside your location (POI/switch).  Each provider must supply their mutually agreed upon fiber termination equipment at their location and each provider is responsible for their own cost and maintenance from the agreed upon vault/manholes.

In all states except Arizona, Colorado and Washington, you should initiate your request for one of the above options using the Bona Fide Request (BFR) process.

If you are interested in one of the MSMP options in Arizona, Colorado or Washington you should contact your CenturyLink Service Manager to coordinate a meeting to identify a mutually agreed upon interconnection arrangement.