| Ref | 2Q14 POP 120 REF | Field Name | N/R/C/O | Negotiated Business Rules | Field Length | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Connecting Facility Assignment Query (CFAQ) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| CFAQ1 | 3 | Date and Time Sent (D/TSENT) XML: MSG_TIMESTAMP | R |  | 17 | a/n | CCYYMMDDHHMinMin Military Time |
| CFAQ2 | 2 | Transaction Number (TXNUM) | R | Co-Provider generated and may be reused one month after initial inquiry. | 16 | a/n |  |
| CFAQ3 | 4 | Transaction Type (TXTYP) | R |  | 1 | a | K = CFA Validation |
| CFAQ4 | 5 | Transaction Activity (TXACT) | R |  | 1 | a | A = New Inquiry |
| CFAQ5 | 7 | CC | R |  | 4 | a/n | Products 1, 2, 3, 4, 5, 7, 8, 9, 9a, 10, 16, 17, 22, 23, 23a, 24, 25, 26, 27, 28, 29, 30, 31, 34, 35, 38, 39, 40, 41, 42, 43: Format: ANN Where: A = alphanumeric character N = numeric character Product 14: All uppercase Alpha characters |
| CFAQ6 |  | ACNA | R | Access Carrier Name: This field will be populated with a code identifying the Co-Provider. | 3 | a/n |  |
|  |  | Cable Facility Query Section (CFAQ) |  |  |  |  |  |
| CFAQ7 |  | Loc A (LOCA) | R | Location A: This is the originating CLLI Code. If only LOCA and LOCZ are populated then the Co-Provider wants all the Cable Group records matching the given ACNA, LOCA and LOCZ.. | 11 | a/n |  |
| CFAQ8 |  | Loc Z (LOCZ) | R | Location Z: This is the terminating CLLI Code. If only LOCA and LOCZ are populated then the Co-Provider wants all the Cable Group records matching the given ACNA, LOCA and LOCZ. | 11 | a/n |  |
| CFAQ9 |  | Cable Name (CABNM) | C | Cable Name: A unique designation assigned to a group of cable pair/units between two terminal points. | 10 | a/n |  |
| CFAQ10 |  | First Unit (FIRST UNIT) | C | First Unit: Field identifies the unit by number within the inter-office or tie cable complement. | 5 | n |  |
| CFAQ11 |  | Last Unit (LAST UNIT) | C | Last Unit: Last unit in cable group described in CABNM; identifies the unit by number within the inter-office or tie cable complement. | 5 | n |  |
|  |  | Connecting Facility Assignment Response (CFAR) |  |  |  |  |  |
|  |  | Administrative Section |  |  |  |  |  |
| CFAR1 | 3 | Date and Time Sent (D/TSENT) XML: MSG_TIMESTAMP | R |  | 17 | a/n | CCYYMMDDHHMinMin Military Time |
| CFAR2 | 2 | Transaction Number (TXNUM) | R |  | 16 | a/n | Echo from query |
| CFAR3 | 4 | Transaction Type (TXTYP) | R |  | 1 | a | K= CFA Validation |
| CFAR4 | 5 | Transaction Activity (TXACT) | R |  | 1 | a | A = New Inquiry |
| CFAQ5 | 7 | CC | R |  | 4 | a/n |  |
| CFAQ6 |  | TOT_CFA_GROUPS | O |  | 5 | n |  |
| CFAR7 |  | Loc A (LOCA) | R | Location A: This is the originating CLLI Code. | 11 | a/n | Echo from query. |
| CFAR8 |  | Loc Z (LOCZ) | R | Location Z: This is the terminating CLLI Code. | 11 | a/n | Echo from query. |
| CFAR9 |  | ACNA | R | Access Carrier Name: This is a code identifying the Co-Provider. | 3 | a/n |  |
|  |  | Cable Pair Groups Section This section is present (enabled) only if RESPONSE = G and searching Cable Groups. |  |  |  |  |  |
| CFAR10 |  | Cable Name (CABNM)* | C | Cable Name: A unique designation assigned to a group of cable pair/units between two terminal points. | 10 | a/n |  |
| CFAR11 |  | Cable Type (CABTYP)* | C | Cable Type: A 4-sectioned field used to describe a facility in terms of gauge, load coil spacing, load coil inductance, or in terms of inter-office or tie cable type and modifier. | 13 | a/n |  |
| CFAR12 |  | First Unit (FIRST UNIT)* | C | First Unit: Field identifies the unit by number within the inter-office or tie cable complement. | 5 | n |  |
| CFAR13 |  | Last Unit (LAST UNIT)* | C | Last Unit: Last unit in cable group described in CABNM; identifies the unit by number within the inter-office or tie cable complement. | 5 | n |  |
| CFAR14 |  | Inventory Status (INVSTAT)* | C | Inventory Status: Current status of the Circuit. | 2 | a | IE = In effect; PA = Pending add; PC = Pending change; PR = Pending remove; PX = Pending retirement (for equipment only); RP = Retired (for equipment only); RM = Coded for removal when spare (for equipment only) |
| CFAR15 |  | Quantity Spare (QTYSPARE)* | C | Quantity Spare: Connecting Facility Assignments that are remaining. | 5 | n | 0 to 99999 |
| CFAR16 |  | Percent Available (PCTAVAIL)* | C | Percentage Available: Percent of remaining Connecting Facility Assignments. | 3 | n | 0 to 100 |
|  |  | Cable Pair Units Section This section is present (enabled) only if RESPONSE = G and Cable Units |  |  |  |  |  |
| CFAR17 |  | Cable Name (CABNM)* | R | Cable Name: A unique designation assigned to a group of cable pair/units between two terminal points. | 10 | a/n | Echo from query. |
| CFAR18 |  | Cable Type (CABTYP)* | C | Cable Type: A 4-sectioned field used to describe a facility in terms of gauge, load coil spacing, load coil inductance, or in terms of inter-office or tie cable type and modifier. | 13 | a/n |  |
| CFAR19 |  | Unit (UNIT)* | C | Unit: Identifies the unit number. | 5 | n | 0 to 99999 |
| CFAR20 |  | Loc A (LOCA)* | R | Location A: This is the originating CLLI Code. | 11 | a/n | Echo from query. |
| CFAR21 |  | Loc Z (LOCZ)* | R | Location Z: This is the terminating CLLI Code. | 11 | a/n | Echo from query. |
| CFAR22 |  | Sub Division From (SUBDF)* | C | Subdivision From: This field identifies the routing from the originating Central Office. | 1 | n | 0 to 8 |
| CFAR23 |  | Sub Division To (SUBDT)* | C | Subdivision To: This field identifies the routing to the terminating Central Office. | 1 | n | 0 to 8 |
| CFAR24 |  | Assignment Restriction Code (ASGTRSTN)* | C | Assignment Restriction Code: Code used to identify restrictions against a specific CFA. | 3 | a |  |
| CFAR25 |  | Current Activity (CURACT)* | C | Current Activity: Current activity regarding the Cable Pair Unit. | 1 | a/n | A - Pending add or connect; D - Pending disconnect or move; H - Hold (reserve); J – Jumpered (working but circuit ID unknown); P – Working (CAC entered); R - Reuse, rearrange, rename; W – Working(CKT ID entered); Not Populated– No assignment; $ - Spare History; |
| CFAR26 |  | Pending Activity (PNDACT)* | C | Pending Activity: Pending activity regarding the Cable Pair Unit. | 1 | a/n | A - Pending add or connect; D - Pending disconnect or move; H - Hold (reserve); J – Jumpered (working but circuit ID unknown); P – Working (CAC entered); R - Reuse, rearrange, rename; W – Working(CKT ID entered); Not Populated– No assignment; $ - Spare History; |
| CFAR27 |  | Diversity Indicator (D)* | C | Diversity Indicator: Indicator of alternate transmission route. | 1 | a | Y=Yes |
| CFAR28 |  | Circuit ID/Circuit Layout Order Number (CKTID/CLO)* | C | Circuit ID/Circuit Layout Order Number: This is the layout of the ECCKT. | 45 | a/n |  |
| CFAR29 |  | Due Date (DUEDT)* | C | Due Date: Date the CFA will be utilized. | 8 | n | ccyymmdd |
|  |  | ResponseSection |  |  |  |  |  |
| CFAR30 | 139 | RESPC* | O |  | 3 | n |  |
| CFAR31 | 140 | RESPD* | O |  | 80 | a/n |  |
| CFAR32 | 141 | PRESPC* | C | Provider Response Code: Indicates a predetermined responser code | 14 | a/n | Format(Character positions): 1-3 : Form Name 4-11 : Field Name 12-14 : Error Code |
| CFAR33 | 142 | PRESPD* | C | Provider Response Description: Identifies a predetermined responsecode. Indicates additional information about the error RESPONSE G (Good) or B (Bad) | 264 | a/n |  |