| Ref | Field Name | Action Type | POTS Resale(EB) | BRI ISDN Resale(QB plus LSR P field = A) | Centrex Plus/Centron(PB) | Centrex 21(PB) | PBX(NB plus LSR P field = A) | Designed Trunk ResaleNB plus LSR P field = A) | UNE POTS (MB) | CLSP ISDN BRI(QB plus LSR P field =C) | UNE Centrex(MB) | UNE Centrex 21 (MB) | CLSP PBX Designed Trunk(NB plus LSR P field = C) | UNE POTS (P) Split(MB) | CLSP PBX Designed Trunk Split(MB) | UNE Centrex 21 Split(MB) | Negotiated Business Rules Rules apply to individual products. Product number precedes the applicable business rule. (e.g., 1 - 4 means rule applies to products 1, 2, 3 & 4. 1, 4 means rule applies only to 1 & 4.) | Field Length | Field Characteristics | Valid Values |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 8 | 9 | 9a | 10 | 17 | 25 | 29 | 30 | 31 | 35 | 38 | 39 | 40 | N=Not Req’d., R=Required, O=Optional, C=Conditional, P=Prohibited, Blank=Action Type is N/A, * = Repeating Field (# of stars indicates level of repetition) |  |  |  |
|  | 3.2 Administrative Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | PON | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Purchase Order Number | 16 | a/n |  |
| 2 | VER | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Version Identification | 2 | a/n |  |
| 3 | AN | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Account Number | 20 | a/n | Account number N = New account number requested |
| 4 | ATN | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Account Telephone Number | 10 | a/n | Account telephone number N = New account telephone number requested |
| 5 | PG_of_ | Not used by CenturyLink except for manual forms |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Page of | 6 | n |  |
|  | 3.3 Hunt Group Identification Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | This Section applies to establish, change or to discontinue hunting. The following fields (HGI 6 – HGI 17) repeat as a group. |  |  |  |
| 6 | LOCNUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | R R R R R R R R R | R R R R R R R | R R R | R R R R R R R R | R R R R R R R R R | R R R R R R R R R | R R R R R R R R R | R R R R R R R | R R R | R R R R R R R R | R R R R R R R R R | R R R | R R R | R R R | Location Number All Products: The LOCNUM value will match the LOCNUM entry on the EU form of the LSR. | 3 | n |  |
| 7 | HNUM* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C C C | C C C | C C C | Hunt Number Products 1, 8, 9, 9a, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: This field is required when HA is populated. HNUM must be unique within a single request /PON and sequential on the initial order starting with 001. Once HNUM is populated, it cannot be changed and is retained through completion of the request. Example: There are three hunt groups on the request. HNUM 001 is an existing Series Hunt Group; HNUM 002 is an existing Multi-Line Hunt Group; HNUM 003 is a new Hunt Group type. | 5 | n |  |
| 8 | CB* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | N N N N N N N N N | N N N N N N N | N C C | C N N C C C N N | N N N N N N N N N | N N N N N N N N N | N N N N N N N N | N N N N N N N | N C C | C N N C C C N N | N N N N N N N N N | N N N | N N N | C C C | Common Block This field is optional but if populated must match the CB from the product form. | 11 | a/n |  |
| 9 | HA* | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C C C | C C C | C C C | Hunt Group Activity Products 1, 8, 9, 9a, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: If ACT = N, C, T or V, to establish a new Hunt group HA must = N. If ACT = C, V, or T, with existing hunt group HA must = C to modify or D to remove. | 1 | a | Products 1, 8, 9, 9a, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: N – New C – Change to hunt group sequence D – Remove hunt group arrangement |
| 10 | HID* | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | N N N C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | P C P | P C P | P C P | Hunt Group Identifier Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35: This field is required if HA = C or D. When HA = C or D, HID must match the current HID that is being changed, removed, or converted. When ACT=V, and HA=N, HID is optional. When ACT=C, N or T, and HA=N, HID is prohibited and CenturyLink will assign the HID. Product 9a: When ACT = V, no changes are allowed to the current Hunting if it exists. When ACT = C, LNA must be D. Products 38, 39, 40: When ACT = N or T, HID Prohibited when HA=N. When HA=N, CenturyLink will assign the HID. When ACT=V, and HA=N, HID is optional. | 12 | a/n | Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: For Series Hunt Groups, the format is 1 to 4 alphanumeric characters, i.e.: - A or AN - AA or AAN - AAA or AAAN For Multi-Line Hunt Groups the format is 4 or 5 numeric characters - 0123 - 01231 Hunt group identifier N = New hunt group identifier |
| 11 | TIP* | Not used by CenturyLink |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Telephone Line Identifier Type For Information Only: CenturyLink does not have a current use for this information If field data is populated by the customer, integrity edits are applied against field length, characteristics, and valid values. | 1 | a | T = Telephone number L = LNUM (line number) B = ISDN BRI directory number P = PBX (Private Branch Exchange) |
| 12 | TLI* | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | N N N C N N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C C C | C C C | C C C | Telephone Line Identifier Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: When HNTYP = 4, 6, 13, 14 or 15, TLI is required. Product 9a: When ACT = V, no changes are allowed to the current Hunting if it exists. When ACT = C, LNA must be D. | 14 | a/n | Telephone number Line number “N” |
| 13 | HNTYP* | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | R R R | R R R | R R R | Hunting Type Code Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35: When HA is populated, HNTYP is required. | 2 | n | Products 1, 8, 9, 10, 17, 29, 30, 31, 35, 38, 39, 40: 2 – Sequential Series 3 – Circular Series 4 – Multi-Line series completion with terminal or maintenance numbers 6 – Circular Multi-line 7 – Preferential List Series 13 – Group Make Busy Multi-Line 14 – Random Make Busy Multi-Line 15 – Stop Hunt Multi-Line 16 – Random Make Busy Series 17 – Stop Hunt Series Product 25: 2 – Sequential Series 3 – Circular Series 4 – Multi-Line series completion with terminal or maintenance numbers 6 – Circular Multi-line 7 – Preferential List Series 13 – Group Make Busy Multi-Line 14 – Random Make Busy Multi-Line 15 – Stop Hunt Multi-Line |
|  | 3.4 Hunt Detail Section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The HLA, NOTYP and HTN will collectively identify the individual line activity, number type, and TN or TER associated with any Hunt Activity (HA). This section may repeat, and is required when HA = N or C. |  |  |  |
| 14 | HLA** | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C C C | C C C | C C C | Line Hunt Group Activity Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: If HA = N, HLA must be N. | 1 | a | Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: N = New/install E = Existing/no change D = Disconnect/delete |
| 15 | HTSEQ** | N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified Z=Conv As Spec/No DL C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C C N N N | C N N C C C C N | N C C | C N N C C C C N N | C N N C C C C N N N | C N N C C C C N N N | C N N C C C C N N N | C N N C C C C N | N C C | C N N C C C C N N | C N N C C C C N N N | R R R R | R R R R | R R R R | Hunt Sequence: Identifies the desired hunting sequence or range of hunt sequence. Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: This field is required if HA= N or C. When ACT=C, and HA=C or N, and HNTYP = 2, 3, 7, 16 or 17, then every TN that is included in the Hunting Sequence (HTSEQ) on the HGI form must be present on the CSR, or on the Product Specific Form (Resale, Centrex or Port Service) with an LNA=N, C or X. When ACT=C, and HA=C or N, and HNTYP=4, 6, 13, 14 or 15, then every TLI and TER combination on the HGI form must be present on the CSR with TLI and TER FIDs, or on the Product Specific Form (Resale, Centrex or Port Service) with an LNA=N, C or X. When ACT=N or T, and HA=N, and HNTYP = 2, 3, 7, 16 or 17, then every TN that is included in the Hunting Sequence (HTSEQ) on the HGI form must be present on the Product Specific Form (Resale, Centrex or Port Service). When ACT=N or T, and HA=N and HNTYP=4, 6, 13, 14 or 15 then every TLI and TER combination on the HGI form must be present on the Product Specific Form (Resale, Centrex or Port Service). When ACT=V, and HA=N, and HNTYP = 2, 3, 7, 16 or 17, then every TN that is included in the Hunting Sequence (HTSEQ) on the HGI form must be present on the Product Specific Form (Resale, Centrex or Port Service) with an LNA=N or V. When ACT=V, and HA=N and HNTYP=4, 6, 13, 14 or 15 then every TLI and TER combination on the HGI form must be present on the Product Specific Form (Resale, Centrex or Port Service) with an LNA=N or V. The duplicate TN or TER is not allowed in the hunting sequence (HTSEQ) (sequence, range or combination as shown in the valid values). TN and TER sequence ranges shall always be specified in increasing order. (Example TN 303-804-0506-0509) CenturyLink will manually determine the hunting options of circular, preferential or sequential by the numbering sequence as specified in HTSEQ. In Central region, HTSEQ can be a maximum of 512 characters. In Eastern region, HTSEQ can be a maximum of 143 characters. In Western region, HTSEQ can be a maximum of 300 characters. | 10 | a/n | Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: Valid characters include comma, dash (or hyphen), numerics, space after comma and alpha TER only. The following formats apply for Series (HNTYP = 2, 3, 7, 16 or 17): Allow 7 to 10 digit TN (NPA-NXX-NNNN) For a TN Sequence range, use the following format: NPA-NXX-1234-1238 For a TN Sequence, use: NPA-NXX-1238, NXX-1234, NXX-1236 For a combination, use: NPA-NXX-1238, NXX-1237, NXX-1234-1236. Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 38, 39, 40: For TER the following formats apply (HNTYP = 4, 6, 13, 14, or 15): For TER Sequence range, use the following format: TER1-5 For TER Sequence, use: TER2, 1, 3, 5, 4 For a combination, use: TER5, 4, 1-3 |
| 16 | NOTYP** | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | R R R | R R R | R R R | Number Type Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: An entry is required for each HLA entered. | 1 | a | Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: T = Telephone Number L = Terminal number B = ISDN BRI directory number M = Maintenance number R=Line Number |
| 17 | HTN** | N N=New Installation D=Disconnect W=Conversion As Is V=Conv As Specified C=Change T=Outside Move L=Seasonal Suspend Y=Deny B=Restore M=Inside Move | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N N N | C N N C C C N | N C C | C N N C C C N N | C N N C C C N N N | C C C | C C C | C C C | Hunting Telephone Number Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35: Required when HLA is populated. Valid entries will be the hunting number. TER (ex. TER 1-8) will be valid only when HNTYP = 4, 6, 13, 14 or 15. TN is valid only when HNTYP = 2, 3, 7, 16 or 17. When HLA=N or E, HTN value specified must be present in the HTSEQ field. When HLA=D, HTN value specified must not be present in the HTSEQ field. Duplicate HTN values are not allowed for the same Hunt Group. Products 38, 39, 40: Valid entries will be a telephone number or terminal number (TER). TER (ex. TER 1-8) will be valid only when HNTYP = 4, 6, 13, 14 or 15. TN is valid only when HNTYP = 2, 3, 7, 16 or 17. When HLA=N, HTN value specified must be present in the HTSEQ field. Duplicate HTN values are not allowed for the same Hunt Group. | 17 | a/n | Telephone number Terminal number Maintenance number LNUM (line number) ISDN BRI directory number Products 1, 8, 9, 10, 17, 25, 29, 30, 31, 35, 38, 39, 40: Valid characters include comma, dash (or hyphen), numerics, space after comma and alpha TER only. |